| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0172 |
| --- | --- |

| Title: | Non-EE2: Long tap interpolation filtering on chroma components |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xi Xie1, Kai Zhang2, Li Zhang2, Junru Li1, Meng Wang1, Shiqi Wang1 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | xixie.jensen@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to replace the 4-tap interpolation filter on chroma components in motion compensation with a 12-tap interpolation filter. On top of ECM-3.1, experimental results reportedly show {-0.04%, -1.59%, -1.70%} and {-0.11%, -2.10%, -1.78%} BD-rate changes on average for {Y, Cb, Cr}, under RA and LDB configurations, respectively.

# Introduction

In ECM-3.1, a 4-tap interpolation filter is applied on chroma components while a 12-tap interpolation filter is applied on the luma component in the motion compensation process.

# Proposed methods

This contribution proposes to replace the 4-tap interpolation filter on chroma components in motion compensation with a 12-tap interpolation filter as shown in Table 1

Table 1 The coefficients of the 12-tap interpolation filter for chroma components

| Fractional position | Coefficients (12 taps) |
| --- | --- |
| 1/32 | { 0, 0, 0, 0, 0, 256, 0, 0, 0, 0, 0, 0, }, |
| 2/32 | { 0, 1, -2, 3, -7, 255, 8, -3, 2, -1, 0, 0, }, |
| 3/32 | { -1, 2, -3, 6, -14, 254, 16, -7, 4, -2, 1, 0, }, |
| 4/32 | { -1, 2, -5, 9, -20, 253, 25, -10, 5, -3, 1, 0, }, |
| 5/32 | { -1, 3, -7, 12, -26, 249, 35, -15, 8, -4, 2, 0, }, |
| 6/32 | { -1, 3, -7, 14, -31, 245, 44, -18, 9, -4, 2, 0, }, |
| 7/32 | { -2, 5, -9, 17, -36, 241, 54, -22, 12, -6, 3, -1, }, |
| 8/32 | { -1, 4, -9, 18, -40, 235, 64, -25, 13, -6, 3, 0, }, |
| 6/32 | { -2, 5, -11, 21, -43, 230, 75, -29, 15, -8, 4, -1, }, |
| 7/32 | { -2, 5, -11, 21, -46, 225, 86, -32, 16, -8, 3, -1, }, |
| 8/32 | { -2, 6, -13, 24, -48, 216, 97, -36, 19, -10, 4, -1, }, |
| 9/32 | { -2, 6, -12, 24, -49, 208, 107, -38, 19, -10, 4, -1, }, |
| 10/32 | { -2, 7, -14, 25, -51, 200, 119, -42, 22, -12, 5, -1, }, |
| 11/32 | { -2, 6, -13, 24, -51, 191, 129, -43, 22, -11, 5, -1, }, |
| 12/32 | { -2, 7, -14, 26, -51, 181, 140, -46, 24, -13, 6, -2, }, |
| 13/32 | { -1, 6, -12, 24, -50, 170, 151, -47, 23, -12, 5, -1, }, |
| 14/32 | { -2, 6, -13, 25, -50, 162, 162, -50, 25, -13, 6, -2, }, |
| 15/32 | { -1, 5, -12, 23, -47, 151, 170, -50, 24, -12, 6, -1, }, |
| 16/32 | { -2, 6, -13, 24, -46, 140, 181, -51, 26, -14, 7, -2, }, |
| 17/32 | { -1, 5, -11, 22, -43, 129, 191, -51, 24, -13, 6, -2, }, |
| 18/32 | { -1, 5, -12, 22, -42, 119, 200, -51, 25, -14, 7, -2, }, |
| 19/32 | { -1, 4, -10, 19, -38, 107, 208, -49, 24, -12, 6, -2, }, |
| 20/32 | { -1, 4, -10, 19, -36, 97, 216, -48, 24, -13, 6, -2, }, |
| 21/32 | { -1, 3, -8, 16, -32, 86, 225, -46, 21, -11, 5, -2, }, |
| 22/32 | { -1, 4, -8, 15, -29, 75, 230, -43, 21, -11, 5, -2, }, |
| 23/32 | { 0, 3, -6, 13, -25, 64, 235, -40, 18, -9, 4, -1, }, |
| 24/32 | { -1, 3, -6, 12, -22, 54, 241, -36, 17, -9, 5, -2, }, |
| 25/32 | { 0, 2, -4, 9, -18, 44, 245, -31, 14, -7, 3, -1, }, |
| 26/32 | { 0, 2, -4, 8, -15, 35, 249, -26, 12, -7, 3, -1, }, |
| 27/32 | { 0, 1, -3, 5, -10, 25, 253, -20, 9, -5, 2, -1, }, |
| 28/32 | { 0, 1, -2, 4, -7, 16, 254, -14, 6, -3, 2, -1, }, |
| 31/32 | { 0, 0, -1, 2, -3, 8, 255, -7, 3, -2, 1, 0, }, |

# Experimental results

The proposed method is tested on top of ECM-3.1 [1], according to the common test conditions [2]. The simulation results are summarized in Table 2.

# Conclusions

This contribution proposes a long tap interpolation filter on chroma components. It is recommended to further study the proposed method in EE2.

Table 2. Performance of 12-tap chroma interpolation filter

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.09% | -0.24% | 107% | 112% |  |  |  |  |  |
| Class A2 | -0.01% | -1.00% | -1.30% | 106% | 114% |  |  |  |  |  |
| Class B | -0.08% | -1.52% | -1.72% | 107% | 113% | -0.13% | -2.37% | -2.05% | 104% | 104% |
| Class C | -0.05% | -3.25% | -3.08% | 106% | 113% | -0.16% | -3.47% | -3.06% | 106% | 108% |
| Class E |  |  |  |  |  | 0.01% | 0.19% | 0.36% | 106% | 107% |
| Overall | -0.04% | -1.59% | -1.70% | 106% | 113% | -0.11% | -2.10% | -1.78% | 105% | 106% |
| Class D | -0.04% | -4.48% | -5.09% | 107% | 116% | -0.06% | -3.52% | -6.23% | 107% | 112% |
| Class F | -0.02% | -1.17% | -1.46% | 106% | 110% | -0.19% | -1.36% | -1.38% | 104% | 105% |

# References

- ECM-3.1, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-3.1.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-W2017, Jul. 2021.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).