| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0173-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Y0143 (EE1-1.2: Test on Deep In-Loop Filter with Adaptive Parameter Selection and Residual Scaling) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Johannes Sauer | Email: | johannes.sauer@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for EE1-1.2.1 reported in JVET-Y0143 [1], which uses 
convolutional neural network-based models for in-loop filtering on top of the VTM software.

The crosscheck was only performed for class D. The results deviate from the one of the proponent. This is likely due to the floating point implementation of the used neural networks.

Simulations were performed on linux and there are no mismatches when decoding on linux. However, mismatches are observed when decoding the resulting bitstreams on windows.

# Results

Due to the software requiring dynamic linking to libtorch, it could not be run on our cluster. 
Hence, only a subset of the test was crosschecked: class D.  
The code [3] was compiled using gcc 8.3.0 and tested according to the NN-CTC [2] on a single machine running Ubuntu 19.04. Libtorch 1.10.0 for CPU was used. 
The results were compared with the coding results provided by the proponents for AI, RA and LDB.

In addition bitstreams encoded on linux were decoded on windows. For this case mismatches were observed.

## Table 1 – Crosscheck of AI for class D for EE1-1.2.1

|  | All Intra Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over xCheck only class D |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU | DecT GPU | bit DIFF |
| Class D | 0.00% | -0.02% | -0.02% | 113% | 129% | #DIV/0! | 0% |

## Table 2 – Crosscheck of RA for class D for EE1-1.2.1

|  | Random access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over xCheck only class D |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU | DecT GPU | bit DIFF |
| Class D | -0.02% | 0.06% | 0.00% | 106% | 97% | #DIV/0! | 0% |

## Table 3 – Crosscheck of LDB for class D for EE1-1.2.1

|  | Low delay B Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over xCheck only class D |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU | DecT GPU | bit DIFF |
| Class D | 0.10% | 0.55% | 0.25% | 104% | 97% | #DIV/0! | 0% |

# General comments / questions

## NN paths

The paths to the used neural networks are hard-coded absolute file paths. Hence recompilation is required when the binary is moved to another machine. It is suggested to add command line arguments for specification of neural network paths in the future, ideally supporting both absolute and relative paths.

## Libtorch

Libtorch is dynamically linked which makes it more difficult to run the software on our cluster. We are still looking into this.

A downloaded “static” build of libtorch was still linked dynamically to the binaries after building the software using it.

Can different versions of libtorch be expected to give the same results? At pytorch.org one can download only the latest version (other versions can be downloaded if the link is known, but links to various versions do not seem to be listed somewhere).

# References

[1] “EE1-1.2: Test on Deep In-Loop Filter with Adaptive Parameter Selection and Residual Scaling”, JVET-Y0143, Yue Li, Kai Zhang, Li Zhang, Hongtao Wang, Kevin Reuze, Anand Meher Kotra, Marta Karczewicz

[2] Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-X2016, S. Liu, A. Segall, E. Alshina, R.-L. Liao

[3] https://vcgit.hhi.fraunhofer.de/jvet-x-ee1/VVCSoftware_VTM/-/tree/EE1-1.2.1