| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0186-v1 |
| --- | --- |

| Title: | Cross-check of JVET-Y0143 (Test 1.2.2): EE1-1.2: Test on Deep In-Loop Filter with Adaptive Parameter Selection and Residual Scaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhenyu Dai Yue Yu Luhang Xu | Email: | daizhenyu@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of EE1-1.2.2 described in JVET-X2023 [1], which evaluates the effectiveness of the deep in-loop filter presented in JVET-Y0143 [2]. The simulation results show that BD-rate results are slightly different from the results presented by the proponents. Considering the floating-point arithmetic of neural network compute process, some slightly differences are to be expected. In conclusion, the crosscheck is successful.

Introduction

JVET-Y0143 [2] proposed a deep learning-based in-loop filtering method. First, the proposed filter takes auxiliary information as input. Second, external attention mechanism is introduced. Third, a slice or block could determine whether to apply the CNN-based filter or not. When the CNN-based filter is determined to be applied to a slice/block, the filtering parameter selected from multiple candidates is further decided. At last, when a NN filter is being applied to reconstructed pictures, the difference between the input samples and the NN filtered samples (residues) are scaled by the scaling factors before being added to input samples.

Results

The code base provided by the proponent [3] was compiled using gcc 8.1.0 and libtorch-cpu 1.9.0 and tested according to the ECM-CTC [4] on Linux clusters running Ubuntu 16.04.3 LTS. In EE1-1.2.2, the deep filter presented in JVET-Y0143 [2] is tested based on the ECM anchor, and the results were compared with a similarly prepared EE anchor of ECM3.0 [5], the detailed results could be seen in the attached excel files.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.45% | -13.17% | -15.40% | 101% | 9448% |
| Class A2 | -4.32% | -14.90% | -14.62% | 93% | 8190% |
| Class B | -4.19% | -15.56% | -15.48% | 91% | 6696% |
| Class C | -4.51% | -13.79% | -16.43% | 87% | 4494% |
| Class E | -6.77% | -15.92% | -16.44% | 92% | 7126% |
| Overall | -4.76% | -14.72% | -15.69% | 92% | 6782% |
| Class D | -4.78% | -13.72% | -16.61% | 88% | 4289% |
| Class F | -2.96% | -8.58% | -8.54% | 84% | 3944% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over ECM3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -7.19% | -14.40% | -16.36% | 118% | 7441% |
| Class A2 | -8.00% | -17.21% | -19.99% | 120% | 5811% |
| Class B | -6.83% | -18.62% | -19.32% | 118% | 7136% |
| Class C | -7.19% | -17.19% | -18.36% | 110% | 6087% |
| Class E |  |  |  |  |  |
| Overall | -7.23% | -17.11% | -18.61% | 116% | 6620% |
| Class D | -8.64% | -18.72% | -20.33% | 108% | 5328% |
| Class F | -3.68% | -9.99% | -10.19% | 138% | 4144% |

|  |  |  | Low Delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -6.63% | -16.47% | -17.59% | 123% | 9546% |
| Class C | -6.51% | -16.53% | -17.47% | 108% | 8270% |
| Class E | -7.34% | -20.44% | -17.91% | 152% | 11137% |
| Overall | -6.77% | -17.48% | -17.63% | 124% | 9458% |
| Class D | -7.97% | -19.87% | -20.40% | 103% | 7238% |
| Class F | -4.40% | -11.58% | -10.23% | 135% | 4815% |

References

[1] Exploration Experiments on Neural Network-based Video Coding (EE1), JVET-X2023, E. Alshina, S. Liu, W. Chen, F. Galpin, Y. Li, Z. Ma, H. Wang.

[2] EE1-1.2: Test on Deep In-Loop Filter with Adaptive Parameter Selection and Residual Scaling, JVET-Y0143, Y. Li, K. Zhang, L. Zhang, H. Wang, K. Reuze, A.M. Kotra, M. Karczewicz.

[3] https://vcgit.hhi.fraunhofer.de/jvet-x-ee1/ecm-jvet-x-ee-1/-/tree/EE1-1.2.2 .

[4] Common test conditions and evaluation procedures for enhanced compression tool testing, JVET-X2017, M. Karczewicz, Y. Ye.

[5] https://vcgit.hhi.fraunhofer.de/jvet-x-ee1/ecm-jvet-x-ee-1/-/tree/master .