| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0046-v1 |
| --- | --- |

| Title: | VTM Software Implementation for GREEN-MPEG SEI Messaging |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christian Herglotz Matthias Kränzler André Kaup | Email: | christian.herglotz@fau.de Matthias.kraenzler@fau.de Andre.kaup@fau.de |
| Source: | Multimedia Communications and Signal Processing, Friedrich-Alexander Universität Erlangen-Nürnberg (FAU) |  |  |

_____________________________

# Abstract

This document proposes a reference implementation for signaling the Green MPEG complexity metrics for decoder-power reduction and quality metrics for quality recovery after low-power encoding, which are both defined for VVC SEI messages. The implementation complies with the issued DIS ISO/IEC 23001-11 3rd edition Energy-Efficient Media Consumption (Green Metadata). The implementation is available for the VTM-16.0 encoder and decoder implementation. We propose to include the code for a future version of VTM.

# Introduction

In the January 2022 meeting of ISO/IEC JTC 1/SC 29/WG, the DIS of ISO/IEC 23001-11 3rd edition Energy-Efficient Media Consumption (Green Metadata) [2] was issued. In this document, SEI metadata for VVC for energy-efficient media consumption is specified. In particular, Chapters 5.2 and 8 of ISO/IEC 23001-11 3rd edition define syntax used in VVC SEI messages targeting complexity metrics for decoder-power reduction and metrics for quality recovery after low-power encoding, respectively. In this document, we present a software patch for the VTM encoder and decoder that implements the functionality of this syntax based on the implementation presented in [1]. In this respect, Section 2 presents the decoder implementation, Section 3 the encoder implementation, and Section 4 concludes this document.

# Green-Metadata Decoder Implementation

The Green-Metadata parsing code is implemented on top of VTM-16.0 and available at https://gitlab.lms.tf.fau.de/LMS/vtm-analyzer. The additional code is indicated by the preprocessor JVET_W0071_SEI [1]. The main functionality is included in the function “xParseSEIGreenMetadataInfo()“ in the file SEIread.cpp, which parses the SEI message and saves the data in the class SEIGreenMetadataInfo, which is derived from the SEI class. Parsing functionality is available for both decoder complexity metrics and quality recovery after low-power encoding (Chapters 5.2 and 8 of ISO/IEC 23001-11 3rd edition, respectively). The corresponding syntax is described in [2].

In addition, a dedicated parser for Green-MPEG metadata is available. It takes any VVC coded bit stream as in input and prints available Green-Metadata to the screen. The application is called “GreenMPEGAnalyzerApp” and part of the repository.

# Green-Metadata Encoder Implementation

For the generation of GREEN-Metadata in VVC SEI messages, functionality for generating complexity metrics, quality metrics, and for writing the corresponding syntax in the bit stream is implemented on top of VTM-16.0 and available at https://gitlab.lms.tf.fau.de/LMS/vtm-analyzer.

Concerning complexity metrics (CMs) for decoder-power reduction, the determination of the CMs is implemented using the bit stream feature analyzer (BSFA) presented in [3]. Originally, BSFA was restricted to the decoder. For SEI signalling, the functionality was extended to also count features during encoding. To determine CMs, the feature numbers of the subset of features corresponding to the CMs defined in [2] are exploited to determine the values of the CMs. The code for writing the syntax to the bit stream is wrapped by the preprocessor JVET_W0071_SEI.The BSFA implementation for the encoder is indicated by the preprocessor JVET_W0071_BSFA. The current implementation is restricted to write picture-wise CM signaling (period_type = 0x0; granularity_type = 0x0).

Concerning quality metrics (QMs) for quality recovery after low-power encoding, the code for determining picture-wise PSNR, WPSNR, and SSIM values is already included in the VTM software. We exploit the precomputed PSNR value and implemented the code for signaling the PSNR as SEI metadata using the preprocessor JVET_W0071_SEI. This implementation allows to write picture-wise PSNRs, which means that a segment is considered to be a single frame.

A full example of an encoder configuration is provided in the file cfg/sei_vui/greenMetadataSEI.cfg. Note that functionality in this implementation is only available for the cases described above.

# Conclusion

This document presented a reference implementation of the Green MPEG complexity metrics for decoder-power reduction and the quality recovery for low-power encoding defined for VVC SEI messages. The decoder implementation is fully compliant with the DIS of Green MPEG [2]. The encoder implementation allows writing SEI messages on picture granularity. We propose to adopt the implementation for a future version of the VTM software.

# References

[1] 	C. Herglotz, M. Kränzler, A. Kaup (FAU), E. Francois, M. Radosavljevic, E. Reinhard (InterDigital), X. Ducloux (Harmonic), D. Menard (INSA), Y. He, M. Coban, D. Rusanovskyy, M. Karczewicz (Qualcomm), “AHG9: Green Metadata SEI message for VVC”, JVET-W0071, July 2021.

[2] WG 04 MPEG Systems, “Text of ISO/IEC DIS 23001-11 3rd edition Energy-Efficient Media Consumption (Green Metadata)”, output document MDS21216_WG03_N00495, January 2022.

[3] M. Kränzler, C. Herglotz, A. Kaup: "Bit Stream Feature Analyzer (BSFA) for Coding Tool Statistics based on VTM-10.0", document JVET-T0067, Teleconference, October 2020.

# Patent rights declaration(s)

The Friedrich-Alexander Universität Erlangen-Nürnberg do not have any current or pending patent rights relating to the technology described in this contribution.