| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0054-v1 |
| --- | --- |

| Title: | EE2-2.1: On the extended number of active reference pictures and block level reference picture reordering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 USA | Email: | hanhuang@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution reports the test results of EE2 Test 2.1. In Test 2.1a, the number of active reference pictures in random access configurations is increased to the number of available reference pictures, and encoder fast algorithm is applied. It reports -0.21%, -0.25%, -0.32 BD-rate with 102% encoder time and 98% decoder time. In Test 2.1b, the block level reference picture reordering method is tested. It reports -0.10%, -0.10%, -0.08% BD-rate with 101% encoder time and 101% decoder time in random access, and -0.29%, -0.39%, -0.55% BD-rate with 103% encoder time and 107% decoder time in low delay B configurations. In Test 2.1c, the block level reference picture reordering method is tested with the increased number of active reference pictures. It reports -0.31%, -0.37%, -0.38% BD-rate with 103% encoder time and 101% decoder time in random access configurations.

# Introduction

In JVET-Y0139, the result of extending the number of active reference pictures in the random access configuration by setting the number of active reference pictures is equal to the number of available reference pictures are reported. A block level reference picture reordering method based on template matching was also proposed. For the uni-prediction AMVP mode, the reference pictures in List 0 and List 1 are interweaved to generate a joint list. For each hypothesis of the reference picture in the joint list template matching is performed to calculate the cost. The joint list is reordered based on ascending order of the template matching cost. The index of the selected reference picture in the reordered joint list is signaled in the bitstream. For the bi-prediction AMVP mode, a list of pairs of reference pictures from List 0 and List 1 is generated and similarly reordered based on the template matching cost. The index of the selected pair is signaled in the bitstream. It was decided to study the proposed method in EE2.

# Implementation in ECM-4.0

As reported in merge request 99, MHP bug fix is needed to enable extended number of active reference pictures in ECM-4.0. The BD-rate performance impact under common test condition is 0.00%, 0.02%, -0.02% in random access, and -0.06%, -0.08%, -0.15% in low delay B configurations. The detail results are shown in Table 1 (runtime is not accurate, but it is supposed to be no impact on runtime). The bug fix is applied in both Test 2.1a and Test 2.1c.

Table 1 BD-rate performance impact of MHP bug fix

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.03% | 84% | 91% |
| Class A2 | 0.01% | 0.01% | 0.01% | 94% | 100% |
| Class B | -0.01% | 0.02% | -0.02% | 116% | 122% |
| Class C | -0.01% | 0.04% | -0.09% | 94% | 105% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | -0.02% | 98% | 106% |
| Class D | 0.04% | -0.01% | 0.04% | 95% | 104% |
| Class F | -0.02% | 0.00% | -0.04% | 94% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.10% | 0.03% | 80% | 87% |
| Class C | -0.05% | 0.38% | -0.38% | 84% | 96% |
| Class E | -0.19% | -0.65% | -0.15% | 91% | 99% |
| Overall | -0.06% | -0.08% | -0.15% | 84% | 93% |
| Class D | -0.01% | 0.43% | 0.81% | 85% | 95% |
| Class F | 0.06% | 0.41% | -0.08% | 90% | 93% |

To reduce the complexity in the motion estimation process for the extended number of active reference pictures, two early skip strategies are applied:

- The reference picture with the index larger than 1 in the affine motion estimation process is skipped if it is not selected in the regular AMVP mode.

- The reference picture with the index larger than 1 is skipped if the ME cost for the reference index 1 is larger than 1.25 times the cost of the reference index 0.

In ECM-4.0, MVD sign prediction is applied to the regular and affine AMVP modes. The derivation of the predicted MVD signs requires the reference pictures to be known. However, the reference picture reordering method requires the MVDs to be known in the process. To solve this problem, the minimum template matching cost among all MVD sign hypothesis is assigned to the reference picture hypothesis. Then, the selected reference picture can be determined by the decoded index and the reordered reference picture list. The MVD sign prediction is performed thereafter by reusing the calculated template matching cost. For simplification, in the case of bi-prediction, MVD sign prediction in List 1 is only enabled if MVD in list 0 is zero.

# Test results

The 3 tests of Test 2.1 were performed on top ECM-4.0. Test 2.1a reports -0.21%, -0.25%, -0.32 BD-rate with 102% encoder time and 98% decoder time. Test 2.1b reports -0.10%, -0.10%, -0.08% BD-rate with 101% encoder time and 101% decoder time in random access, and -0.29%, -0.39%, -0.55% BD-rate with 103% encoder time and 107% decoder time in low delay B configurations. Test 2.1c reports -0.31%, -0.37%, -0.38% BD-rate with 103% encoder time and 100% decoder time in random access configurations. Details of the test results are shown in Table 2, Table 3 and Table 4.

Table 2 Results of Test 2.1a

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.16% | -0.29% | 102% | 97% |
| Class A2 | -0.02% | -0.03% | -0.07% | 102% | 97% |
| Class B | -0.32% | -0.36% | -0.40% | 102% | 98% |
| Class C | -0.29% | -0.36% | -0.42% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.21% | -0.25% | -0.32% | 102% | 98% |
| Class D | -0.04% | -0.18% | -0.05% | 101% | 99% |
| Class F | -0.22% | -0.17% | -0.37% | 102% | 96% |

Table 3 Results of Test 2.1b

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.20% | -0.02% | 100% | 100% |
| Class A2 | -0.12% | -0.16% | -0.21% | 100% | 100% |
| Class B | -0.10% | -0.04% | 0.01% | 101% | 101% |
| Class C | -0.11% | -0.04% | -0.13% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.10% | -0.10% | -0.08% | 101% | 101% |
| Class D | -0.13% | -0.09% | -0.06% | 101% | 103% |
| Class F | -0.11% | -0.14% | -0.17% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.23% | -0.38% | -0.73% | 103% | 106% |
| Class C | -0.28% | -0.29% | -0.61% | 103% | 108% |
| Class E | -0.42% | -0.55% | -0.17% | 103% | 106% |
| Overall | -0.29% | -0.39% | -0.55% | 103% | 107% |
| Class D | -0.39% | 0.18% | -0.59% | 102% | 112% |
| Class F | -0.21% | 0.58% | -0.55% | 102% | 107% |

Table 4 Results of Test 2.1c

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.36% | -0.42% | 103% | 99% |
| Class A2 | -0.15% | -0.30% | -0.32% | 103% | 100% |
| Class B | -0.40% | -0.39% | -0.38% | 102% | 99% |
| Class C | -0.39% | -0.42% | -0.40% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.31% | -0.37% | -0.38% | 103% | 100% |
| Class D | -0.20% | -0.16% | -0.18% | 101% | 101% |
| Class F | -0.34% | -0.45% | -0.45% | 102% | 99% |

# Conclusion

In this contribution, the test results of EE2 Test2.1 are reported. It’s suggested to adopt Test 2.1c in the next version of ECM.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).