| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0057-v3 |
| --- | --- |

| Title: | On subjective evaluation of video quality with the crowdsourcing approach |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Babak Naderi Ross Cutler | Email: | babak.naderi@tu-berlin.de Ross.Cutler@microsoft.com |
| Source: | TU-Berlin, Microsoft |  |  |

_____________________________

# Abstract

This contribution presents an open-sourced framework for conducting subjective video quality assessment tests using crowdsourcing. The platform is under study in ITU-T SG12/Q7 work item P.CrowdV. The aim of this contribution is to seek feedback from JVET experts and to suggest considering to use this framework for subjective testing in JVET work.

# Introduction

The gold standard for measuring video quality is the subjective test, and the most prevalent is the ITU-T Rec. P.910 [1], a lab-based subjective standard in use for the past two decades.  However, in practice using ITU-T Rec. P.910 is slow, expensive, and requires a lab, which all create barriers to usage for many research bodies. As a result, most research papers that need to measure video quality don't use subjective test but rather metrics that are not well correlated to subjective opinion. We provide an open-sourced framework for conducting video quality assessment tests using crowdsourcing.  It addresses the speed, usage cost, and barrier to usage issues. We implement Absolute Category Rating (ACR), ACR with hidden reference (ACR-HR), and Degradation Category Rating (DCR). It includes rater, environment, hardware, and network qualifications, as well as gold and trapping questions to ensure quality. We have validated that the implementation is both accurate and highly reproducible compared to existing P.910 based lab studies.

This contribution summarizes our paper which can be found in [2].

An alternative to lab-based subjective tests is to crowdsource the testing, and there are several such systems that do this (see Table 1). However, none of the systems that we are aware of have the rater, environment conditions, and hardware qualifications that P.910 requires. In addition, these existing systems have not been rigorously validated to show they are accurate compared to a P.910 lab-based study and give reproducible results.

Table 1. Open-source crowdsourcing video quality assessment systems.

| Tool | Measures | Rater qual. | Viewing cond. | Hardware | Network | Accur. | Reproducibility |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QualityCrowd [3] [4] | ACR, DSCQS | N | N | N | N | N | N |  |
| WESP [5] | ACR, ACR-HR, DCR, PC | N | N | N | N | N | N |  |
| avrateNG [6] | ACR | N | N | N | N | Y | N |  |
| Ours | ACR, ACR-HR. DCR | Y | Y | Y | Y | N |  | Y |

# Implementation

Our implementation can be used either as an integrated survey in Amazon Mechanical Turk (AMT) or as an external survey deployed on a dedicated Web server. Currently the integrated surveys within AMT are not able to play back videos in full-screen mode, therefore we strongly recommend to deploy the test as an external survey. Consequently, AMT or any other crowdsourcing platform can be used for recruiting and paying test participants.

We also provide a lightweight container-based web application that can serve the experiment. The web application can easily be deployed on any Linux virtual machine. As a result, this implementation can be used for both crowdsourcing tests or remote testing with a dedicated panel of participants.

## Tools

We provide a set of program scripts to ease the interaction with the system and avoid operation errors. The scripts are used to prepare the subjective test (e.g., create customized trapping sequences), process the submitted answers (i.e., data cleansing, aggregating ratings, and generating reports), and interact with AMT (e.g., sending bonuses to workers).

Figure 1. Data Flow diagram

The entire process is visualized in Figure 1, and briefly described here:

- trapping clips customized to the dataset under the test should be created using the corresponding script. They will be created by adding a text message, in the middle of selected existing video sequences, asking participants to select a specific score when rating this trapping clip.

- test configuration and URLs for test sequences, trapping clips, gold, and training clips should be provided to master script. It creates the HTML template of the test, list of variables (dynamic content), and configuration file for result parser.

- Create the test in the HIT App Server by providing the HTML template and a list of variables generated in the last step. Finally, a generic project description and list of URLs can be downloaded.

- Create a new external survey test in AMT (or any other crowdsourcing platform) using the template and list of URLs. Now the test will be started and crowdworkers can take part in the study.

- When the test is finished, the submitted answers to AMT, and HIT App Server should be provided to the result parser script in addition to the configuration file which was created in step 2. It will perform the data cleansing process, and aggregate the valid and reliable ratings over test sequence and over Hypothetical Reference Circuits (HRCs) if exist. In addition, reports on the list of bonus assignments, and lists of the submissions to be accepted/rejected or extended will be generated.

- Based on the reports, worker’s submissions should be accepted/rejected and bonuses should be assigned.

## Test Components

The subjective test is organized in different sections from the participant's perspective (see Figure 2).

The instructions and ratings sections are included in all tests, whereas the qualification, and calibration only need to be performed once per test. The setup and training sections are shown periodically.

In the following each section is briefly discussed. For further details please check [1].

Figure 2. Different sections of subjective test from the participant's perspective

### Qualification

Within the qualification section, the eligibility of test participants is evaluated. According to the ITU-T Rec. P.910, participants should usually be screened for normal color vision and normal or corrected-to-normal visual acuity (i.e., no error on the 20/30 line of a standard eye chart).

We use 2 plates from Ishihara test for color vision check which in our pretests showed to reach 98% accuracy on detection.

For the visual acuity test, we use the Landolt ring optotypes. Our implementation of the visual acuity test consists of two steps: 1) setup, 2) answer to up to 5 Landlot rings on the specific size. In the setup section, the size of a pixel in the user’s screen is estimated by asking them to adapt the size of a given picture (here a credit card) on their screen by clicking on +/- buttons to reach the specified size. In addition, we asked them to sit in the range of 50 to 75cm from the screen during the test. Given the pixel size, the corresponding Landlort ring size is calculated in which correctly answering them in 50 cm distance shows a minimum of 20/30 visual acuity. The participant passes the visual acuity test if they correctly select the direction of three out of the five Landlort rings.

### Display calibration and instructions

Participants are asked to set the resolution of their device to default or recommended value suggested by their operating system. We also ask them to perform display color calibration using methods provided by their operating system. In the instruction section, a sample video for some of the perceptual quality dimensions (c.f. ITU-T Rec. P.918) (i.e., fragmentation, discontinuity, and uncleanness) are presented to the participants. They are also informed that impairments can happen in a specific area or time within a video clip and the impairments are not limited to the presented samples.

### Setup

In the setup section, the viewing conditions including the brightness of the screen, background room light, and viewing distance are evaluated. The ITU-T Rec. P.910 provides a detailed specification for viewing conditions that can only be measured in a controlled laboratory environment using professional devices. We created a brightness/light calibration task in which participants should count the number of geometric shapes presented in a picture. They can insert their answers and verify them. Given the correct answer, they can continue to the next steps whereas on mismatch they are encouraged to modify the brightness of their screen, lighting of the environment, or repeat the display calibration task until they can insert the correct number for each picture. The presented picture is a matrix of 4x4 squares. Each square has a different level of a gray background and may contain a triangle or circle in different sizes and locations. The foreground color of the shapes is close to the background color of the square. A pretest, with a limited group of experts, showed that the calibration task performs best when the difference between foreground and background colors are four integer points in RGB24 color space. Figure 3 illustrates an example matrix picture used in our tests. Within the setup section, a second matrix is also presented for which no feedback is given.

Figure 3. Matrix of squares with 4 circles and 10 triangles used for evaluating screen calibration.

The crowdsourcing test also includes a short paired-comparison task to evaluate participants' viewing distance. In this task, three pairs of images are presented to the participants who are asked to sit at a distance of 1.5 times their screen height. In each pair, one of the images is distorted with a blur effect. Participants are asked to select the image of better quality. They can choose between either of the images or select that both have the same quality.

In a separate experiment, we used blur effects with different radiuses from 1 pixel to 9. A small group of participants took part in six Just-Noticeable Difference (JND) quality tests (adaptive staircase method 2 down–1 up) with two different screen sizes and three different viewing distances (too close, expected, too far). Using the results, we selected three pairs for this task: 1) the difference is mostly recognized if the participant is too close to the screen, 2) the difference is correctly recognized if the participant is seated in the expected distance, 3) the difference is still recognized even they are too far from their screen. Given their response, there might be a feedback message asking them to adjust their seating distance to 1.5 times their screen height. An example from the paired-comparison questions is given in Figure 4.

Figure 4. One of the questions used in paired-comparison test to check viewing distance. Participants recognize the difference mostly when they are too close to their display.

### Training and rating

In the training section, participants should watch the videos from the training set and rate their quality. The training set should represent the dataset under the test and cover the entire range of the rating scale. We also add a trapping clip to the training set, which instructs the participant to make a specific selection with text in the video. Finally, the training section is shown periodically (every 60 minutes) to keep its anchoring effect. Within the rating section, participants rate the quality of ten video clips, one trapping, and one gold clip. The trapping and gold clip will be inserted automatically and will be used in the data cleansing step in post-processing.

# Validation

We used the VQEG HDTV datasets [7] to evaluate the validity and reliability of our implementation.

The datasets contain coding only and coding plus transmission error impairments. They were created to validate objective video quality models that predicted the quality of High Definition Television (HDTV). The video materials and subjective data from experiments VQEGHD1-5 (in a laboratory) are made publicly available in the Consumer Digital Video Library. Each experiment includes 168 video clips. We used the datasets from experiments VQEGHD3 and VQEGHD5 for which 40" (native resolution 1920x1080) and 24" (native resolution 1920x1200) displays were used in the laboratory viewing sessions by 24 test participants, respectively. The test sequences have various bit rates; 1-15 Mbps for VQEGHD3 and 2-16 Mbps for VQEGHD5.

To prepare the video clips for our crowdsourcing experiments, we re-encode the video sequences using FFmpeg with the H.264/AVC codec keeping the original bit-rate, frame-rate, and resolution with a GOP of one second and Constant Rate Factor of 17.

We conducted six crowdsourcing studies with the two above-mentioned datasets. In one of the tests, the VQEGHD5 dataset was used. In the rest of five tests, we used VQEGHD3 dataset on five separate days, each with unique raters, to evaluate the reproducibility of our implementation.

In all tests, participants with minimum display resolution of 1280x720 and refresh rate of 30Hz were allowed.

## Accuracy

In each experiment, 10 test sequences, one gold clip, and one trapping clip were presented in one session and we aimed to collect 30 ratings per clip.

In the five experiments using the VQEGHD3 dataset on average 71% of submissions passed the data cleansing step; the rest were not used due to the rater providing a wrong answer to the gold question, longer play back duration, failure on the second brightness check, low variance in ratings, or wrong verification code. On average we have 21 accepted votes per test sequence (with a minimum of 15 ratings).

Table 2. Comparison between laboratory and crowdsourcing

| Dataset | PCC | SPCC | RMSE after mapping |
| --- | --- | --- | --- |
| VQEG HDTV3 -run1 | 0.956 | 0.949 | 0.333 |
| VQEG HDTV3 -run2 | 0.964 | 0.951 | 0.302 |
| VQEG HDTV3 -run3 | 0.959 | 0.949 | 0.323 |
| VQEG HDTV3 -run4 | 0.917 | 0.913 | 0.455 |
| VQEG HDTV3 -run5 | 0.947 | 0.923 | 0.367 |
| VQEG HDTV5 | 0.970 | 0.957 | 0.278 |

The results are presented in Table 2 per test sequences. We observed a strong correlation between laboratory and crowdsourcing subjective tests PCC = 0.952 per test sequence and 0.964 per HRC). The distribution of MOS values from crowdsourcing and laboratory tests, before mapping, are illustrated in Figure 5.

| - HDTV DB5 | - HDTV DB3 | - reproducibility |
| --- | --- | --- |

Figure 5 - Scatter plots showing the distribution of MOS values in different crowdsourcing studies

## Reproducibility

On average 63 unique workers participated in each run. We observed PCC=0.971 and SRCC=0.95 on average between MOS values of sequences in the five different runs. The correlation coefficients between the runs are reported in Table 3.

We also fitted a linear mixed-effects model (LMEMs) with random intercept in which test sequences and runs considered as fixed factors and participant as random factor. Result shows there was no significant effect of run on the subjective ratings (χ2(4)=2.691, p=0.611).

Table 3. Correlation coefficients between five runs of the VQEGHD3 dataset. Pearson correlation coefficient on upper triangle and Spearman’s rank correlation coefficient on lower triangle.

|  | Run 1 | Run 2 | Run 3 | Run 4 | Run 5 |
| --- | --- | --- | --- | --- | --- |
| Run 1 |  | 0.984 | 0.987 | 0.957 | 0.977 |
| Run 2 | 0.959 |  | 0.985 | 0.957 | 0.977 |
| Run 3 | 0.974 | 0.969 |  | 0.952 | 0.972 |
| Run 4 | 0.943 | 0.941 | 0.942 |  | 0.956 |
| Run 5 | 0.954 | 0.947 | 0.942 | 0.933 |  |

## Ablation study

We evaluated the effect of different components on the performance of the crowdsourcing test.

In pretests, we observed that percentage of correct answers to brightness check increases by 19% when participants are asked to perform screen calibration.

In 24.8% of submissions, the play back duration was longer than 1.15 times of the original duration of the videos. After a software update, in which videos are fully downloaded in advance, only 5.06% of cases showed longer play back duration than expected.

Using the data from reproducibility tests, we evaluated the performance of the gold clips, play back duration, brightness check, and straightliner detection by comparing the MOS values from laboratory test (as the ground truth) with the MOS values calculated from submissions that passed or failed in each criterion.

In addition, we conducted multiple crowdsourcing experiment to evaluate the effect of 1) viewing distance test 2) visual acuity test (VAT), 3) display calibration and 4) trapping clips. In each of them the test had to be modified for the aim of that test. For details please refer to [2].

Results are presented in Table 4. The correlation coefficients between the laboratory scores and the aggregated scores that passed all the criteria are significantly higher than the other groups. Similarly, the RMSEs were significantly lower.

# Conclusions

We have created the first crowd-sourced implementation of P.910 that is shown to be as accurate as a lab study and is highly reproducible. In a set of experiments, we showed that each test component significantly improves the validity of collected ratings.

It should be noted that the impact of the test components depends on the reliability of crowd workers (and their environment and device) and the test sequences under the study. If a dedicated group of workers performs the test in a suitable environment using proper devices, those components might look redundant. However, they strongly increase the validity of collected scores despite the group of test participants and make large-scale data collection possible.

We plan to include the simultaneous presentation of sequence pairs for future work and implement P.910 Annex E: An advanced data analysis technique for tests under challenging conditions.

We would kindly ask experts of JVET to consider using our framework and provide feedback which we can use in drafting P.CrowdV recommendation text.

Table 4. Effect of each component on the accuracy of test results

| Case | PCC | SPCC | RMSE | RMSE after mapping |
| --- | --- | --- | --- | --- |
| All passed | 0.96 | 0.96 | 0.62 | 0.31 |
| Gold clips failed | 0.57 | 0.53 | 1.02 | 0.93 |
| Play back duration failed | 0.62 | 0.57 | 1.12 | 0.89 |
| Brightness check failed | 0.89 | 0.88 | 0.84 | 0.52 |
| Straightliners | 0.29 | 0.30 | 1.53 | 1.09 |
| Viewing distance - passed | 0.93 | 0.92 | 0.76 | 0.41 |
| Viewing distance - failed | 0.83 | 0.78 | 0.95 | 0.62 |
| VAT Passed & All criteria passed | 0.91 | 0.90 | 0.88 | 0.47 |
| VAT Failed & All criteria passed | 0.87 | 0.89 | 0.96 | 0.59 |
| Complete test -all passed | 0.95 | 0.95 | 0.72 | 0.34 |
| No calibration | 0.91 | 0.89 | 0.80 | 0.34 |
| No Trapping clip | 0.92 | 0.92 | 0.88 | 0.46 |

# References

| [1] | ITU-T Recommendation P.910, Subjective video quality assessment methods for multimedia applications, Geneva: International Telecommunication Union, 2021. |
| --- | --- |
| [2] | B. Naderi and R. Cutler, A Crowdsourced Implementation of ITU-T P.910, arXiv, 2022. |
| [3] | C. Keimel, J. Habigt, C. Horch and K. Diepold, "QualityCrowd-A Framework for Crowd-based Quality Evaluation," 2012. |
| [4] | E. Upenik, M. Testolina, J. Ascenso, F. Pereira and T. Ebrahimi, Eds."Large-Scale Crowdsourcing Subjective Quality Evaluation of Learning-Based Image Coding," IEEE Visual Communications and Image Processing (VCIP 2021), 2021. |
| [5] | B. Rainer, M. Waltl and C. Timmerer, "A web based subjective evaluation platform," in 2013 Fifth International Workshop on Quality of Multimedia Experience (QoMEX), 2013. |
| [6] | R. R. R. Rao, S. Göring and A. Raake, "Towards High Resolution Video Quality Assessment in the Crowd," in 2021 13th International Conference on Quality of Multimedia Experience (QoMEX), 2021. |
| [7] | Video Quality Experts Group, "Report on the validation of video quality models for high definition video content," 2010. |
| [8] | ITU-T Recommendation P.808, Subjective evaluation of speech quality with a crowdsourcing approach, Geneva: International Telecommunication Union, 2021. |
| [9] | C. Keimel, J. Habigt, C. Horch and K. Diepold, "Qualitycrowd—a framework for crowd-based quality evaluation," in Picture Coding Symposium (PCS), 2012, 2012. |

https://github.com/microsoft/P.910

Gold clips are video sequences whose quality are known to the experimenter (either excellent or bad) [8]

https://www.cdvl.org