| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0059 |
| --- | --- |

| Title: | Non-EE2: Adaptive width for GPM blending area |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Haruhisa Kato Kyohei Unno Kei Kawamura (KDDI) 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 70 3508 5829 yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes an adaptive width for geometric partitioning mode (GPM) blending area to further enhance coding performance of ECM. In VVC spec and the current ECM design, GPM can select only one fixed width for GPM blending area to derive the final prediction samples with the two prediction signals of each part of GPM. This is not always optimal for diverse video sequences. Therefore, the proposed method enables GPM to select the adaptive blending width by signalling among the three different widths: the same width as that of VVC and ECM, a quarter of that width, and four times of that width. Simulation test results of over all classes on the top of ECM-4.0 in RA and LB are reported as follows:

- RA (Overall): BD-rate Y/U/V: -0.15%/-0.18%/-0.14%; EncT: 101%; DecT: 99%

- LB (Overall): BD-rate Y/U/V: -0.23%/-0.17%/-0.32%; EncT: 102%; DecT: 99%

Moreover, an optional signalling that restricts selectable width only to the narrowest width, is proposed to optimize the proposed method for screen content (SCC). Simulation test results show that it provides an additional coding gain for Class TGM but not in Class F because of the coding loss in BasketballDrillText. The comparison results of Class TGM in RA/LB for the proposed method without and with the option as follows:

- Proposal w/o optional signalling for SCC

  - RA (TGM): BD-rate Y/U/V: -2.37%/-1.76%/-1.70%; EncT: 100%; DecT: 98%

  - LB (TGM): BD-rate Y/U/V: -3.27%/-2.62%/-2.51%; EncT: 100%; DecT: 97%

- Proposal w/ optional signalling for SCC

  - RA (TGM): BD-rate Y/U/V: -2.50%/-1.94%/-1.83%; EncT: 99%; DecT: 97%

  - LB (TGM): BD-rate Y/U/V: -3.54%/-2.89%/-2.71%; EncT: 99%; DecT: 96%

Introduction

In GPM of VVC spec [1] and the current ECM design [2], the weighted prediction, i.e., blending, is applied to the two prediction signals for each part of GPM derived by its own motion vector or its own intra prediction mode, in order to generate the final prediction samples around GPM partitioning boundary.

The two integer blending matrices (W0 and W1) are utilized for the GPM blending process. The weights in the GPM blending matrices contain the value range of [0, 8] and are derived based on the displacement from a sample position to the GPM partition boundary as shown in Figure 1.

Figure 1 Exemplified generation of a blending weight W0 of GPM

Specifically, the weights are given by a discrete ramp function with the displacement and two thresholds as shown in Figure 2.

Figure 2 The ramp function for the weight of GPM derived based on the displacement from a sample position to the GPM partition boundary and the fixed threshold τ.

Here, the threshold τ defines the width of the GPM blending area and is selected as the fixed value in VVC spec [1] and the current ECM design [2].

However, the fixed value, i.e., the fixed width for the GPM blending area, is not always optimal for diverse video sequences. For example, the wide width is better for the video sequences with motion or focus blurs, whereas narrow width is better for those without these blurs or for screen content.

Proposed methods

Adaptive width of GPM blending area

To tackle the problem, we propose an adaptive width for GPM blending area. Specifically, the optimal width is selected by signalling from a total of three different widths: the same width as VVC and ECM (τ), a quarter of that width (τ/4), and four times of that width (4τ), as shown in Figure 3.

Figure 3 The ramp functions for the weight of GPM derived based on the displacement from a sample position to the GPM partition boundary and the adaptive thresholds: τ, τ/4, and 4τ.

Regarding signalling, the selectable widths are restricted based on the current block size to maximize the coding performance. When the shorter side of the current block is equal to or smaller than 16 samples, only two narrower widths (τ, τ/4) can be selected; otherwise only two wider widths (τ, 4τ) can be selected.

In addition, the maximum value of the weighs is changed from 8 to 32 to maximize the effect especially for wider widths.

Optional signalling for SCC

Furthermore, aim to optimize the proposed method for screen content, we also propose an optional signalling that restricts the selectable width only to the narrowest width (τ/4) irrespective of the current block size. It can be realized by a control flag on the high-level syntax as like JVET-Q0060 [3], which proposes non-blending (i.e., the width of the blending area is zero).

Test Results

The proposed methods are implemented on top of ECM-4.0 [2]. The simulation tests were conducted following the common test condition and evaluation procedures for enhanced compression tool testing [4]. Test results under RA and LB configurations are summarized in Table 1. From Table 1, the proposed method provides the coding performance gain with -0.15%/-0.23% BD-rate saving in RA/LB compared to ECM-4.0 without increment of encoder and decoder runtime.

Additionally, the test results of the proposed method including the additional signalling restriction only for Class F and Class TGM are shown in Table 2. From Table 2, the additional signalling restriction provides the additional coding gain for Class TCM but not Class F because of the coding loss in the non-pure screen content BasketballDrillText.

More detailed results can be found in the attached spreadsheets.

Table 1 Test results of the proposed method on the top of ECM-4.0.

|  |  |  | RA |  |  |  |  | LB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | -0.16% | -0.47% | -0.18% | 101% | 99% |  |  |  |  |  |
| A2 | -0.06% | -0.12% | 0.00% | 101% | 100% |  |  |  |  |  |
| B | -0.13% | -0.10% | -0.27% | 101% | 99% | -0.13% | -0.26% | -0.19% | 102% | 99% |
| C | -0.24% | -0.13% | -0.05% | 101% | 99% | -0.36% | 0.16% | -0.52% | 102% | 99% |
| E |  |  |  |  |  | -0.21% | -0.44% | -0.26% | 102% | 100% |
| Overall | -0.15% | -0.18% | -0.14% | 101% | 99% | -0.23% | -0.17% | -0.32% | 102% | 99% |
| D | -0.26% | -0.28% | -0.07% | 102% | 100% | -0.42% | 0.18% | -0.68% | 103% | 101% |
| F | -0.61% | -0.67% | -0.67% | 102% | 99% | -0.72% | -0.40% | -1.00% | 102% | 100% |
| TGM | -2.37% | -1.76% | -1.70% | 100% | 98% | -3.27% | -2.62% | -2.51% | 100% | 97% |

Table 2 Test results of the proposed method including the additional signalling restriction on the top of ECM-4.0 (tool-off test).

|  |  |  | RA |  |  |  |  | LB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| F | -0.45% | -0.34% | -0.24% | 100% | 100% | -0.59% | 0.50% | -0.54% | 100% | 100% |
| TGM | -2.50% | -1.94% | -1.83% | 99% | 97% | -3.54% | -2.89% | -2.71% | 99% | 96% |

Supplemental Test Results

Table 3 shows the supplemental test results where the non-blending method proposed by JVET-Q0060 is implemented on the top of ECM-4.0. From the comparison of Table 2 and Table 3, the proposed method

Table 3 Test results of the proposed method including the additional signalling restriction on the top of ECM-4.0 (tool-off test).

|  |  |  | RA |  |  |  |  | LB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| F | -0.31% | -0.18% | 0.06% | 100% | 100% | -0.18% | 0.56% | 0.02% | 100% | 102% |
| TGM | -2.47% | -1.80% | -1.69% | 99% | 98% | -3.52% | -2.67% | -2.49% | 99% | 98% |

Conclusion

In this contribution, the adaptive width for GPM blending area methods were proposed. The proposed methods can bring some coding gains with a good trade-off between coding performance and complexity. It is recommended to adopt the proposed methods into the next EE2.

This work was supported by Ministry of Internal Affairs and Communications (MIC) of Japan (Grant no. JPJ000595).

Reference

- B. Bross, J. Chen, S. Liu, and Y. K. Wang, “Versatile Video Coding Editorial Refinements on Draft 10,” JVET-T2001, by teleconference, 7-16, October. 2020.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-4.0

- H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, and M. Bläser, J. Sauer, “VCE4-4: Geometric Inter Prediction with Adaptive Blending for SCC,” JVET-Q0060, Brussels, 7–17 January 2020.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, by teleconference, 12-21, January. 2022.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).