| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0079-v3 |
| --- | --- |

| Title: | Non-EE2: Support RPR on ECM-4.0 and handling method for template matching based coding tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Naeri Park Jaehyun Lim Seung-hwan Kim | Tel: Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to support RPR on ECM-4.0 and introduce a handling method for template matching based coding tools. Firstly, it is observed encoder crash in ECM-4.0 when RPR is enabled because RPR case is not considered for IPM mode storing process for GPM intra-inter. Therefore this proposal suggests to fix the bug. Secondly, this proposal further suggests to set maximum cost so that calculating template matching cost can be bypassed when the reference picture is resampled for the reordering process of MMVD, Affine MMVD, MVSD, TMVP, NAMVP and AML. Most of coding tools handled by this proposed method were adopted in the last JVET meeting. The implementation of this coding tools carry out the template matching process to calculate cost even though the reference picture of the candidates are resampled. However, it has been decided to not apply template matching process for RPR at the last JVET meeting. Therefore this contribution also suggests to disallow calculating template matching based methods for RPR.

Problem Statement

Problem Statement 1

Current implementation of GPM intra-inter coding tools has lack of considering for RPR case. For example, during spanning the IPM, encoder can access the position out of the reference picture boundary. From this illegal access, the encoder crash can occur when RPR is enabled.

Problem Statement 2

The template matching based coding tools adopted in the last JVET meeting do not properly consider RPR case. Even though the reference picture of the candidate is resampled, template matching process to calculate the cost is applied. Note that it has been decided to not apply the template matching process for RPR in the last JVET meeting.

Proposed method

Proposed method 1

In order to avoid encoder crash in ECM-4.0 with RPR configuration, it is proposed to consider RPR case for the spanning GPM intra-inter process. The proposed implementation is the same way as other coding tools which are implemented considering RPR case.

ECM-4.0 spanGeoMotionInfo()

Position PosY; PosY.x = pu.Y().x + (x << MIN_CU_LOG2) + cMv.getHor(); PosY.y = pu.Y().y + (y << MIN_CU_LOG2) + cMv.getVer(); clipColPos(PosY.x, PosY.y, pu); PosY.x = (PosY.x & mask); PosY.y = (PosY.y & mask); ipm = pu.cu->slice->getRefPic(RefPicList(list), refIdx)->cs->getIpmInfo(PosY);

Proposed implementation for spanGeoMotionInfo()

Position PosY; PosY.x = pu.Y().x + (x << MIN_CU_LOG2) + cMv.getHor(); PosY.y = pu.Y().y + (y << MIN_CU_LOG2) + cMv.getVer(); clipColPos(PosY.x, PosY.y, pu); scalePositionInRef(pu, *pu.cs->pps, RefPicList(list), refIdx, PosY); PosY.x = (PosY.x & mask); PosY.y = (PosY.y & mask); ipm = pu.cu->slice->getRefPic(RefPicList(list), refIdx)->unscaledPic->cs->getIpmInfo(PosY);

Proposed method 2

Proposed method 2.1

For the template matching cost during AML, if the reference picture of the candidate is RPR then the template matching process is bypassed, and the TM cost for that candidate is set to 0 while the TM cost is set to maximum for motion vector refinement for the RPR. This misaligned cost setting causes different priority during sorting process for the candidates which refers RPR picture. In order to have unified design philosophy, it is proposed to set the cost to maximum value if the reference picture is RPR.

Proposed implementation for the calculation of template matching cost at adjustAffineMergeCandidates() at adjustInterMergeCandidates ()

uiCost = 0; … bool bRefIsRescaled = false; for (uint32_t refList = 0; refList < NUM_REF_PIC_LIST_01; refList++) { const RefPicList eRefPicList = refList ? REF_PIC_LIST_1 : REF_PIC_LIST_0; bRefIsRescaled ¦= (pu.refIdx[refList] >= 0) ? pu.cu->slice->getRefPic(eRefPicList, pu.refIdx[refList])->isRefScaled(pu.cs->pps) : false; } if (!bRefIsRescaled) { getBlkAMLRefTemplate(pu, pcBufPredRefTop, pcBufPredRefLeft); if (m_bAMLTemplateAvailabe[0]) { m_pcRdCost->setDistParam(cDistParam, pcBufPredCurTop.Y(), pcBufPredRefTop.Y(), pu.cs->sps->getBitDepth(CHANNEL_TYPE_LUMA), COMPONENT_Y, false); uiCost += cDistParam.distFunc(cDistParam); } if (m_bAMLTemplateAvailabe[1]) { m_pcRdCost->setDistParam(cDistParam, pcBufPredCurLeft.Y(), pcBufPredRefLeft.Y(), pu.cs->sps->getBitDepth(CHANNEL_TYPE_LUMA), COMPONENT_Y, false); uiCost += cDistParam.distFunc(cDistParam); } } else { uiCost = std::numeric_limits<Distortion>::max(); }

Additionally for the template matching cost based on reordering of the MMVD, Affine MMVD, TMVP and NAMVP, if the reference picture of the candidate is resampled, the template cost for the candidate is set to maximum value so that the calculating TM cost is bypassed.

Proposed implementation for the calculation of template matching cost at adjustMergeCandidatesInOneCandidateGroup() at sortInterMergeMMVDCandidates() at sortAffineMergeCandidates()

bool bRefIsRescaled = false; for (uint32_t refList = 0; refList < NUM_REF_PIC_LIST_01; refList++) { const RefPicList eRefPicList = refList ? REF_PIC_LIST_1 : REF_PIC_LIST_0; bRefIsRescaled ¦= (pu.refIdx[refList] >= 0) ? pu.cu->slice->getRefPic(eRefPicList, pu.refIdx[refList])->isRefScaled(pu.cs->pps) : false; } if (!bRefIsRescaled) { getBlkAMLRefTemplate(pu, pcBufPredRefTop, pcBufPredRefLeft); if (m_bAMLTemplateAvailabe[0]) { m_pcRdCost->setDistParam(cDistParam, pcBufPredCurTop.Y(), pcBufPredRefTop.Y(), pu.cs->sps->getBitDepth(CHANNEL_TYPE_LUMA), COMPONENT_Y, false); uiCost += cDistParam.distFunc(cDistParam); } if (m_bAMLTemplateAvailabe[1]) { m_pcRdCost->setDistParam(cDistParam, pcBufPredCurLeft.Y(), pcBufPredRefLeft.Y(), pu.cs->sps->getBitDepth(CHANNEL_TYPE_LUMA), COMPONENT_Y, false); uiCost += cDistParam.distFunc(cDistParam); } } else { uiCost = std::numeric_limits<Distortion>::max(); }

Proposed method 2.2

Unlike above mentioned template matching processes for merge candidate, MVD sign prediction considering RPR can be applied based on decoded reference picture which is indicated by previous decoded reference index before MVD. Other coding tools do not know if the reference picture of candidates is RPR or not until candidates are constructed. However AMVP knows if the reference picture is RPR or not before MVD is coded by the decoded reference index. Therefore this proposal suggests to handle MVD sign prediction during syntax parsing for each reference list. Specifically, if the reference index indicates RPR reference picture, MVD sign prediction is disabled and sign syntax for MVD is signalled after magnitude not sign-pair index for the reference picture list.

Simulation result

The proposed methods are implemented on top of ECM-4.0 with the merge request 106 (MR106: Related issue #19).

The RPR is enabled in the test with the following configuration parameters of JVET-Y0128[1]:

| Configuration 1: (Each frame is scaled) | Configuration 2: (Resolution switch after every 16 frames) |
| --- | --- |
| ScalingRatioHor : 2.0 | ScalingRatioHor : 2.0 |
| ScalingRatioVer : 2.0 | ScalingRatioVer : 2.0 |
| FractionNumFrames : 1.0 | FractionNumFrames : 1.0 |
| RPR : 1 | RPR : 1 |
| UpscaledOutput : 2 | UpscaledOutput : 1 |
| SwitchPocPeriod : -1 | SwitchPocPeriod : 16 |

Simulation results (PSNR1) with configuration 1:

|  | Random Access |  |  |  |  | Low-delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 CTC |  |  |  |  | Over ECM-4.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -61.29% | -43.12% | -35.38% | 27% | 31% |  |  |  |  |  |
| Class A2 | -57.11% | -72.94% | -67.00% | 26% | 31% |  |  |  |  |  |
| Class B | -58.75% | -62.36% | -50.09% | 28% | 31% | -57.11% | -58.62% | -44.55% | 26% | 29% |
| Class C | -61.96% | -56.96% | -50.03% | 29% | 32% | -63.42% | -55.89% | -47.46% | 27% | 31% |
| Class E |  |  |  |  |  | -50.77% | -37.85% | -33.69% | 28% | 33% |
| Overall | -59.79% | -59.19% | -50.51% | 28% | 32% | -57.63% | -52.51% | -42.80% | 27% | 31% |
| Class D | -63.61% | -55.63% | -51.90% | 27% | 32% | -63.53% | -53.70% | -47.94% | 26% | 30% |
| Class F | -38.51% | -39.53% | -37.23% | 35% | 36% | -39.01% | -31.07% | -30.31% | 33% | 35% |

Simulation results (PSNR1) with configuration 2:

|  | Random Access |  |  |  |  | Low-delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 CTC |  |  |  |  | Over ECM-4.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -36.01% | -18.62% | -13.62% | 71% | 69% |  |  |  |  |  |
| Class A2 | -30.37% | -52.38% | -42.82% | 73% | 69% |  |  |  |  |  |
| Class B | -34.07% | -46.34% | -31.71% | 74% | 70% | -34.41% | -44.42% | -30.99% | 70% | 74% |
| Class C | -23.74% | -25.99% | -17.79% | 73% | 71% | -30.20% | -30.36% | -21.11% | 70% | 74% |
| Class E |  |  |  |  |  | -29.16% | -30.96% | -28.75% | 75% | 83% |
| Overall | -30.96% | -36.58% | -26.60% | 73% | 70% | -31.70% | -36.37% | -27.14% | 71% | 76% |
| Class D | -26.31% | -25.89% | -20.06% | 74% | 72% | -30.40% | -32.56% | -26.18% | 70% | 74% |
| Class F | -9.23% | -11.44% | -8.94% | 78% | 81% | -11.42% | -12.32% | -10.19% | 80% | 88% |

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).