| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0102-v2 |
| --- | --- |

| Title: | Correction of the ARMC re-ordering step regarding the zero candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Patrice Onno Romain Bellessort | Tel: Email: | 33 299876800 guillaume.laroche@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution presents a correction of the Adaptive Merge Candidates list Reordering process (ARMC). It is reported that when excluding the zero candidates from the reordering process of the ARMC, this exclusion provides a coding efficiency benefit while saving several cost computations performed by template matching.

Compared to ECM-4.0, the average BDR gains and runtimes reported in this contribution are as follows:

-0.04%/-0.04%/-0.04%  -  98%,  97%for the RA configuration,

-0.11%/-0.14%/-0.16%  -  100% , 97%for the Low Delay B configuration.

# Problem Statement

In ECM-3.0, the merge candidates with an adaptive reordering based on a template matching (TM) technique has been adopted. The reordering method was applied to regular merge mode, template matching (TM) merge mode, and affine merge mode. The reordering was limited to a subgroup of candidates. The subgroup size is set to 5 for regular merge mode and TM merge mode and to 3 for affine merge mode. In ECM-4.0, AMRC was updated and the reordering is applied to all merge candidates.

The Merge candidates list contains sometimes several duplicate zero candidates. Consequently, the zero candidates can be set in the final merge list at an early position followed by several times the same candidate as depicted in the following Figure.

Figure 1 Example of reordering process of a merge candidates list.

This behavior should be avoided for coding efficiency.

# Proposed modification

In this contribution, we have simply modified the ECM-4.0 in order to exclude all the zero candidates from the ARMC reordering process during the construction of Merge motion vector candidates list as depicted in the following figure. This modification also reduces the number of ARMC computations.

Figure 2 Proposed Zero candidates exclusion of the ARMC process.

# Results

The proposed modification is implemented on top of the ECM-4.0 [1] and tested according to the common test condition (CTC) for ECM-4.0 [2].

Table 1 coding performances of the proposed modification

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0_ANCHOR_ |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.14% | 0.00% | 98% | 98% |
| Class A2 | -0.05% | -0.03% | -0.03% | 99% | 98% |
| Class B | -0.04% | 0.12% | -0.06% | 100% | 97% |
| Class C | -0.04% | -0.19% | -0.04% | 97% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.04% | -0.04% | 98% | 97% |
| Class D | -0.05% | -0.01% | -0.01% | 95% | 96% |
| Class F | -0.03% | -0.16% | -0.16% | 98% | 97% |
| Class TGM |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0_ANCHOR_ |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.07% | -0.15% | 100% | 99% |
| Class C | -0.04% | -0.09% | -0.24% | 100% | 96% |
| Class E | -0.34% | -0.33% | -0.07% | 100% | 96% |
| Overall | -0.11% | -0.14% | -0.16% | 100% | 97% |
| Class D | -0.04% | 0.80% | 0.38% | 103% | 97% |
| Class F | -0.02% | 0.36% | -0.42% | 98% | 97% |
| Class TGM |  |  |  |  |  |

# Conclusion

In this contribution, a modification regarding ARMC is proposed to exclude any zero motion vector candidates from the reordering process.

The results above show a coding performance for the LB configuration (more than -0.1% gain) and a smaller gain for the RA configuration. The gain is systematic whatever the classes. As the change is minimal, provides coding efficiency gain and reduces the number of cost computation performed by template matching at decoder side, we suggest adopting this modification in the ECM-5.0.

# References

- ECM-4.0 reference software: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-4.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).