| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0117 |
| --- | --- |

| Title: | EE2-2.6: Long-tap interpolation filtering on chroma components |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xi Xie1, Kai Zhang2, Li Zhang2, Junru Li1, Meng Wang1, Shiqi Wang1 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | xixie.jensen@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the test results of EE2-2.6 wherein long-tap interpolation filters are applied on chroma components in motion compensation. On top of ECM-4.0, experimental results are reported as below:

Test #2.6c (6-tap filtering):

RA: {}, LB: {}.

# Introduction

In ECM-4.0, a 4-tap interpolation filter is applied on chroma components while a 12-tap interpolation filter is applied on the luma component in the motion compensation process. It was proposed to apply long-tap interpolation filters on chroma components in JVET-Y0172 [1] and the method was decided to be studied in EE2-2.6.

# Proposed methods

This contribution reports the test results of EE2-2.6 wherein long-tap interpolation filters are applied on chroma components in motion compensation. The 6-tap filter coefficients in EE2-2.6c are shown in Table 1. A flag is signaled in SPS to indicate whether the 6-tap interpolation filter or the 4-tap chroma interpolation filter is used. Since the 6-tap filter is more efficient on low-resolution sequences, the original 4-tap chroma interpolation filter is applied for sequences in class A1/A2 while the proposed 6-tap interpolation filter is applied for sequences in other classes, configured by the encoder in the EE tests.

# Experimental results

The proposed method is tested on top of ECM-4.0 [2], according to the common test conditions [3]. The simulation results are summarized in Table 2, respectively.

It is verified by an internal viewing test that the proposed method shows no obvious impact on the subjective quality.

# Conclusions

This contribution proposes a 6-tap interpolation filter on chroma components. It is recommended to adopt Test #2.6c (6-tap filter).

Table 1. The coefficients of the 6-tap interpolation filter for chroma components

| Fractional position | Coefficients (6 taps) |
| --- | --- |
| 1/32 | {0, 0, 256, 0, 0, 0}, |
| 2/32 | {1, -6, 256, 7, -2, 0}, |
| 3/32 | {2, -11, 253, 15, -4, 1}, |
| 4/32 | {3, -16, 251, 23, -6, 1}, |
| 5/32 | {4, -21, 248, 33, -10, 2}, |
| 6/32 | {5, -25, 244, 42, -12, 2}, |
| 7/32 | {7, -30, 239, 53, -17, 4}, |
| 8/32 | {7, -32, 234, 62, -19, 4}, |
| 6/32 | {8, -35, 227, 73, -22, 5}, |
| 7/32 | {9, -38, 220, 84, -26, 7}, |
| 8/32 | {10, -40, 213, 95, -29, 7}, |
| 9/32 | {10, -41, 204, 106, -31, 8}, |
| 10/32 | {10, -42, 196, 117, -34, 9}, |
| 11/32 | {10, -41, 187, 127, -35, 8}, |
| 12/32 | {11, -42, 177, 138, -38, 10}, |
| 13/32 | {10, -41, 168, 148, -39, 10}, |
| 14/32 | {10, -40, 158, 158, -40, 10}, |
| 15/32 | {10, -39, 148, 168, -41, 10}, |
| 16/32 | {10, -38, 138, 177, -42, 11}, |
| 17/32 | {8, -35, 127, 187, -41, 10}, |
| 18/32 | {9, -34, 117, 196, -42, 10}, |
| 19/32 | {8, -31, 106, 204, -41, 10}, |
| 20/32 | {7, -29, 95, 213, -40, 10}, |
| 21/32 | {7, -26, 84, 220, -38, 9}, |
| 22/32 | {5, -22, 73, 227, -35, 8}, |
| 23/32 | {4, -19, 62, 234, -32, 7}, |
| 24/32 | {4, -17, 53, 239, -30, 7}, |
| 25/32 | {2, -12, 42, 244, -25, 5}, |
| 26/32 | {2, -10, 33, 248, -21, 4}, |
| 27/32 | {1, -6, 23, 251, -16, 3}, |
| 28/32 | {1, -4, 15, 253, -11, 2}, |
| 31/32 | {0, -2, 7, 256, -6, 1}, |

Table 2. Performance of 6-tap chroma interpolation filter (Test #2.6c)

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.04% | -2.38% | -2.05% | 101% |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.04% | -3.51% | -3.77% | 100% |  | -0.13% | -2.62% | -4.14% | -0.13% |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

# References

- X. Xie, K. Zhang, L. Zhang, Junru Li, Meng Wang, Shiqi Wang, “Non-EE2: Long tap interpolation filtering on chroma components,” JVET-Y0172, Jan. 2022.

- ECM-4.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-4.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).