| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0128-v2 |
| --- | --- |

| Title: | EE1-related: on BaseQP parameter in EE1-1.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhihuang Xie Yue Yu Haoping Yu Dong Wang | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution proposes a QP-adjustment method based on EE1-1.1 to improve the coding performance of EE1-1.1. In EE1-1.1, the QP parameters are part of the input to the network at the inference stage and these QP parameters include BaseQP and SliceQP. It is proposed to add an offset to the BaseQP at the frame-level and use the final adjusted BaseQP as the input of the network. Compared with the EE1-1.1, the proposed method can further improve the coding efficiency shown as below:

RA: Y -0.30%, U -0.11%, V -0.13%;

LDB: Y -0.48%, U -0.76%, V -0.62%.

Introduction

In JVET-Y0078[1], a neural network based in-loop filter with constrained storage and low complexity was proposed, which was included in EE1 in last meeting and named as EE1-1.1 for further study. Two models were used in EE1-1.1, including one for I-Slice frame and the another for B-Slice frame.

The network structure of EE1-1.1 is shown in Fig.1. Along with the reconstructed image, the prediction image and the partition image are also taken into the network as input signals. In addition, the I-Slice model also takes SliceQP as an input, while the B-Slice model takes both SliceQP and BaseQP as inputs.

Fig.1 Network structure of JVET-Y0078

The NN-based filter can be turned on/off at the CTU level and slice level.

Proposed method

To reduce the storage burden and avoid switching models at CTU level on the decoder side, many solutions have been proposed to reduce the total number of NN-based loop-filter models. It has been demonstrated in JVET-Y0078 that additionally adding BaseQP and SliceQP as network inputs can significantly improve the capability of NN models, and, as a result, just one B-Slice model can be trained and utilized for all the QPs across the entire common test conditions. However, this single model solution does lead to a relative lower performance compared with the previous model-switch solutions.

This contribution proposes that, instead of using the BaseQP directly, a slice-level adjustment can be made to the BaseQP and the adjusted BaseQP is used as the input to the B-Slice model. In this contribution, the adjustment is done by simply adding a small offset to the BaseQP, and the offset candidates used in this contribution are -5 or 5.

For example, supposing the offset {5} is chosen to add to the BaseQP, the input to the model will be (BaseQP + 5) for each and every CTU in the current slice.

A slice-level flag is signalled to indicate whether BaseQP offset is enable. If this flag is on, another slice-level flag will be signalled to indicate what value of the offset is used, and every CTU in the current slice will take the same offset for BaseQP.

Simulation results

The proposed method is implemented on top of EE1-1.1.1. The simulations were performed following the NNVC common test condition (CTC)[2]. Since BaseQP is only needed for B-Slice mode, only Random Access and Low Delay B were tested.

Over EE1-1.1.1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over EE1-1.1.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | 0.36% | -0.43% | 146% | 101% |
| Class A2 | -0.39% | -0.09% | 0.18% | 146% | 99% |
| Class B | -0.27% | -0.27% | -0.18% | 141% | 101% |
| Class C | -0.34% | -0.29% | -0.08% | 132% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.30% | -0.11% | -0.13% | 140% | 100% |
| Class D | -0.18% | -0.43% | 0.04% | 136% | 101% |
| Class F | -0.06% | -0.09% | 0.19% | 155% | 93% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over EE1-1.1.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.43% | -0.30% | -0.53% | 143% | 105% |
| Class C | -0.51% | -1.33% | -0.72% | 136% | 106% |
| Class E | -0.55% | 1.13% | 0.08% | 174% | 123% |
| Overall | -0.48% | -0.76% | -0.62% | 148% | 109% |
| Class D | -0.34% | -0.80% | 0.26% | 138% | 104% |
| Class F | 0.07% | -0.24% | 0.03% | 156% | 104% |

Over NNVC-anchor

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over NNVC anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -9.47% | -15.34% | -19.10% | 177% | 39718% |
| Class A2 | -9.76% | -19.36% | -14.95% | 168% | 36751% |
| Class B | -8.66% | -20.91% | -20.20% | 172% | 40073% |
| Class C | -8.63% | -22.77% | -22.28% | 153% | 36361% |
| Class E |  |  |  |  |  |
| Overall | -9.03% | -19.99% | -19.49% | 167% | 38309% |
| Class D | -9.42% | -21.51% | -22.45% | 158% | 34302% |
| Class F | -3.69% | -12.06% | -10.96% | 212% | 16381% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over NNVC anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -7.35% | -15.38% | -15.76% | 151% | 27890% |
| Class C | -8.00% | -19.67% | -17.95% | 135% | 26008% |
| Class E | -8.73% | -13.94% | -14.67% | 244% | 26342% |
| Overall | -7.91% | -17.29% | -16.73% | 164% | 26862% |
| Class D | -8.97% | -16.31% | -18.25% | 140% | 27772% |
| Class F | -3.78% | -11.61% | -7.77% | 193% | 10865% |

Conclusion

This contribution proposes a QP adjustment method for EE1-1.1 to further improve the coding performance. This method is implemented on top of EE1-1.1 and provides Y, U, V BD-rate coding efficiency as follows:

RA: Y -0.30%, U -0.11%, V -0.13%;

LDB: Y -0.48%, U -0.76%, V -0.62%.

This method can provide good performance on top of EE1-1.1 without increasing decoder-side complexity. It is suggested to further study this method in EE1.

Reference

[1] Liqiang Wang, etc. “EE1-1.1: neural network based in-loop filter with constrained storage and low complexity”, JVET-Y0078, Jan. 2022.

[2] Shan Liu, etc, “JVET common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-Y2016, Jan. 2022.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).