| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0133-v1 |
| --- | --- |

| Title: | EE2-4.2: CABAC initialization from previous inter slice and windows adjustment |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Hossein Golestani Nan Hu Patrick Garus Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports EE2-4.2 test results for CABAC initialization for inter slices by using probabilities stored from the slice containing last CTU of the previously coded picture having the same slice type, slice QP, and temporal ID, and CABAC windows adjustment.

Experimental results are summarized as follows:

Context initialization retraining

AI: -0.05% (Y), -0.02% (U), 0.02% (V), 101% (EncT), 103% (DecT)

RA: -0.11% (Y), -0.13% (U), 0.00% (V), 97% (EncT), 98% (DecT)

LB: -0.21% (Y), 0.19% (U), -0.30% (V), 96% (EncT), 96% (DecT)

Temporal initialization stored at middle CTU

RA: -0.34% (Y), -0.09% (U), 0.03% (V), 99% (EncT), 99% (DecT)

LB: -0.50% (Y), 0.77% (U), 0.53% (V), 98% (EncT), 100% (DecT)

EE2-4.2a Temporal initialization stored at last CTU

RA: -0.34% (Y), -0.11% (U), 0.00% (V), 101% (EncT), 101% (DecT)

LB: -0.60% (Y), 0.62% (U), 0.60% (V), 100% (EncT), 99% (DecT)

EE2-4.2b CABAC windows adjustment

AI: -0.09% (Y), 0.08% (U), 0.06% (V), 106% (EncT), 97% (DecT)

RA: -0.19% (Y), 0.00% (U), 0.17% (V), 102% (EncT), 98% (DecT)

LB: -0.31% (Y), 0.45% (U), 0.16% (V), 101% (EncT), 99% (DecT)

EE2-4.2c Temporal initialization with windows adjustment (4.2a+4.2b)

AI: -0.09% (Y), 0.08% (U), 0.06% (V), 106% (EncT), 100% (DecT)

RA: -0.38% (Y), 0.08% (U), 0.29% (V), 101% (EncT), 99% (DecT)

LB: -0.66% (Y), 0.93% (U), 1.01% (V), 103% (EncT), 99% (DecT)

# Proposal

## Temporal CABAC

Utilizing previous coded inter slices for CABAC initialization was proposed in JVET-Y0181 and earlier in JVET-K0379.

If the current slice type is a B or P, the probability state of each context model is first obtained after coding CTUs up to a specified location and stored. Then, the stored probability state will be used as the initial probability state for the corresponding context model in the next picture having B- or P-slice coded with the same quantization parameter (QP) and temporal ID. The buffer size for storing previous initializations is set equal to 5 for each slice type, when the buffer is full the entry with the smallest QP and temporal ID is removed first before storing the initialization.

In the tests, the CTU location for storing probability states was set at the end of the picture and at the middle location computed using the following formula:

CTU location = min ((W + C)/2 + 1, C).

where W denote the number of CTUs in a CTU line, and C is the total number of CTUs in a picture.

## Adaptive window size adjustment

The CABAC used in ECM employs two probability states that are updated with a short and a long window size, respectively. The window sizes, predefined for each context model, are not optimal for varying statistics in different regions. In this method, it is proposed to adjust the window sizes according to the previously coded bin of each context.

The previous coded bin is used as an index to get the adjustment parameters from a look-up table: delta0 for the short window and delta1 for the long window. Denote the original short and long window sizes stored in the existed initialization tables and defined for the context model as shift0 and shift1, respectively. The actual window sizes used to code the current bin after adjustment is (shift0+delta0) and (shift1+delta1). The window adjustment values take 4 bits each, so both values are stored in 1 byte and the total size of the LUT for 508 contexts is 1,016 bytes.

# Results

The proposed methods were implemented on top of ECM-4.0 and tested under ECM CTC.

The first test was performed where CABAC context initialization is retrained for the ECM anchor, the results are summarized in Table 1. All other tests use this retrained context initialization.

Table 1. CABAC contexts retraining for ECM-4.0 anchor

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.00% | 0.05% | 102% | 105% |
| Class A2 | -0.02% | -0.06% | -0.04% | 100% | 102% |
| Class B | -0.05% | -0.01% | 0.06% | 100% | 100% |
| Class C | -0.06% | -0.01% | 0.04% | 102% | 103% |
| Class E | -0.05% | -0.02% | -0.05% | 104% | 108% |
| Overall | -0.05% | -0.02% | 0.02% | 101% | 103% |
| Class D | -0.05% | -0.06% | 0.00% | 104% | 108% |
| Class F | -0.09% | 0.17% | 0.01% | 106% | 105% |
| Class TGM | -0.09% | -0.11% | -0.03% | 104% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.10% | 0.07% | 100% | 99% |
| Class A2 | -0.09% | -0.15% | -0.03% | 98% | 99% |
| Class B | -0.16% | -0.05% | 0.03% | 96% | 97% |
| Class C | -0.11% | -0.23% | -0.07% | 97% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.13% | 0.00% | 97% | 98% |
| Class D | -0.10% | -0.04% | -0.26% | 95% | 95% |
| Class F | -0.10% | -0.30% | -0.25% | 99% | 99% |
| Class TGM | -0.10% | -0.21% | -0.14% | 98% | 99% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.13% | 0.42% | 0.22% | 94% | 94% |
| Class C | -0.13% | 0.09% | -0.35% | 95% | 96% |
| Class E | -0.46% | -0.07% | -1.11% | 102% | 100% |
| Overall | -0.21% | 0.19% | -0.30% | 96% | 96% |
| Class D | -0.31% | -0.06% | 0.65% | 93% | 95% |
| Class F | -0.34% | 1.38% | -0.27% | 101% | 100% |
| Class TGM | -0.06% | 0.00% | -0.01% | 99% | 100% |

Table 2. Temporal CABAC initialization stored at middle CTU

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.30% | -0.11% | -0.02% | 100% | 102% |
| Class A2 | -0.19% | -0.17% | -0.11% | 100% | 101% |
| Class B | -0.41% | 0.14% | 0.21% | 97% | 97% |
| Class C | -0.38% | -0.31% | -0.05% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.34% | -0.09% | 0.03% | 99% | 99% |
| Class D | -0.51% | -0.06% | -0.30% | 99% | 99% |
| Class F | -0.35% | -0.58% | -0.41% | 100% | 100% |
| Class TGM | -0.70% | -0.80% | -0.76% | 100% | 101% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.41% | 1.17% | 1.53% | 98% | 100% |
| Class C | -0.43% | 0.06% | 0.17% | 98% | 99% |
| Class E | -0.72% | 1.05% | -0.69% | 99% | 101% |
| Overall | -0.50% | 0.77% | 0.53% | 98% | 100% |
| Class D | -1.11% | 1.11% | 2.48% | 97% | 99% |
| Class F | -0.60% | 1.23% | -0.86% | 99% | 98% |
| Class TGM | -0.98% | -1.01% | -1.06% | 97% | 98% |

Table 3. Temporal CABAC initialization stored at last CTU (EE3-4.2a)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | -0.08% | -0.03% | 99% | 99% |
| Class A2 | -0.17% | -0.15% | -0.05% | 98% | 99% |
| Class B | -0.43% | 0.07% | 0.08% | 101% | 101% |
| Class C | -0.37% | -0.32% | -0.05% | 104% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.34% | -0.11% | 0.00% | 101% | 101% |
| Class D | -0.55% | -0.06% | -0.26% | 99% | 99% |
| Class F | -0.37% | -0.67% | -0.49% | 100% | 100% |
| Class TGM | -0.79% | -0.89% | -0.88% | 98% | 98% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.50% | 1.14% | 1.51% | 100% | 102% |
| Class C | -0.48% | 0.03% | 0.52% | 100% | 96% |
| Class E | -0.92% | 0.56% | -0.82% | 97% | 98% |
| Overall | -0.60% | 0.62% | 0.60% | 100% | 99% |
| Class D | -1.11% | 0.78% | 1.75% | 96% | 95% |
| Class F | -0.86% | 0.82% | -1.04% | 94% | 94% |
| Class TGM | -1.22% | -1.19% | -1.24% | 97% | 94% |

Table 4. CABAC windows adjustment (EE3-4.2b)

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | 0.05% | 0.05% | 106% | 101% |
| Class A2 | -0.05% | 0.07% | 0.03% | 106% | 98% |
| Class B | -0.08% | 0.09% | 0.07% | 107% | 100% |
| Class C | -0.09% | 0.07% | 0.04% | 106% | 93% |
| Class E | -0.12% | 0.08% | 0.11% | 104% | 94% |
| Overall | -0.09% | 0.08% | 0.06% | 106% | 97% |
| Class D | -0.07% | 0.04% | -0.06% | 105% | 94% |
| Class F | -0.19% | 0.14% | 0.01% | 100% | 91% |
| Class TGM | -0.13% | -0.09% | -0.05% | 101% | 92% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | 0.01% | 0.24% | 101% | 100% |
| Class A2 | -0.18% | 0.12% | 0.07% | 102% | 100% |
| Class B | -0.24% | 0.07% | 0.30% | 101% | 98% |
| Class C | -0.16% | -0.17% | 0.03% | 103% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.19% | 0.00% | 0.17% | 102% | 98% |
| Class D | -0.17% | -0.15% | -0.34% | 104% | 100% |
| Class F | -0.17% | -0.35% | -0.28% | 104% | 100% |
| Class TGM | -0.15% | -0.19% | -0.13% | 103% | 99% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.20% | 0.19% | 0.28% | 103% | 100% |
| Class C | -0.32% | 0.54% | -0.11% | 98% | 95% |
| Class E | -0.48% | 0.77% | 0.32% | 103% | 102% |
| Overall | -0.31% | 0.45% | 0.16% | 101% | 99% |
| Class D | -0.28% | 0.15% | 0.84% | 99% | 95% |
| Class F | -0.14% | 1.15% | -0.11% | 100% | 96% |
| Class TGM | -0.12% | 0.11% | 0.13% | 96% | 92% |

Table 5. Temporal CABAC initialization (4.2a) with windows adjustment (4.2b) (EE3-4.2c)

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | 0.05% | 0.05% | 105% | 98% |
| Class A2 | -0.05% | 0.07% | 0.03% | 106% | 101% |
| Class B | -0.08% | 0.09% | 0.07% | 110% | 103% |
| Class C | -0.09% | 0.07% | 0.04% | 104% | 101% |
| Class E | -0.12% | 0.08% | 0.11% | 106% | 94% |
| Overall | -0.09% | 0.08% | 0.06% | 106% | 100% |
| Class D | -0.07% | 0.04% | -0.06% | 103% | 101% |
| Class F | -0.19% | 0.14% | 0.01% | 105% | 102% |
| Class TGM | -0.13% | -0.09% | -0.05% | 105% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | 0.01% | 0.24% | 101% | 98% |
| Class A2 | -0.23% | 0.21% | 0.03% | 100% | 99% |
| Class B | -0.50% | 0.30% | 0.50% | 102% | 99% |
| Class C | -0.38% | -0.23% | 0.26% | 101% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.38% | 0.08% | 0.29% | 101% | 99% |
| Class D | -0.60% | -0.25% | -0.36% | 100% | 97% |
| Class F | -0.41% | -0.64% | -0.58% | 101% | 98% |
| Class TGM | -0.80% | -0.82% | -0.80% | 102% | 97% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.54% | 1.30% | 1.52% | 102% | 98% |
| Class C | -0.64% | -0.15% | 1.00% | 103% | 98% |
| Class E | -0.89% | 1.75% | 0.18% | 104% | 100% |
| Overall | -0.66% | 0.93% | 1.01% | 103% | 99% |
| Class D | -1.21% | 1.05% | 2.07% | 104% | 101% |
| Class F | -0.85% | -0.02% | -0.96% | 102% | 98% |
| Class TGM | -1.05% | -0.95% | -0.94% | 102% | 98% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).