| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0042-v1 |
| --- | --- |

| Title: | EE2-2.8, 2.9, 2.10: Longer luma and chroma filters for RPR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenneth Andersson Ruoyang Yu | Email: | kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution presents results for EE2-2.8, EE2-2.9 and EE2-2.10. In EE2-2.8, longer (6-tap) chroma filters for RPR are tested. In EE2-2.9, longer luma filters for RPR are tested. The EE2-2.9 contains two sub-tests, EE2-2.9a where the luma RPR filters for all inter blocks are 12-tap, and EE2-2.9b where the luma filters RPR for non-affine inter blocks are 12-tap while the luma RPR filters for affine blocks are 10-tap. EE-2.10 contains two sub-tests where combination of longer luma and chroma RPR filters are tested. EE2-2.10a is a combined test of EE2-2.8 and EE2-2.9a. EE2-2.10b is a combined test of EE2-2.8 and EE2-2.9b.

All the tests are reported to be tested under LDB RPR common test configuration. The results on top of ECM-5.0 are reported in the table below. Both BD-rate PSNR1 and BD-rate PSNR2 numbers are provided. The decoding run time numbers are asserted to be accurate. Based on the results, it is proposed to adopt EE2-2.10a or EE2-2.10b in the next version of ECM.

|  | scaling ratio 1.5x |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate PSNR1 |  |  | BD-rate PSNR2 |  |  |  |
| EE2 TEST # | Y | U | V | Y | U | V | DecT |
| 2.8 | -0,02% | -0,52% | -0,56% | -0,04% | -0,66% | -0,70% | 100% |
| 2.9a | -0,72% | -0,26% | -0,38% | -0,45% | -0,33% | -0,46% | 99% |
| 2.9b | -0,70% | -0,34% | -0,30% | -0,43% | -0,38% | -0,39% | 99% |
| 2.10a (2.8 + 2.9a) | -0,73% | -0,77% | -0,62% | -0,44% | -0,96% | -0,81% | 100% |
| 2.10b (2.8 + 2.9b) | -0,76% | -0,98% | -0,80% | -0,47% | -1,23% | -0,97% | 100% |
|  | scaling ratio 2.0x |  |  |  |  |  |  |
|  | BD-rate PSNR1 |  |  | BD-rate PSNR2 |  |  |  |
| EE2 TEST # | Y | U | V | Y | U | V | DecT |
| 2.8 | 0,01% | -0,75% | -1,14% | 0,02% | -1,18% | -1,28% | 100% |
| 2.9a | -0,45% | 0,12% | -0,12% | -0,18% | 0,01% | -0,17% | 99% |
| 2.9b | -0,40% | 0,23% | 0,15% | -0,12% | 0,10% | 0,11% | 99% |
| 2.10a (2.8 + 2.9a) | -0,47% | -0,55% | -0,91% | -0,19% | -1,00% | -1,09% | 100% |
| 2.10b (2.8 + 2.9b) | -0,43% | -0,78% | -1,02% | -0,13% | -1,21% | -1,17% | 100% |

# Introduction

In the current ECM, the luma interpolation filters used in motion compensation under non-RPR scenario have been increased to 12-tap from 8-tap. The luma filters used for non-affine inter blocks under RPR up-sampling scenario are also increased to 12-tap. However, the luma filters used for affine inter blocks under RPR up-sampling scenario remain to be 8-tap. Furthermore, the luma filters used for all inter blocks under RPR down-sampling scenario are still 8-tap.

When it comes to chroma interpolation filters, the chroma filters used in motion compensation under non-RPR scenario have been increased to 6-tap from 4-tap, consequently, the chroma filters used under RPR up-sampling scenario are also increased to 6-tap. However, the chroma filters used under RPR down-sampling scenario are still 4-tap.

Based on the observations above and with aims of improving design consistency, it was proposed to increase the luma RPR filters (i.e., the luma RPR down-sampling filters for all inter blocks and the luma RPR up-sampling filters for affine blocks) to 12-tap in JVET-Z0062 [1] and it was proposed to increase the chroma RPR filters (i.e., the chroma RPR down-sampling filter) to 6-tap in JVET-Z0069 [2].

During the meeting discussion, the increasing of luma RPR filters to 12-tap and the increasing of chroma RPR filters to 6-tap were agreed to be included in the EE tests. Furthermore, to follow the same spirit of luma RPR filter length design in VVC, an alternative design is also included in the EE tests. In this alternative design, the luma RPR filters for non-affine inter blocks are increased to 12-tap while for affine inter blocks 10-tap luma RPR filters are used.

The EE tests [3] are summarized in the table below.

| Test # | Description |
| --- | --- |
| 2.8 | RPR chroma filters 6-tap |
| 2.9a | RPR luma filters 12-tap |
| 2.9b | RPR luma filters 12-tap for non-affine blocks and RPR luma filters 10-tap for affine blocks |
| 2.10a | Test 2.8 + Test 2.9a |
| 2.10b | Test 2.8 + Test 2.9b |

The proposed RPR filters in the EE tests are listed below.

## Proposed 6-tap chroma RPR filters

| Fractional sample position | Proposed coefficients for 1.5x down-sampling | Proposed coefficients for 2.0x down-sampling |
| --- | --- | --- |
| 0/32 | { -14, 64, 156, 64, -14, 0 }, | { -5, 64, 138, 64, -5, 0}, |
| 1/32 | { -13, 59, 155, 68, -12, -1 }, | { -8, 66, 132, 73, -6, -1}, |
| 2/32 | { -13, 56, 154, 71, -11, -1 }, | { -9, 62, 134, 76, -5, -2}, |
| 3/32 | { -13, 52, 153, 76, -10, -2 }, | { -10, 59, 134, 80, -4, -3}, |
| 4/32 | { -12, 47, 153, 79, -9, -2 }, | { -10, 57, 132, 82, -2, -3}, |
| 5/32 | { -13, 45, 151, 84, -8, -3 }, | { -10, 53, 132, 85, 0, -4}, |
| 6/32 | { -13, 39, 154, 88, -9, -3 }, | { -11, 50, 132, 89, 1, -5}, |
| 7/32 | { -14, 34, 156, 94, -10, -4 }, | { -12, 47, 132, 93, 2, -6}, |
| 8/32 | { -15, 33, 152, 99, -7, -6 }, | { -13, 45, 131, 96, 4, -7}, |
| 9/32 | { -15, 30, 150, 103, -5, -7 }, | { -12, 40, 131, 99, 5, -7}, |
| 10/32 | { -14, 26, 148, 107, -4, -7 }, | { -12, 39, 127, 101, 9, -8}, |
| 11/32 | { -12, 23, 143, 110, -1, -7 }, | { -13, 36, 127, 105, 10, -9}, |
| 12/32 | { -13, 18, 145, 116, -2, -8 }, | { -12, 33, 124, 107, 14, -10}, |
| 13/32 | { -10, 15, 139, 118, 1, -7 }, | { -13, 30, 124, 111, 15, -11}, |
| 14/32 | { -15, 19, 134, 122, 9, -13 }, | { -13, 29, 121, 112, 18, -11}, |
| 15/32 | { -14, 18, 129, 123, 13, -13 }, | { -12, 25, 119, 115, 21, -12}, |
| 16/32 | { -14, 18, 124, 124, 18, -14 }, | { -13, 24, 117, 117, 24, -13}, |
| 17/32 | { -13, 13, 123, 129, 18, -14 }, | { -12, 21, 115, 119, 25, -12}, |
| 18/32 | { -13, 9, 122, 134, 19, -15 }, | { -11, 18, 112, 121, 29, -13}, |
| 19/32 | { -7, 1, 118, 139, 15, -10 }, | { -11, 15, 111, 124, 30, -13}, |
| 20/32 | { -8, -2, 116, 145, 18, -13 }, | { -10, 14, 107, 124, 33, -12}, |
| 21/32 | { -7, -1, 110, 143, 23, -12 }, | { -9, 10, 105, 127, 36, -13}, |
| 22/32 | { -7, -4, 107, 148, 26, -14 }, | { -8, 9, 101, 127, 39, -12}, |
| 23/32 | { -7, -5, 103, 150, 30, -15 }, | { -7, 5, 99, 131, 40, -12}, |
| 24/32 | { -6, -7, 99, 152, 33, -15 }, | { -7, 4, 96, 131, 45, -13}, |
| 25/32 | { -4, -10, 94, 156, 34, -14 }, | { -6, 2, 93, 132, 47, -12}, |
| 26/32 | { -3, -9, 88, 154, 39, -13 }, | { -5, 1, 89, 132, 50, -11}, |
| 27/32 | { -3, -8, 84, 151, 45, -13 }, | { -4, 0, 85, 132, 53, -10}, |
| 28/32 | { -2, -9, 79, 153, 47, -12 }, | { -3, -2, 82, 132, 57, -10}, |
| 29/32 | { -2, -10, 76, 153, 52, -13 }, | { -3, -4, 80, 134, 59, -10}, |
| 30/32 | { -1, -11, 71, 154, 56, -13 }, | { -2, -5, 76, 134, 62, -9}, |
| 31/32 | { -1, -12, 68, 155, 59, -13 }, | { -1, -6, 73, 132, 66, -8}, |

## Proposed 12-tap luma RPR filters

| Fractional sample position | Proposed coefficients for 1.5x down-sampling | Proposed coefficients for 2.0x down-sampling |
| --- | --- | --- |
| 0/16 | { 0, 10, -12, -19, 70, 158, 70, -19, -12, 10, 0, 0 } | { 4, -6, -18, 12, 78, 116, 78, 12, -18, -6, 4, 0 } |
| 1/16 | { 0, 10, -9, -22, 62, 156, 79, -17, -13, 9, 1, 0 } | { 4, -6, -18, 8, 76, 116, 86, 14, -18, -8, 2, 0 } |
| 2/16 | { -1, 10, -8, -22, 54, 155, 87, -13, -16, 9, 2, -1 } | { 4, -4, -18, 4, 70, 116, 88, 18, -16, -8, 2, 0 } |
| 3/16 | { -2, 10, -5, -23, 46, 152, 95, -9, -18, 9, 2, -1 } | { 2, -4, -18, 2, 68, 116, 92, 22, -16, -10, 2, 0 } |
| 4/16 | { -2, 9, -4, -23, 39, 150, 103, -6, -20, 8, 4, -2 } | { 2, -2, -16, -2, 62, 114, 94, 26, -14, -10, 2, 0 } |
| 5/16 | { -2, 8, -1, -24, 33, 144, 111, -2, -20, 6, 5, -2 } | { 2, -2, -16, -4, 58, 112, 98, 30, -14, -12, 2, 2 } |
| 6/16 | { -3, 9, -1, -24, 26, 142, 117, 4, -22, 5, 5, -2 } | { 2, 0, -16, -6, 52, 110, 102, 34, -14, -12, 2, 2 } |
| 7/16 | { -3, 8, 0, -23, 20, 137, 123, 10, -23, 4, 5, -2 } | { 2, 0, -14, -8, 48, 108, 104, 38, -12, -14, 2, 2 } |
| 8/16 | { -2, 6, 3, -25, 16, 130, 130, 16, -25, 3, 6, -2 } | { 2, 0, -14, -10, 44, 106, 106, 44, -10, -14, 0, 2 } |
| 9/16 | { -2, 5, 4, -23, 10, 123, 137, 20, -23, 0, 8, -3 } | { 2, 2, -14, -12, 38, 104, 108, 48, -8, -14, 0, 2 } |
| 10/16 | { -2, 5, 5, -22, 4, 117, 142, 26, -24, -1, 9, -3 } | { 2, 2, -12, -14, 34, 102, 110, 52, -6, -16, 0, 2 } |
| 11/16 | { -2, 5, 6, -20, -2, 111, 144, 33, -24, -1, 8, -2 } | { 2, 2, -12, -14, 30, 98, 112, 58, -4, -16, -2, 2 } |
| 12/16 | { -2, 4, 8, -20, -6, 103, 150, 39, -23, -4, 9, -2 } | { 0, 2, -10, -14, 26, 94, 114, 62, -2, -16, -2, 2 } |
| 13/16 | { -1, 2, 9, -18, -9, 95, 152, 46, -23, -5, 10, -2 } | { 0, 2, -10, -16, 22, 92, 116, 68, 2, -18, -4, 2 } |
| 14/16 | { -1, 2, 9, -16, -13, 87, 155, 54, -22, -8, 10, -1 } | { 0, 2, -8, -16, 18, 88, 116, 70, 4, -18, -4, 4 } |
| 15/16 | { 0, 1, 9, -13, -17, 79, 156, 62, -22, -9, 10, 0 } | { 0, 2, -8, -18, 14, 86, 116, 76, 8, -18, -6, 4 } |

It should be noted that the 12-tap luma RPR filter for affine blocks under RPR up-sampling scenario is proposed to be the same as the 12-tap luma RPR filter for non-affine blocks. The 12-tap luma RPR filters under up-sampling scenario are the same as the 12-tap ones used in the normal (non-RPR) scenario.

## Proposed 10-tap luma RPR filters (for affine blocks)

| Fractional sample position | Proposed coefficients for 1.5x down-sampling | Proposed coefficients for 2.0x down-sampling |
| --- | --- | --- |
| 0/16 | { 0, 6, -10, -19, 74, 154, 74, -19, -10, 6, 0, 0 } | { 0, -3, -14, 10, 77, 116, 77, 10, -14, -3, 0, 0 } |
| 1/16 | { 0, 8, -8, -22, 66, 153, 82, -18, -13, 8, 0, 0 } | { 0, -4, -15, 8, 73, 116, 82, 14, -15, -5, 2, 0 } |
| 2/16 | { 0, 7, -6, -24, 58, 152, 90, -14, -15, 8, 0, 0 } | { 0, -3, -15, 5, 69, 115, 86, 18, -15, -6, 2, 0 } |
| 3/16 | { 0, 7, -5, -25, 51, 150, 97, -11, -17, 8, 1, 0 } | { 0, -2, -15, 2, 65, 114, 89, 22, -14, -7, 2, 0 } |
| 4/16 | { 0, 6, -2, -26, 43, 147, 105, -7, -19, 7, 2, 0 } | { 0, -1, -14, -1, 60, 113, 93, 26, -14, -8, 2, 0 } |
| 5/16 | { 0, 6, -1, -26, 35, 144, 112, -2, -21, 7, 2, 0 } | { 0, -1, -14, -3, 56, 112, 96, 30, -13, -9, 2, 0 } |
| 6/16 | { 0, 5, 1, -26, 28, 140, 118, 3, -22, 6, 3, 0 } | { 0, 0, -13, -5, 51, 110, 100, 34, -12, -10, 1, 0 } |
| 7/16 | { 0, 5, 2, -26, 22, 135, 125, 9, -24, 5, 3, 0 } | { 0, 0, -13, -7, 47, 108, 103, 38, -10, -11, 1, 0 } |
| 8/16 | { 0, 4, 4, -25, 15, 130, 130, 15, -25, 4, 4, 0 } | { 0, 1, -12, -9, 43, 105, 105, 43, -9, -12, 1, 0 } |
| 9/16 | { 0, 3, 5, -24, 9, 125, 135, 22, -26, 2, 5, 0 } | { 0, 1, -11, -10, 38, 103, 108, 47, -7, -13, 0, 0 } |
| 10/16 | { 0, 3, 6, -22, 3, 118, 140, 28, -26, 1, 5, 0 } | { 0, 1, -10, -12, 34, 100, 110, 51, -5, -13, 0, 0 } |
| 11/16 | { 0, 2, 7, -21, -2, 112, 144, 35, -26, -1, 6, 0 } | { 0, 2, -9, -13, 30, 96, 112, 56, -3, -14, -1, 0 } |
| 12/16 | { 0, 2, 7, -19, -7, 105, 147, 43, -26, -2, 6, 0 } | { 0, 2, -8, -14, 26, 93, 113, 60, -1, -14, -1, 0 } |
| 13/16 | { 0, 1, 8, -17, -11, 97, 150, 51, -25, -5, 7, 0 } | { 0, 2, -7, -14, 22, 89, 114, 65, 2, -15, -2, 0 } |
| 14/16 | { 0, 0, 8, -15, -14, 90, 152, 58, -24, -6, 7, 0 } | { 0, 2, -6, -15, 18, 86, 115, 69, 5, -15, -3, 0 } |
| 15/16 | { 0, 0, 8, -13, -18, 82, 153, 66, -22, -8, 8, 0 } | { 0, 2, -5, -15, 14, 82, 116, 73, 8, -15, -4, 0 } |

The proposed 10-tap luma RPR filter under up-sampling scenario is in table below.

| Fractional sample position | Proposed coefficients for up-sampling |
| --- | --- |
| 0/16 | { 0, 0, 0, 0, 0, 256, 0, 0, 0, 0, 0, 0 } |
| 1/16 | { 0, 2, -3, 6, -14, 254, 16, -7, 4, -2, 0, 0 } |
| 2/16 | { 0, 3, -6, 12, -26, 249, 34, -14, 7, -4, 1, 0 } |
| 3/16 | { 0, 4, -9, 17, -36, 241, 54, -22, 11, -6, 2, 0 } |
| 4/16 | { 0, 5, -11, 20, -43, 230, 75, -29, 15, -8, 2, 0 } |
| 5/16 | { 0, 5, -12, 23, -48, 216, 96, -36, 18, -9, 3, 0 } |
| 6/16 | { 0, 5, -13, 25, -51, 200, 118, -42, 21, -11, 4, 0 } |
| 7/16 | { 0, 5, -13, 25, -51, 181, 140, -46, 23, -12, 4, 0 } |
| 8/16 | { 0, 5, -13, 25, -50, 161, 161, -50, 25, -13, 5, 0 } |
| 9/16 | { 0, 4, -12, 23, -46, 140, 181, -51, 25, -13, 5, 0 } |
| 10/16 | { 0, 4, -11, 21, -42, 118, 200, -51, 25, -13, 5, 0 } |
| 11/16 | { 0, 3, -9, 18, -36, 96, 216, -48, 23, -12, 5, 0 } |
| 12/16 | { 0, 2, -8, 15, -29, 75, 230, -43, 20, -11, 5, 0 } |
| 13/16 | { 0, 2, -6, 11, -22, 54, 241, -36, 17, -9, 4, 0 } |
| 14/16 | { 0, 1, -4, 7, -14, 34, 249, -26, 12, -6, 3, 0 } |
| 15/16 | { 0, 0, -2, 4, -7, 16, 254, -14, 6, -3, 2, 0 } |

# Experimental results

The tests are reported to be tested under LDB RPR common test configuration as specified in [4]. The results on top of ECM-5.0 are reported in the table below. Both BD-rate PSNR1 and BD-rate PSNR2 results are provided. The encoding run times are asserted to be inaccurate since the encoding of the anchor and the tests were run on machines with different capabilities. However, the decoding run times are asserted to be enough accurate since the decoding of the anchor and the test were run on same type of machines.

## Results for test 2.8 (chroma RPR filters 6-tap)

### Results for test 2.8 under 1.5x scaling ratio

|  | Low delay B Main10 (BD-rate PSNR1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,01% | -0,10% | -0,31% | 94% | 101% |
| Class C | -0,03% | -0,43% | -0,73% | 103% | 99% |
| Class E | -0,05% | -1,33% | -0,77% | 108% | 100% |
| Overall | -0,02% | -0,52% | -0,56% | 100% | 100% |
| Class D | -0,01% | -0,43% | -0,72% | 111% | 97% |
| Class F | -0,20% | -2,06% | -2,07% | 100% | 100% |

|  | Low delay B Main10 (BD-rate PSNR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,01% | -0,14% | -0,37% | 94% | 101% |
| Class C | -0,07% | -0,44% | -0,76% | 103% | 99% |
| Class E | -0,07% | -1,81% | -1,18% | 108% | 100% |
| Overall | -0,04% | -0,66% | -0,70% | 100% | 100% |
| Class D | -0,01% | -0,55% | -0,73% | 111% | 97% |
| Class F | -0,18% | -1,48% | -1,25% | 100% | 100% |

### Results for test 2.8 under 2.0x scaling ratio

|  | Low delay B Main10 (BD-rate PSNR1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,01% | -0,55% | -0,69% | 94% | 99% |
| Class C | 0,03% | -1,18% | -1,32% | 112% | 102% |
| Class E | 0,01% | -0,52% | -1,65% | 119% | 98% |
| Overall | 0,01% | -0,75% | -1,14% | 106% | 100% |
| Class D | -0,12% | 0,09% | -1,37% | 122% | 100% |
| Class F | -0,18% | -1,02% | -1,26% | 102% | 100% |

|  | Low delay B Main10 (BD-rate PSNR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,01% | -0,70% | -0,76% | 94% | 99% |
| Class C | 0,03% | -1,26% | -1,28% | 112% | 102% |
| Class E | 0,06% | -1,89% | -2,16% | 119% | 98% |
| Overall | 0,02% | -1,18% | -1,28% | 106% | 100% |
| Class D | -0,09% | -0,22% | -1,63% | 122% | 100% |
| Class F | -0,18% | -0,97% | -1,28% | 102% | 100% |

## Results for test 2.9a (luma RPR filters 12-tap)

### Results for test 2.9a under 1.5x scaling ratio

|  | Low delay B Main10 (BD-rate PSNR1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,28% | 0,36% | -0,30% | 92% | 98% |
| Class C | -0,69% | -0,46% | -0,36% | 103% | 98% |
| Class E | -1,50% | -1,05% | -0,52% | 105% | 101% |
| Overall | -0,72% | -0,26% | -0,38% | 99% | 99% |
| Class D | -0,73% | -0,72% | 0,29% | 105% | 100% |
| Class F | -5,07% | -1,37% | -1,20% | 93% | 100% |

|  | Low delay B Main10 (BD-rate PSNR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,26% | 0,35% | -0,34% | 92% | 98% |
| Class C | -0,38% | -0,59% | -0,52% | 103% | 98% |
| Class E | -0,85% | -1,13% | -0,57% | 105% | 101% |
| Overall | -0,45% | -0,33% | -0,46% | 99% | 99% |
| Class D | -0,44% | -0,72% | 0,19% | 105% | 100% |
| Class F | -1,76% | -1,60% | -1,46% | 93% | 100% |

### Results for test 2.9a under 2.0x scaling ratio

|  | Low delay B Main10 (BD-rate PSNR1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,16% | -0,06% | -0,17% | 94% | 100% |
| Class C | -0,50% | 0,20% | 0,06% | 109% | 98% |
| Class E | -0,86% | 0,30% | -0,27% | 110% | 101% |
| Overall | -0,45% | 0,12% | -0,12% | 103% | 99% |
| Class D | -0,77% | 0,49% | -0,68% | 106% | 98% |
| Class F | -3,33% | 1,06% | 0,51% | 95% | 102% |

|  | Low delay B Main10 (BD-rate PSNR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,10% | -0,12% | -0,20% | 94% | 100% |
| Class C | -0,25% | 0,18% | 0,01% | 109% | 98% |
| Class E | -0,20% | -0,01% | -0,36% | 110% | 101% |
| Overall | -0,18% | 0,01% | -0,17% | 103% | 99% |
| Class D | -0,34% | 0,30% | -0,97% | 106% | 98% |
| Class F | -0,51% | -0,41% | -0,82% | 95% | 102% |

## Results for test 2.9b (luma RPR filters 12-tap for non-affine blocks and luma RPR filters 10-tap for affine blocks)

### Results for test 2.9b under 1.5x scaling ratio

|  | Low delay B Main10 (BD-rate PSNR1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,28% | 0,03% | -0,14% | 94% | 99% |
| Class C | -0,64% | -0,15% | -0,04% | 99% | 98% |
| Class E | -1,49% | -1,21% | -0,93% | 107% | 100% |
| Overall | -0,70% | -0,34% | -0,30% | 99% | 99% |
| Class D | -0,90% | -0,34% | 0,22% | 101% | 97% |
| Class F | -5,07% | -1,07% | -1,20% | 93% | 102% |

|  | Low delay B Main10 (BD-rate PSNR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,25% | 0,04% | -0,21% | 94% | 99% |
| Class C | -0,33% | -0,24% | -0,21% | 99% | 98% |
| Class E | -0,85% | -1,27% | -0,93% | 107% | 100% |
| Overall | -0,43% | -0,38% | -0,39% | 99% | 99% |
| Class D | -0,56% | -0,43% | 0,09% | 101% | 97% |
| Class F | -1,65% | -1,46% | -1,51% | 93% | 102% |

### Results for test 2.9b under 2.0x scaling ratio

|  | Low delay B Main10 (BD-rate PSNR1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,13% | -0,09% | 0,04% | 93% | 99% |
| Class C | -0,39% | 0,23% | -0,12% | 104% | 101% |
| Class E | -0,88% | 0,75% | 0,70% | 105% | 99% |
| Overall | -0,40% | 0,23% | 0,15% | 100% | 100% |
| Class D | -0,66% | 0,18% | 0,04% | 101% | 102% |
| Class F | -3,40% | 0,90% | 0,42% | 94% | 102% |

|  | Low delay B Main10 (BD-rate PSNR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,06% | -0,19% | -0,03% | 93% | 99% |
| Class C | -0,15% | 0,24% | -0,06% | 104% | 101% |
| Class E | -0,17% | 0,41% | 0,56% | 105% | 99% |
| Overall | -0,12% | 0,10% | 0,11% | 100% | 100% |
| Class D | -0,28% | 0,02% | -0,22% | 101% | 102% |
| Class F | -0,57% | -0,49% | -0,95% | 94% | 102% |

## Results for 2.10a (Test 2.8 + Test 2.9a)

### Results for 2.10a under 1.5x scaling ratio

|  | Low delay B Main10 (BD-rate PSNR1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,32% | -0,14% | -0,47% | 86% | 99% |
| Class C | -0,72% | -1,07% | -0,54% | 102% | 100% |
| Class E | -1,43% | -1,43% | -0,96% | 108% | 99% |
| Overall | -0,73% | -0,77% | -0,62% | 96% | 100% |
| Class D | -0,78% | -1,01% | -0,84% | 115% | 101% |
| Class F | -5,11% | -3,44% | -3,96% | 92% | 101% |

|  | Low delay B Main10 (BD-rate PSNR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,27% | -0,19% | -0,56% | 86% | 99% |
| Class C | -0,40% | -1,17% | -0,71% | 102% | 100% |
| Class E | -0,77% | -1,98% | -1,36% | 108% | 99% |
| Overall | -0,44% | -0,96% | -0,81% | 96% | 100% |
| Class D | -0,49% | -1,21% | -1,01% | 115% | 101% |
| Class F | -1,81% | -2,84% | -3,12% | 92% | 101% |

### Results for 2.10a under 2.0x scaling ratio

|  | Low delay B Main10 (BD-rate PSNR1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,18% | -0,42% | -0,43% | 87% | 100% |
| Class C | -0,53% | -0,93% | -1,36% | 106% | 100% |
| Class E | -0,88% | -0,24% | -1,12% | 118% | 100% |
| Overall | -0,47% | -0,55% | -0,91% | 100% | 100% |
| Class D | -0,82% | -0,58% | -1,14% | 118% | 101% |
| Class F | -3,40% | -0,21% | -0,70% | 96% | 101% |

|  | Low delay B Main10 (BD-rate PSNR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,10% | -0,61% | -0,59% | 87% | 100% |
| Class C | -0,25% | -1,08% | -1,30% | 106% | 100% |
| Class E | -0,27% | -1,53% | -1,63% | 118% | 100% |
| Overall | -0,19% | -1,00% | -1,09% | 100% | 100% |
| Class D | -0,39% | -1,16% | -1,76% | 118% | 101% |
| Class F | -0,48% | -1,68% | -2,38% | 96% | 101% |

## Results for 2.10b (Test 2.8 + Test 2.9b)

### Results for 2.10b under 1.5x scaling ratio

|  | Low delay B Main10 (BD-rate PSNR1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,33% | -0,19% | -0,55% | 86% | 100% |
| Class C | -0,68% | -1,08% | -0,73% | 103% | 101% |
| Class E | -1,56% | -2,17% | -1,32% | 106% | 100% |
| Overall | -0,76% | -0,98% | -0,80% | 96% | 100% |
| Class D | -0,85% | -0,98% | -0,58% | 100% | 102% |
| Class F | -5,35% | -3,53% | -3,75% | 87% | 104% |

|  | Low delay B Main10 (BD-rate PSNR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,30% | -0,29% | -0,60% | 86% | 100% |
| Class C | -0,38% | -1,25% | -0,85% | 103% | 101% |
| Class E | -0,88% | -2,78% | -1,74% | 106% | 100% |
| Overall | -0,47% | -1,23% | -0,97% | 96% | 100% |
| Class D | -0,53% | -1,17% | -0,76% | 100% | 102% |
| Class F | -1,92% | -3,01% | -2,91% | 87% | 104% |

### Results for 2.10b under 2.0x scaling ratio

|  | Low delay B Main10 (BD-rate PSNR1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,14% | -0,74% | -0,57% | 86% | 98% |
| Class C | -0,44% | -1,28% | -1,44% | 107% | 100% |
| Class E | -0,92% | -0,19% | -1,20% | 107% | 102% |
| Overall | -0,43% | -0,78% | -1,02% | 98% | 100% |
| Class D | -0,73% | -0,03% | -2,01% | 102% | 100% |
| Class F | -3,52% | -0,08% | -0,70% | 92% | 99% |

|  | Low delay B Main10 (BD-rate PSNR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,06% | -0,91% | -0,70% | 86% | 98% |
| Class C | -0,18% | -1,43% | -1,27% | 107% | 100% |
| Class E | -0,19% | -1,44% | -1,80% | 107% | 102% |
| Overall | -0,13% | -1,21% | -1,17% | 98% | 100% |
| Class D | -0,34% | -0,68% | -2,47% | 102% | 100% |
| Class F | -0,67% | -1,67% | -2,33% | 92% | 99% |

# Conclusion

In this contribution, results for EE2-2.8, EE2-2.9 and EE2-2.10 are reported. It is asserted that increasing the length of luma and chroma RPR filters can bring a healthy compression efficiency improvement and can also improve design consistency. It is proposed to adopt EE2-2.10a or EE2-2.10b into the next version of ECM.

# Reference

[1] R. Yu, K. Andersson, “AHG12: RPR luma filter modifications and RPR enabling fixes in ECM”, JVET-Z0062, April 2022.

[2] K. Andersson, R. Yu, “AHG12: Longer chroma filters for RPR in ECM”, JVET-Z0069, April 2022.

[3] V. Seregin, J. Chen, G. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang , “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-Z2024, April 2022.

[4] J. Luo, V. Seregin, “JVET functionality confirmation test conditions for reference picture resampling”, JVET-Q2015, Jan. 2020.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).