| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0043 |
| --- | --- |

| Title: | IntraTMP Adaptation for Camera Captured Contents |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Naser T. Poirier F. Galpin A. Robert | Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes enabling IntraTMP for all CTC classes. To well balance the bitrate gain and the coding time, certain adaptations of IntraTMP are proposed. Specifically, it is proposed to speed-up the template matching process by subsampling the search range and perform iterative refinement. Two variants are studied:

Variant 1:

AI: -0.23% -0.30% -0.26% with 103% EncT and 103% DecT

RA: xx% xx% xx% with xx% EncT and xx% DecT

Variant 2:

-0.21% -0.22% -0.26% with 103% EncT and 102% DecT

RA: xx% xx% xx% with xx% EncT and xx% DecT

# Introduction

IntraTMP is one of the first coding tools adopted in the ECM software (JVET-V0130). In the current common test conditions, it is only enabled for screen contents (classF and class TGM). For camera captured contents, the initial version of intraTMP did not show a good trade-of between gain and coding time. However, the coding gain of intraTMP in camera captured contents remains relatively large. Therefore, this contribution aims at keeping the gains with a reasonable complexity.

# Proposed method

The complexity of intraTMP is proportional to the template matching process, where the difference between the templates of the current block and of the predicted block is computed for each position within the search available range. In the current ECM, four regions of search areas are considered (Figure 1).

Figure 1 Search areas for intraTMP in ECM software

To reduce the coding complexity, the following is proposed:

- Search range subsampling: It is proposed to subsample each of the search area by a sampling factor “s=1<<i” (“s” is a variable of power 2). For example, if “s” equals two, only 1/4th of the template matching search is performed.

- Iterative refinement: After finding the best matching within the subsampled search area, the search is refined around the best match. The refinement is repeated with the sampling factor divided by 2, until the full pixel search.

In screen contents, the “s” value is kept unchanged (s=1), where full pixel search is performed without iterative refinement. In other contents, different values of “s” can be used. The “s” is signaled to the decoder by signaling the value “i” value in SPS.

# Experimental results

The proposed method is implemented on top of ECM-5.0. Two values of the sampling factor “s” are considered, s=4 and s=8. The following test results are obtained on top of ECM-5.0 under the common test conditions:

Table 1 proposed method (s=4) on top of ECM-5.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.01% | -0.07% | 104% | 101% |
| Class A2 | -0.29% | -0.50% | -0.46% | 103% | 103% |
| Class B | -0.29% | -0.38% | -0.33% | 100% | 100% |
| Class C | -0.09% | -0.11% | -0.02% | 104% | 106% |
| Class E | -0.44% | -0.50% | -0.45% | 107% | 109% |
| Overall | -0.23% | -0.30% | -0.26% | 103% | 103% |
| Class D | -0.02% | -0.03% | -0.10% | 102% | 105% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | -0.03% | 106% | 108% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.15% | -0.32% | -0.14% | 101% | 108% |
| Class C | -0.03% | 0.04% | -0.11% | 109% | 121% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.03% | -0.08% | 0.41% | 111% | 124% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 2 proposed method (s=8) on top of ECM-5.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.00% | 0.04% | 106% | 103% |
| Class A2 | -0.26% | -0.36% | -0.43% | 103% | 100% |
| Class B | -0.26% | -0.27% | -0.36% | 100% | 98% |
| Class C | -0.08% | -0.05% | -0.02% | 101% | 102% |
| Class E | -0.42% | -0.46% | -0.52% | 106% | 109% |
| Overall | -0.21% | -0.22% | -0.26% | 103% | 102% |
| Class D | -0.01% | -0.03% | -0.11% | 99% | 103% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.13% | -0.06% | 102% | 105% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.03% | 0.09% | -0.06% | 109% | 122% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -0.06% | 0.29% | 111% | 123% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).