| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0048-v1 |
| --- | --- |

| Title: | Some VVC text changes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Editorial proposed draft text |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang (Bytedance) Gary J. Sullivan (Microsoft) | Email: Email: | yekui.wang@bytedance.com garysull@microsoft.com |
| Source: | Bytedance, Microsoft |  |  |

_____________________________

# Abstract

This contribution suggests to include the following VVC text changes into JVET-AA2005 (New level and systems-related supplemental enhancement information for VVC (Draft 3), and the corresponding WG 5 output document, if any:

- Not to specify the constraints on the values of BinCountsInPicNalUnits and BinCountsInSubpicNalUnits for the unlimited level (i.e., level 15.5).

- A fix for bug ticket #1556 for an asserted typo in clause 7.4.2.4.5 (Order of VCL NAL units and their association to coded pictures), wherein the phrase "subpicture level index" is suggested to be replaced with "subpicture index".

# Constraints on BinCountsInPicNalUnits and BinCountsInSubpicNalUnits

The following notes from the JVET-Z meeting minutes under JVET-Z0122 have not been integrated into JVET-Z2005 and the corresponding WG 5 output document:

- It was asked of the limitation in 7.4.3.10 about constraining the number of bins per CU should still apply in the unlimited level. It was agreed that relaxing this constraint is desirable for the unlimited level.

- The same modification also needs to be applied to the VVC text of unlimited level.

In VVC, there are constraints on the maximum numbers of bins for each coded picture and for each subpicture. Obviously, both should not be specified for the unlimited level. Furthermore, since there is no constraint that is not specified in Annex A and depends on level, it is suggested to move these constraints to Annex A.

The suggested changes are provided below.

# In subclause 7.4.3.15, make the following changes:

…

The value of BinCountsInPicNalUnits shall be less than or equal to vclByteScaleFactor * NumBytesInPicVclNalUnits + ( RawMinCuBits * PicSizeInMinCbsY )  32.

NOTE – The constraint on the maximum number of bins resulting from decoding the contents of the coded slice NAL units could be met by inserting a number of rbsp_cabac_zero_word syntax elements to increase the value of NumBytesInPicVclNalUnits. Each rbsp_cabac_zero_word is represented in a NAL unit by the three-byte sequence 0x000003 (as a result of the constraints on NAL unit contents that result in requiring inclusion of an emulation_prevention_three_byte for each rbsp_cabac_zero_word).

…

For each subpicture with subpicture index subpicIdxA for which sps_subpic_treated_as_pic_flag[ subpicIdxA ] is equal to 1, the value of BinCountsInSubpicNalUnits shall be less than or equal to vclByteScaleFactor * NumBytesInSubpicVclNalUnits + ( RawMinCuBits * subpicSizeInMinCbsY ) ÷ 32.

NOTE – The constraints on the maximum number of bins resulting from decoding the contents of the coded slice NAL units of a coded picture or subpicture, as specified in clause A.4.1, could be met by inserting a number of rbsp_cabac_zero_word syntax elements to increase the value of NumBytesInPicVclNalUnits. Each rbsp_cabac_zero_word is represented in a NAL unit by the three-byte sequence 0x000003 (as a result of the constraints on NAL unit contents that result in requiring inclusion of an emulation_prevention_three_byte for each rbsp_cabac_zero_word).

…

# In subclause A.4.1, make the following changes:

…

When the specified level is not level 15.5, bitstreams conforming to a profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex C:

- PicSizeMaxInSamplesY shall be less than or equal to MaxLumaPs, where MaxLumaPs is specified in Table A.2.

- The value of PicWidthMaxInSamplesY shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

- The value of PicHeightMaxInSamplesY shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

- For each referenced PPS, the value of NumTileColumns shall be less than or equal to MaxTileCols and the value of NumTilesInPic shall be less than or equal to MaxTilesPerAu, where MaxTileCols and MaxTilesPerAu are specified in Table A.2.

- For each referenced PPS, the value of ColWidthVal[ i ] * CtbSizeY, for each i in the range of 0 to NumTileColumns − 1, inclusive, shall be less than or equal to 0.5 * Sqrt( Max( level4Val, MaxLumaPs ) * 8 ), where MaxLumaPs is specified in Table A.2 and level4Val is equal to 2 228 224.

NOTE – The maximum tile width in luma samples is also less than or equal to the picture width, which is less than or equal to Sqrt( MaxLumaPs * 8 ).

- For the VCL HRD parameters, CpbSize[ Htid ][ i ] shall be less than or equal to CpbVclFactor * MaxCPB for at least one value of i in the range of 0 to hrd_cpb_cnt_minus1, inclusive, where CpbSize[ Htid ][ i ] is specified in clause 7.4.6.3 based on parameters selected as specified in clause C.1, CpbVclFactor is specified in Table A.4 and MaxCPB is specified in Table A.2 in units of CpbVclFactor bits.

- For the NAL HRD parameters, CpbSize[ Htid ][ i ] shall be less than or equal to CpbNalFactor * MaxCPB for at least one value of i in the range of 0 to hrd_cpb_cnt_minus1, inclusive, where CpbSize[ Htid ][ i ] is specified in clause 7.4.6.3 based on parameters selected as specified in clause C.1, CpbNalFactor is specified in Table A.4, and MaxCPB is specified in Table A.2 in units of CpbNalFactor bits.

- The value of BinCountsInPicNalUnits shall be less than or equal to vclByteScaleFactor * NumBytesInPicVclNalUnits + ( RawMinCuBits * PicSizeInMinCbsY )  32.

- For each subpicture with subpicture index subpicIdxA for which sps_subpic_treated_as_pic_flag[ subpicIdxA ] is equal to 1, the value of BinCountsInSubpicNalUnits shall be less than or equal to vclByteScaleFactor * NumBytesInSubpicVclNalUnits + ( RawMinCuBits * subpicSizeInMinCbsY ) ÷ 32.

…

# A typo in clause 7.4.2.4.5 (Order of VCL NAL units and their association to coded pictures)

The first bullet item in clause 7.4.2.4.5 (Order of VCL NAL units and their association to coded pictures) is as follows:

- For any two coded slice NAL units A and B of a coded picture, let subpicIdxA and subpicIdxB be their subpicture level index values, and sliceAddrA and sliceAddrB be their sh_slice_address values.

It is believed that the phrase "subpicture level index" is a typo of "subpicture index", thus should be should be replaced with "subpicture index". In the specification there is no term of "subpicture level index" defined, and it is not used anywhere else. Furthermore, the context is clear that it was meant to refer to the subpicture index, and this can be easily seen from the variable names subpicIdxA and subpicIdxB.