| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0074-v1 |
| --- | --- |

| Title: | [EE1-related] Lighter WCDANN: CNN Based In-Loop Filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hao Zhang Cheolkon Jung Dan Zou Ming Li | Author(s) or Contact(s): | 13227706628@163.com zhengzk@xidian.edu.cn zoudan@oppo.com myron.li@oppo.com |
| Source: | Xidian University, OPPO |  |  |

_____________________________

- Abstract

This contribution presents convolutional neural network-based in-loop filters. In this contribution, we modify the network structure and training strategy of WCDANN proposed by JVET-Y0052, make WCDANN lightweight, and provide two filters based on the QP value, namely Filter1 and Filter2. Compared with VTM-11.0-NNVC, the proposed Filter1 achieves average {5.29%, 12.53%, 13.07%} BD-rate reductions for {Y, Cb, Cr} under AI configuration, and the proposed Filter2 achieves average {5.12%, 13.08%, 13.93%} BD-rate reductions for {Y, Cb, Cr} under AI configuration.

Introduction

A CNN based post-processing filter was proposed in JVET-Y0052 for compression artifact removal, called WCDANN. However, WCDANN is only trained to enhance the Y channel component, and it is still not lightweight enough. In response to these problems, we make some improvements to WCDANN. First, we concatenate the Y and UV channels at the input and feed them into the network for training so that the network can process both luma and chroma components. Second, we downsample the feature maps after feature extraction and perform channel segmentation in the residual block, which greatly speeds up the network. Furthermore, we use the prediction map and partition map as auxiliary information to improve the performance of the network. In addition, we propose two filters: Filter1 can be used to process videos with multiple QPs by using QP map as input, and Filter2 contains five models to process videos under different QPs respectively.

Proposed method

Network architecture

The network architecture of the proposed CNN post-processing filter is shown in Fig. 1. Compared with JVET-Y0052, the following improvements are mainly made. First, auxiliary information (partition map and prediction map) is added to the input part to make full use of the information in the loop and better improve the network performance. Secondly, a downsample operation (implemented by common convolution) is introduced before the backbone of the network, which greatly improves the running speed of the network. At the same time, the number of channels is doubled to reduce the loss of feature information as much as possible. In addition, in the residual block, the channel segmentation operation is introduced, which once again reduces the computational complexity of the network, and the modified residual block is shown in Fig.2. Finally, an upsample operation (implemented by pixelshuffle) is introduced after the reconstruction part of the network to upsample the residual map to the same size as the original luma component.

It should be noted that for the two proposed filters, the only difference is that compared to Filter 2, Filter 1 additionally introduces a QP map in the input.

Fig. 1 Network architecture of the proposed Lighter WCDANN. The parameter M is set to 4.

Fig. 2 Network architecture of modified RAB. The parameter p is set to 0.5.

Training

In the training phase, we used DIV2K [2] and BVI-DVC [3] as datasets and used EE1 anchors [4] to compress them. It should be noted that we obtain reconstruction map, prediction map, and partition map directly after LMCS, which ensures that the features of the input image in the training and testing phases are consistent as much as possible. More details are shown in Table 1.

Table 1. Network information of the proposed post-processing filter in the training stage.

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: GeForce RTX 3090 |
|  | Framework: | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 200 |
|  | Batch size: | 16 |
|  | Training time: | ~100h/model |
|  | Training data information: | BVI-DVC, DIV2K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0, QP {22, 27, 32, 37, 42} |
|  | Loss function: | Weighted L1 and L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | random flipped |
|  | Other information: |  |

Inference

During the inference phase, we embed the network into the EE1 anchor, placed between the LMCS and DBF, which is consistent with how the training data was obtained. At the same time, since the training data of the network are I Slice, we believe it is good at processing I Slice, so we disable DBF and SAO for I Slice and rely on the proposed filter to process them.

More details of the inference process are shown in Table 2.

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
|  | Filters | Filter 1 and Filter 2 |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Libtorch v1.9.0 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 1.3M |
|  | Total Number of Parameters (All Models) | Filter 1: 1.3M total Filter 2: 6.5M total |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | Filter 1: 5.4M total Filter 2: 27.0M total |
|  | Multiply Accumulate (MAC)/pixel |  |
| Optional |  |  |
|  | Total Conv. Layers | 48 Depthwise separable convolution 24 Common convolution |
|  | Total FC Layers | 40 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144x144 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Experimental results

The proposed CNN post-processing filter is evaluated based on VTM-11.0-NNVC according to the common test conditions (CTCs) defined in [5]. We perform the evaluation in the All Intra (AI) configurations. Experimental results of filter 1 are shown in Tables 3. Experimental results of filter 2 are shown in Tables 4. In the AI configuration, the proposed filter achieves {5.29%, 12.53%, 13.07%} BD-rate reduction on average for {Y, Cb, Cr} channel, and the proposed filter2 achieves {5.12%, 13.08%, 13.93%} BD-rate reduction on average for {Y, Cb, Cr} channel.

Filter 1

Table 3 BD rate of WCDANN over VTM-11.0 NNVC in AI configuration.

|  | All Intra Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | bit DIFF |  |
| Class C | -5.09% | -13.18 % | -13.07 % | -6.39 % | -16.29 % | -16.16 % | 0 % |  |
| Class D | -5.48% | -11.88 % | -13.07 % | -6.11 % | -16.14 % | -17.06 % | 0 % |  |

Filter 2

Table 4 BD rate of WCDANN over VTM-11.0 NNVC in AI configuration.

|  | All Intra Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | bit DIFF |  |
| Class C | -4.95% | -13.34 % | -13.58 % | -6.01 % | -17.23 % | -17.91 % | 0 % |  |
| Class D | -5.29% | -12.82 % | -14.28 % | -5.43 % | -17.03 % | -18.87 % | 0 % |  |

Discussion

As shown in Tables 3 and 4, the performance of the two filters is very similar. A close look reveals that Filter2 achieves better performance in the chroma component than Filter1, while its performance in the luma component is slightly worse. Since the parameter quantity of Filter1 is one-fifth of Filter2, the overall performance of Filter1 is better. In addition, Filter2 contains total five models, but the performance is relatively limited. It seems that the generalization performance of Filter2 on images of different quality is not as good as that of Filter1. Because for a compressed image under a certain QP, the quality of each patch is different, and it is difficult for a single QP network to adapt to these high or low quality patches, resulting in the limited performance.

Conclusions

This contribution presents CNN based in-loop filters. We have upgraded the JVET-Y0052 filter, making it suitable for processing both luma and chroma components, while greatly increasing the speed of the network. Specifically, we propose two filters (Filter1 and Filter2) and verify the performance of the modified network under the AI configuration. Experimental results demonstrate that Filter1 and Filter2 achieve average {5.29%, 12.53%, 13.07%} and {5.12%, 13.08%, 13.93%} BD-rate reductions over VTM-11.0-NNVC anchor for {Y, Cb, Cr} channels, respectively.

In our future work, we are planning to collect B Slice images to train the CNN filters in RA configurations.  We recommend further study on the CNN filters by including it in EE1.

References

- H. Zhang, C. Jung, D. Zou and M. Li, “AHG11: CNN post-processing filter based on depthwise separable convolution and attention mechanism,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 Document JVET-Y0052, January 2022.

- F. Zhang, C. Feng and D. R. Bull, "Enhancing VVC Through Cnn-Based Post-Processing," Proceedings of the IEEE Conference on Multimedia and Expo (ICME), 2020, pp. 1-6, doi: 10.1109/ICME46284.2020.9102912.

- Di Ma, Fan Zhang, and David Bull, “BVI-DVC: A Training Database for Deep Video Compression”, arXiv:2003.13552, 2020.

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc

- S. Liu, A. Segall, E. Alshina, and R.-L. Liao, “JVET common test conditions and evaluation procedures for neural network-based video coding technology,” JVET-W2016, April 2021.

Patent rights declaration(s)

Xidian University and Guangdong OPPO Mobile Telecommunications Corp., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).