| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0078-v2 |
| --- | --- |

| Title: | EE2-1.6: Weighted chroma prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Zhenyao Zhang, Hongqing Du, YanZhuo Ma, Fuzheng Yang, Jing Ren, Ming Li | Email: | jyhuo@mail.xidian.edu.cn zhyzhang@stu.xidian.edu.cn hqdu@stu.xidian.edu.cn yzma@mail.xidian.edu.cn fzhyang@mail.xidian.edu.cn renjing1@oppo.com myron.li@oppo.com |
| Source: | Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of EE2 Test 1.6 on weighted chroma prediction (WCP) method. The new chroma intra prediction is proposed, that predicted chroma samples of the current block are derived based on the weights and neighboring chroma samples and the weights are designed according to the difference between neighboring luma samples and the reconstructed luma samples of a coding block as proposed originally in JVET-Z0100. It is reported that on top of ECM-5.0, the overall coding performance impact for {Y, U, V, EncT, DecT} is:

- Test 1.6a (Weighted chroma prediction):

AI {-0.08%, -0.47%, -0.44%, xxx%, 99%}

- Test 1.6b (Convolutional cross-component intra prediction model + Weighted chroma prediction):

AI {-1.42%, -3.11%, -3.01%, xxx%, 102%}

Introduction

In ECM5.0 [1], both cross-component linear model prediction(CCLM) and multi-model linear model prediction(MMLM) i.e., linear model(LM) approaches use correlation between luma and chroma components to obtain predicted chroma samples. When LM mode is used, the chroma samples are predicted from the reconstructed luma samples of co-located block as follows:

PredCxy=α×CurL'xy+β

where PredC indicates predicted chroma samples in a block and CurL indicates the reconstructed luma samples in the block. Parameters α and β are derived from neighboring reconstructed samples of the current block. For subsampled color format, downsampled reconstructed luma CurL' is necessary for LM modes to match size of chroma signal.

In order to improve the coding efficiency of chroma intra prediction, one method is proposed in the last JVET meeting [2], and studied in EE2 Test 1.6 [3], including the weighted chroma prediction (WCP) method.

Proposed method

Test 1.6a: Weighted chroma prediction

In this contribution, a weighted chroma prediction (WCP) method is proposed. Fig. 1 depicts the procedure of WCP. For each sample CurL'xy in the downsampled luma block, the luma difference vector DeltaL is first obtained according to the absolute value of the difference between CurL'xy and the neighboring luma vector; second, the normalized weight vector Weight is derived according to a nonlinear mapping model related to DeltaL; third, with the weight vector, the predicted chroma sample PredCxy can be obtained by performing vector multiplication based on the weight vector and the neighboring chroma vector.

Fig. 1 The procedure of weighted chroma prediction (WCP)

Specifically, the predicted chroma samples PredCxy with x = 0…W − 1, y = 0…H − 1 are derived as follows:

PredCxy=k=0K-1Weightxyk×RefCk

Test 1.6b: Test 1.1a + Test 1.6a

In the Test 1.6b the convolutional cross-component intra prediction model and the weighted chroma prediction are combined. Specifically, the weighted chroma prediction described in Test 1.6a is applied. And details of the convolutional cross-component intra prediction model can be found in the report provided for Test 1.1a.

Simulation results

The described methods are implemented on top ECM-5.0 software [4] and are tested under CTC for enhanced compression tool testing [5].

Table 1. Simulation results of Test 1.6a

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.60% | -0.58% | #NUM! | 100% |
| Class A2 | -0.05% | -0.65% | -0.48% | #NUM! | 101% |
| Class B | -0.05% | -0.47% | -0.54% | #NUM! | 99% |
| Class C | -0.09% | -0.39% | -0.33% | #NUM! | 98% |
| Class E | -0.04% | -0.28% | -0.24% | #NUM! | 98% |
| Overall | -0.08% | -0.47% | -0.44% | #NUM! | 99% |
| Class D | -0.06% | -0.20% | -0.29% | #NUM! | 99% |
| Class F | -0.13% | -0.35% | -0.44% | #NUM! | 99% |

Table 2. Simulation results of Test 1.6b

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.26% | -3.18% | -5.11% | #NUM! | 103% |
| Class A2 | -5.61% | -4.53% | -7.27% | #NUM! | 101% |
| Class B | -0.61% | -3.56% | -1.85% | #NUM! | 102% |
| Class C | -0.19% | -1.16% | -0.97% | #NUM! | 102% |
| Class E | 0.37% | -3.50% | -1.31% | #NUM! | 102% |
| Overall | -1.42% | -3.11% | -3.01% | #NUM! | 102% |
| Class D | -0.13% | -0.95% | -0.82% | #NUM! | 102% |
| Class F | -0.95% | -3.51% | -2.87% | #NUM! | 100% |

References

- M. Coban, et. al., “Algorithm description of Enhanced Compression Model 5 (ECM 5),” JVET-Z2025, April 2022.

- J. Huo, et. al., “Non-EE2: Weighted Chroma Prediction (WCP),” JVET-Z0100, April 2022.

- V. Seregin, et. al., “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-Z2024, April 2022.

- ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-5.0

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).