| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0096-v1 |
| --- | --- |

| Title: | EE2-2.2: Motion compensated picture boundary padding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fabrice Le Léannec, Pierre Andrivon, Miloš Radosavljevć (Xiaomi) Zhi Zhang, Han Huang, Chun-Chi Chen, Yao-Jen Chang, Y. Zhang, Vadim Seregin, Muhammed Coban, Marta Karczewicz (Qualcomm) | Tel: Email: | fabricel@xiaomi.com zhizhang@qti.qualcomm.com |
| Source: | Xiaomi, Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the experiment results of tests EE2-2.2a and EE2-2.2b. These two tests introduce motion compensated picture boundary padding on top of ECM-5.0. When an inter block is predicted with a reference block partially or completely out of the reference picture bounds, then the outside samples are computed through the proposed motion compensation-based padding.

In the implementation, test 2.2a increases the padded area size by 64 luma samples compared to ECM-5.0 to keep MV clipping, which implements repetitive padding, non normative. Test 2.2b increases the padded area by 16 luma samples, targeting reduced memory consumption compared to test 2.2a.

On top of ECM-5.0, reported simulation results (BDR Y/Cb/Cr, encoder time, decoder time) are as below:

- Test 2.2	a:

  - RA: -0.13%	 / -0.09% / -0.16%, 100%, 102%  ;    LB: -0.02% / -0.19% / 0.02%, 99%, 101%

- Test 2.2b:

  - RA: -0.13% / -0.08% / -0.14%, 101%, 102%  ;   LB:  -0.05% / -0.11% / -0.02%, 100%, 102%

# Introduction

In ECM-5.0 [1][2], bi-prediction is performed in a way that avoids relying on reference samples out of a reference picture bounds (OOB), if possible.

To do so, in the case of a bi-predicted block with an OOB reference block in one of the two reference pictures, the OOB prediction samples are not used. The concerned part of the block is rather uni-predicted based on non-OOB prediction samples, if available in the other reference picture.

However, for a uni-predicted block with an OOB reference block or for a bi-directional predicted block with both OOB reference samples, repetitive padded pixels are used for MC.

That is, in ECM-5.0, pictures are extended by an area surrounding the picture with a size of (maxCUwidth + 16) in each direction of the picture boundary. The pixel in the extended area is derived by repetitive boundary padding. When a reference block used for uni-prediction locates partially or completely out of the picture boundary (OOB), the repetitive padded pixel is used for motion compensation (MC).

In JVET-Z0130 [3], a method called motion compensated boundary padding replaces the repetitive boundary padding, for increased coding efficiency.

# Proposed method

In the proposed method, samples outside of the picture boundary are derived by motion compensation instead of using only repetitive padding as in ECM. In the implementation, the total padded area size is increased by 64 (test 2.2a) or 16 (test 2.2b) compared to ECM. This is to keep MV clipping, which implements repetitive padding, non-normative.

Figure 1. MC boundary padding method

For motion compensation padding, MV of a 4×4 boundary block is utilized to derive a M×4 or 4×M padding block. The value M is derived as the distance of the reference block to the picture boundary as shown on Figure 2. Moreover, M is set at least equal to 4 as soon as the motion vector points to a position internal to the reference picture bounds. If boundary block is intra coded, then MV is not available, and M is set equal to 0. If M is less than 64, the rest of the padded area is filled with the repetitive padded samples.

Figure 2. An example of deriving a M×4 padding block with a left padding direction

In case of bi-directional inter prediction, only one prediction direction, which has a motion vector pointing to the pixel position farther away from the picture boundary in the reference picture in terms of the padding direction, is used in MC boundary padding.

The pixels in MC padding block are corrected with an offset, which is equal to the difference between the DC values of the reconstructed boundary block and its corresponding reference block.

# Simulation Results

This section reports the simulation results of proposed motion compensated boundary padding, according to EE2 test conditions [3].

## Test 2.2a

The following Table 1 reports the performances of test 2.2.a. It can be observed the proposed method brings -0.13% luma coding gain with no complexity change over ECM-5.0.

Table 1: performances of test 2.2a (MC padding size 64) over ECM-5.0

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | 0.01% | -0.11% | 100% | 101% |
| Class A2 | -0.13% | 0.02% | -0.10% | 101% | 101% |
| Class B | -0.12% | -0.23% | -0.27% | 101% | 103% |
| Class C | -0.16% | -0.06% | -0.11% | 100% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.09% | -0.16% | 100% | 102% |
| Class D | -0.31% | -0.41% | -0.31% | 100% | 104% |
| Class F | -0.04% | -0.04% | -0.02% | 100% | 103% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over ECM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.12% | -0.15% | 98% | 99% |
| Class C | -0.03% | -0.20% | 0.34% | 100% | 102% |
| Class E | -0.06% | -0.29% | -0.11% | 101% | 104% |
| Overall | -0.02% | -0.19% | 0.02% | 99% | 101% |
| Class D | -0.08% | 0.80% | 0.62% | 101% | 102% |
| Class F | -0.29% | -0.52% | -0.36% | 99% | 103% |

## Test 2.2b

The following Table 2 reports the performances of test 2.2.b. It can be observed the proposed method brings similar performances as test 2.2a (-0.13% coding gain in luma) with no complexity change over ECM-5.0.

It shows the memory increase due to the MC padding method can be significantly reduced to a very limited amount, without any penalty in coding efficiency.

Table 2: performances of test 2.2a (MC padding size 64) over ECM-5.0

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.10% | -0.14% | 100% | 101% |
| Class A2 | -0.11% | 0.02% | -0.01% | 101% | 100% |
| Class B | -0.13% | -0.16% | -0.24% | 101% | 103% |
| Class C | -0.15% | -0.05% | -0.10% | 101% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.08% | -0.14% | 101% | 102% |
| Class D | -0.33% | -0.53% | -0.33% | 100% | 104% |
| Class F | -0.05% | -0.05% | 0.00% | 100% | 103% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over ECM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.19% | -0.12% | 100% | 101% |
| Class C | -0.03% | 0.03% | 0.05% | 100% | 102% |
| Class E | -0.10% | -0.18% | 0.07% | 101% | 104% |
| Overall | -0.05% | -0.11% | -0.02% | 100% | 102% |
| Class D | -0.07% | 0.83% | 0.80% | 101% | 101% |
| Class F | -0.09% | -0.21% | -0.38% | 99% | 103% |

# Conclusion

This contribution reports the results of EE2 tests 2.2a and 2.2b. It shows -0.13% BD-rate gain can be achieved in random access in luma, without any runtime increase, over ECM-5.0.

Furthermore, the size of the motion compensation padded area can be reduced to 16 luma samples with no penalty in performance.

As a conclusion, it is proposed to adopt test 2.2b in next version of ECM.

# Cited references

[1] ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM

[2] M. Coban, F. Le Léannec, K. Naser, J. Ström, « Algorithm description of Enhanced Compression Model 5 (ECM 5)”, JVET-Z2025, April 2022.

[3] Zhi Zhang, Han Huang, Chun-Chi Chen, Yao-Jen Chang, Y. Zhang, Vadim Seregin, Muhammed Coban, Marta Karczewicz (Qualcomm), Fabrice Le Léannec, Pierre Andrivon, Miloš Radosavljevć, Emmanuel Thomas (Xiaomi) “EE2-related: Motion compensation boundary padding, JVET-Z0130, April 2022.

[4] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, Oct. 2021.

# Patent rights declaration(s)

Xiaomi may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).