| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0105-v1 |
| --- | --- |

| Title: | AHG9: Metadata for display on transparent screens based on ACI SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Emmanuel Thomas Pierre Andrivon Fabrice Le Léannec Miloš Radosavljevć Mary-Luc Champel | Email: | thomase@xiaomi.com pandrivon@xiaomi.com fabricel@xiaomi.com milosr@xiaomi.com champelmaryluc@xiaomi.com |
| Source: | Xiaomi |  |  |

_____________________________

# Abstract

Following up on JVET-Y0104 and JVET-Z0129, this contributions asserts that the signaling of metadata for assisting with the adjustment of the transparency effect of video sequences on transparent screens has two main benefits. First, it is asserted that this metadata would prevent a degraded quality of experience when displaying conventional content on transparent screens due to the possible loss of important visual content made unintentionally transparent (e.g. missing or barely visible logo, text, objects, etc.). Second, it is also asserted that a content creator may leverage the metadata to use the transparency effect as a way to enhance the narrative effect (e.g. visual content fading in and out in the viewer’s surroundings, highlighting an important part of the image such as a character, a face, an object, etc..).

Following the recommendations from JVET 26th, this contribution provides a technical solution to carry this metadata and enabling the use case by reusing the alpha plane auxiliary pictures and the associated alpha channel information (ACI) SEI messages. The changes introduce a new mode for the parameter alpha_channel_use_idc as well as semantic constraints for this new mode for the values of the parameters alpha_transparent_value and alpha_opaque_value. Lastly, an informative note is provided to describe the possible post decoding operations for this new mode as it is done for the existing modes. As comparison point, an approach is also presented for enabling the use case with the same benefits using a new type of auxiliary pictures.

# Introduction

Over the past few years, the LCD and OLED transparent display technologies significantly progressed. This allowed the commercialization in different markets such as head-mounted displays, digital signage and recently in consumer electronics with models manufactured by Xiaomi, Panasonic and LG. It is to be expected that such screens will become progressively more popular as seen in recent tradeshows such as the CES 2022 [6].

This contribution is a follow-up of JVET-Y0104 [1] and JVET-Z0129 [2] which argues for the signaling of metadata for assisting with adjusting the transparency effect of the decoded video sequence at the receiver side. The benefits are twofold. First, this metadata prevents a degraded quality of experience when displaying conventional content on transparent screens due to the possible loss of important visual content made unintentionally transparent (e.g. missing or barely visible logo, text, objects, etc.). Second, a content creator may leverage the metadata to use the transparency effect as a way to enhance the narrative effect (e.g. visual content fading in and out in the viewer’s environment, highlighting an important part of the image such as a character, a face, an object, etc..).

At JVET #25, the group recommended to follow an approach based an auxiliary picture concept. At JVET #26, it was further recommended to study:

- whether the alpha channel information (ACI) SEI message under standardization may be reused for this use case; and,

- the possible interaction with the use case of existing alpha blending.

Following this recommendation, this contribution provides the outcome of this study. Although the alpha plane auxiliary pictures do transport coefficients relative to samples in the main video, we identified two drawbacks with reusing the ACI SEI message as is:

- The syntax and semantics of the ACI SEI message are coupled with the alpha blending operation as intended purpose.

- Since the alpha plane coefficients would be different for alpha blending and transparent screen display purposes, using the same ACI SEI message would not allow to distinguish the intended purpose (alpha blending or transparent screen display) and not guarantee interoperability.

As a result, this contribution proposes modifications to the ACI SEI message’s semantic under standardization to alleviate those technical issues in order to pursue the recommendations formulated during JVET #26. In addition, the contribution also presents the originally proposed approach using a new auxiliary picture type as a comparison point. The two options thus described in this contribution are:

- Option 1: New usage mode indicator in the alpha channel information (ACI) SEI message

- Option 2: New transparency mask SEI message

# Motivations

## General

As explained in the introduction, the goal of the contribution is to study how the current ACI SEI message could be extended for the use case of transparent display. The following clause briefly reminds the concept of alpha blending based on an alpha channel and then presents the new proposed use case of transparent display using an alpha channel.

## Alpha channel for alpha blending operation (conventional use case)

In video and image processing, the alpha channel provides the alpha planes corresponding to the amount of transparency associated with luma samples in the associated picture or video. The name alpha channel comes from the corresponding image processing operation called alpha blending wherein a foreground picture is combined with a background picture. The formula defining this operation has been historically written with an α (alpha) character, hence the name alpha channel.

In the VSEI draft 6 (JVET-Y2006) [4], the following definition is provided for alpha blending:

alpha blending: A process not specified by this Specification, in which an auxiliary coded picture is used in combination with a primary coded picture and with other data not specified by this Specification in the display process. In an alpha blending process, the samples of an auxiliary coded picture are interpreted as indications of the degree of opacity (or, equivalently, the degrees of transparency) associated with the corresponding luma samples of the primary coded picture.

An example of alpha channel use for overlaying is given in Figure 1.

Figure 1 - Example of alpha channel usage (HEVC Video with Alpha, Apple’s WWDC 2019 [3])

In terms of device support, specifications using alpha channel typically mandate the support by the device since the content cannot be used as intended otherwise. In the case of Figure 1, it would be indeed of little interest for a device to display the image tree without the interpretation of the transparency information.

## Transparency information for transparent display (proposed use case)

As presented in previous contributions [1] and [2], transparent screens provide the ability to the viewer to see through the screen. The transparency effect perceived can depend on the local luminosity on the panel, the ambient lighting conditions and the possible presence of a dimming back panel on the screen. Figure 2 is an example of a transparent TV commercialized by Xiaomi.

Figure 2 - Xiaomi Mi TV LUX [5]

Regarding the use of alpha channel for transparent display, the samples of the auxiliary coded picture are to be interpreted as indications of the degree of opacity associated with their corresponding positions in the display area.

That is, the device may implement different techniques in order to achieve the transparency effect corresponding to the artistic intent. One of such techniques is to adjust the decoded sample values of the main video, similarly as an alpha blending operation, considering the main video as the foreground and the background as a black image for OLED screens, respectively as a white image for LCD screens. If the device is equipped with a dimming back panel blocking the light coming from behind the screen, the device may instead, or in addition to the sample value adjustment, control the intensity of this dimming back panel. The sample value adjustment may even further be adjusted based on ambient lighting measurement.

# Proposals

## Option 1: New usage mode indicator in SEI alpha channel message

### Design approach

In order to reuse the ACI SEI message for the purpose of transparent screen display, the following approach is proposed:

- Reuse of the ACI SEI message with minimum syntax and semantics changes.

- Enable interoperability by distinguishing between alpha blending and transparent display purposes with a new value (mode ‘3’) for the syntax element alpha_channel_use_idc.

- Constrain the values of some of the ACI SEI message’s syntax elements for the new mode ‘3’.

### Updates to ACI syntax and semantics

  - Alpha channel information SEI message

8.23.1 Alpha channel information SEI message syntax

(no syntax change needed)

| alpha_channel_info( payloadSize ) { | Descriptor |
| --- | --- |
| alpha_channel_cancel_flag | u(1) |
| if( !alpha_channel_cancel_flag ) { |  |
| alpha_channel_use_idc | u(3) |
| alpha_channel_bit_depth_minus8 | u(3) |
| alpha_transparent_value | u(v) |
| alpha_opaque_value | u(v) |
| alpha_channel_incr_flag | u(1) |
| alpha_channel_clip_flag | u(1) |
| if( alpha_channel_clip_flag ) |  |
| alpha_channel_clip_type_flag | u(1) |
| } |  |
| } |  |

8.23.2 Alpha channel information SEI message semantics

The alpha channel information (ACI) SEI message provides information about alpha channel sample values and post-processing applied to the decoded alpha planes coded in auxiliary pictures of type AUX_ALPHA and one or more associated primary pictures.

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i, no picture in the CVS shall be associated with an ACI SEI message.

When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i and an ACI SEI message, the SDI SEI message shall precede the ACI SEI message in decoding order.

When an access unit contains an auxiliary picture picA in a layer, with nuh_layer_id equal to nuhLayerIdA, that is indicated as an alpha auxiliary layer by an SDI SEI message, the alpha channel sample values of picA persist in output order until one or more of the following conditions are true:

–	The next picture, in output order, with nuh_layer_id equal to nuhLayerIdA is output.

–	A CLVS containing the auxiliary picture picA ends.

–	The bitstream ends.

–	A CLVS of any associated primary layer of the auxiliary picture layer with nuh_layer_id equal to nuhLayerIdA ends.

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the ACI SEI message applies.

alpha_channel_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous ACI SEI message in output order that applies to the current layer. alpha_channel_cancel_flag equal to 0 indicates that ACI follows.

Let currPic be the picture that the ACI SEI message is associated with. The semantics of ACI SEI message persist for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an ACI SEI message is output that follows the current picture in output order.

alpha_channel_use_idc equal to 0 indicates that for alpha blending purposes the decoded samples of the associated primary picture should be multiplied by the interpretation sample values of the auxiliary coded picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 1 indicates that for alpha blending purposes the decoded samples of the associated primary picture should not be multiplied by the interpretation sample values of the auxiliary coded picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 2 indicates that the usage of the auxiliary picture is unspecified. alpha_channel_use_idc equal to 3 indicates that for transparent display area purposes the decoded samples of the associated primary picture should be displayed with an opacity level corresponding to the interpretation sample values of the auxiliary coded picture and these decoded samples may be adjusted in the display process after output from the decoding process. Values greater than 3 for alpha_channel_use_idc are reserved for future use by ITU-T | ISO/IEC. When not present, the value of alpha_channel_use_idc is inferred to be equal to 2. Decoders shall ignore alpha channel information SEI messages in which alpha_channel_use_idc is greater than 3.

alpha_channel_bit_depth_minus8 plus 8 specifies the bit depth of the samples of the luma sample array of the auxiliary picture. alpha_channel_bit_depth_minus8 plus 8 shall be equal to the bit depth of the associated primary picture.

alpha_transparent_value specifies the interpretation sample value of an auxiliary coded picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered transparent for purposes of alpha blending. When alpha_channel_use_idc is equal to 3, alpha_transparent_value shall be equal to alpha_opaque_value. The number of bits used for the representation of the alpha_transparent_value syntax element is alpha_channel_bit_depth_minus8 + 9.

alpha_opaque_value specifies the interpretation sample value of an auxiliary coded picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered opaque for purposes of alpha blending. When alpha_channel_use_idc is equal to 3, alpha_transparent_value shall be equal to alpha_opaque_value.  The number of bits used for the representation of the alpha_opaque_value syntax element is alpha_channel_bit_depth_minus8 + 9.

alpha_channel_incr_flag equal to 0 indicates that the interpretation sample value for each decoded auxiliary picture luma sample value is equal to the decoded auxiliary picture sample value for purposes of alpha blending. alpha_channel_incr_flag equal to 1 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample value that is greater than Min( alpha_opaque_value, alpha_transparent_value ) should be increased by one to obtain the interpretation sample value for the auxiliary picture sample and any auxiliary picture luma sample value that is less than or equal to Min( alpha_opaque_value, alpha_transparent_value ) should be used, without alteration, as the interpretation sample value for the decoded auxiliary picture sample value.

When alpha_channel_incr_flag is equal to 1, alpha_transparent_value shall not be equal to alpha_opaque_value and Log2( Abs( alpha_opaque_value − alpha_transparent_value ) ) shall have an integer value. A value of alpha_transparent_value that is equal to alpha_opaque_value indicates that the auxiliary coded picture is not intended for alpha blending purposes.

NOTE 1 – For alpha blending purposes, alpha_opaque_value may be greater than alpha_transparent_value, or it may be less than alpha_transparent_value. Interpretation sample values should be clipped to the range of alpha_opaque_value to alpha_transparent_value, inclusive.

alpha_channel_clip_flag equal to 0 indicates that no clipping operation is applied to obtain the interpretation sample values of the decoded auxiliary picture. alpha_channel_clip_flag equal to 1 indicates that the interpretation sample values of the decoded auxiliary picture are altered according to the clipping process described by the alpha_channel_clip_type_flag syntax element. When not present, the value of alpha_channel_clip_flag is inferred to be equal to 0.

alpha_channel_clip_type_flag equal to 0 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample that is greater than ( alpha_opaque_value + alpha_transparent_value ) / 2 is set equal to alpha_opaque_value to obtain the interpretation sample value for the auxiliary picture luma sample and any auxiliary picture luma sample that is less or equal than ( alpha_opaque_value + alpha_transparent_value ) / 2 is set equal to alpha_transparent_value to obtain the interpretation sample value for the auxiliary picture luma sample. alpha_channel_clip_type_flag equal to 1 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample that is greater than alpha_opaque_value is set equal to alpha_opaque_value to obtain the interpretation sample value for the auxiliary picture luma sample and any auxiliary picture luma sample that is less than or equal to alpha_transparent_value is set equal to alpha_transparent_value to obtain the interpretation sample value for the auxiliary picture luma sample.

NOTE 2 – When both alpha_channel_incr_flag and alpha_channel_clip_flag are equal to one, the clipping operation specified by alpha_channel_clip_type_flag should be applied first followed by the alteration specified by alpha_channel_incr_flag to obtain the interpretation sample value for the auxiliary picture luma sample.

NOTE 3 – Alpha blending composition is normally performed with a background picture B, a foreground picture F, and a decoded auxiliary coded picture A, all of the same size. Assume for purposes of example illustration that the chroma resolution of B and F have been upsampled to the same resolution as the luma. Denote corresponding samples of B, F and A by b, f and a, respectively. Denote luma and chroma samples by subscripts Y, Cb and Cr.

Define the variables alphaRange, alphaFwt and alphaBwt as follows:

alphaRange = Abs( alpha_opaque_value − alpha_transparent_value )	(53)

alphaFwt = Abs( a − alpha_transparent_value )		(54)

alphaBwt = Abs( a − alpha_opaque_value )		(55)

Then, in alpha blending composition, samples d of the displayed picture D could be calculated as follows:

dY = ( alphaFwt * fY + alphaBwt * bY + alphaRange / 2 ) / alphaRange	(56)

dCb = ( alphaFwt * fCb + alphaBwt * bCb + alphaRange / 2 ) / alphaRange	(57)

dCr = ( alphaFwt * fCr + alphaBwt * bCr + alphaRange / 2 ) / alphaRange	(58)

The samples of pictures D, F and B could also represent red, green, and blue component values (see clause 7.3). Here the Y, Cb and Cr component values are assumed. Each component, e.g., Y, is assumed for purposes of example illustration above to have the same bit depth in each of the pictures D, F and B. However, different components, e.g., Y and Cb, need not have the same bit depth in this example.

If alpha_channel_use_idc is equal to 0, the following applies:

- F would be the decoded picture obtained from the decoded luma and chroma, and A would be the decoded picture obtained from the decoded auxiliary coded picture. In this case, the indicated example alpha blending composition involves multiplying the samples of F by factors obtained from the samples of A.

- A picture format that is useful for editing or direct viewing, and that is commonly used, is called pre-multiplied-black video. When the foreground picture was F, the pre-multiplied-black video S is given by

sY  = ( alphaFwt * fY ) / alphaRange		(59)

sCb = ( alphaFwt * fCb ) / alphaRange		(60)

sCr = ( alphaFwt * fCr ) / alphaRange		(61)

- Pre-multiplied-black video has the characteristic that the picture S will appear correct if displayed against a black background. For a non-black background B, the composition of the displayed picture D may be calculated as

dY  = sY + ( alphaBwt * bY + alphaRange / 2 ) / alphaRange	(62)

dCb = sCb + ( alphaBwt * bCb + alphaRange / 2 ) / alphaRange	(63)

dCr = sCr + ( alphaBwt * bCr + alphaRange / 2 ) / alphaRange	(64)

Otherwise, when alpha_channel_use_idc is equal to 1, alpha blending composition does not involve multiplication of the samples of S by factors obtained from the samples of A, S would be the decoded picture obtained from the decoded luma and chroma, and A would again be the decoded picture obtained from the decoded auxiliary coded picture.

NOTE 4 For purposes of display on transparent display area (i.e. alpha_channel_use_idc is equal to 3), the luma samples of the decoded picture P may be adjusted based on the decoded samples of the decoded auxiliary picture A. Assume for purposes of example illustration that both P an A are of the same spatial resolution, possibly after upsampling. Denote corresponding samples of P and A by p and a respectively. Denote d0 the luma sample value corresponding to a transparent region when displayed on the transparent display area, typically black for some transparent screens and white for some other transparent screens.
Then, luma samples d of the displayed picture D could be obtained as follows, omitting possible clipping if needed:

d = p * a / (1 << (alpha_channel_bit_depth_minus8 + 8) − 1) + d0 * (1 − a / (1 << (alpha_channel_bit_depth_minus8 + 8) − 1))

When other post decoding operations are expected to be performed on the decoded samples, e.g. alpha blending, film grain synthesis, etc., the modification of the samples for purposes of transparent display area should be executed last to reflect as close as possible the content creator intent.

Other variants or different techniques to adjust the transparency effect on the display area may also be implemented but they are unspecified by this Specification.

## Option 2: New transparency mask SEI message

Add new clauses 8.X.1 and 8.X.2.

8.X.1 Transparency mask SEI message syntax

| transparency_mask( payloadSize ) { | Descriptor |
| --- | --- |
| tm_cancel_flag | u(1) |
| if(!tm_cancel_flag) { |  |
| tm_transparent_luma_value_flag | u(1) |
| } |  |
| } |  |

      - 8.X.2 Transparency mask SEI message semantics

The transparency mask (TM) SEI message provides information about the interpretation of the transparency mask sample values coded in auxiliary pictures of type AUX_TRANSPARENCY_MASK and the post-processing intended to be applied to the one or more associated primary pictures.

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to 3 for at least one value of i, no picture in the CVS shall be associated with a TM SEI message.

When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to 3 for at least one value of i and a TM SEI message, the SDI SEI message shall precede the TM SEI message in decoding order.

When an access unit contains an auxiliary picture picA in a layer, with nuh_layer_id equal to nuhLayerIdA, that is indicated as a transparency mask auxiliary layer by an SDI SEI message, the transparency mask sample values of picA persist in output order until one or more of the following conditions are true:

–	The next picture, in output order, with nuh_layer_id equal to nuhLayerIdA is output.

–	A CLVS containing the auxiliary picture picA ends.

–	The bitstream ends.

–	A CLVS of any associated primary layer of the auxiliary picture layer with nuh_layer_id equal to nuhLayerIdA ends.

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the TM SEI message applies.

tm_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous transparency information layer SEI message in output order. tm_cancel_flag equal to 0 indicates that transparency information layer parameters follow.

tm_transparency_value_flag specifies the interpretation of the transparency mask sample values.

tm_transparent_luma_value_flag equal to 0 specifies that the transparency mask sample values indicate the level of opacity of the collocated samples in the one or more associated primary pictures when displayed on a transparent display area.

tm_transparent_luma_value_flag equal to 1 specifies that the transparency mask sample values indicate the level of transparency of the collocated samples in the one or more associated primary pictures when displayed on a transparent display area.

NOTE 1 For purposes of display on transparent display area, the luma samples of the decoded picture P may be adjusted based on the decoded samples of the decoded auxiliary picture M of type AUX_TRANSPARENCY_MASK. Assume for purposes of example illustration that both P an M are of the same spatial resolution, possibly after upsampling and that the bit depth of M is 8 . Denote corresponding samples of P and M by p and m respectively. Denote d0 the luma sample value corresponding to a transparent region when displayed on the transparent display area, black for some transparent screens and white for some other transparent screens.

Then, luma samples d of the displayed picture D could be obtained as follows, omitting possible clipping if needed:

d = p * m / (1 << 8 − 1) + d0 * (1 − m / (1 << 8 − 1)) when tm_transparency_value_flag is equal to 0,

d = p * (1 − m / (1 << 8 – 1)) + d0 * m / (1 << 8 − 1) when tm_transparency_value_flag is equal to 1

When other post decoding operations are expected to be performed on the decoded samples, e.g. film grain synthesis, alpha blending, etc., the modification of the samples for purposes of transparent display area should be executed last to reflect as close as possible the content creator intent.

Other variants or different techniques to adjust the transparency effect on the display area may also be implemented but they are unspecified by this Specification.

Add a new line for the new type in table 15.

8.19.2 Scalability dimension information SEI message semantics

Table 15 – Mapping of sdi_aux_id[ i ] to the type of auxiliary pictures

| sdi_aux_id[ i ] | Name | Type of auxiliary pictures |
| --- | --- | --- |
| 1 | AUX_ALPHA | Alpha plane |
| 2 | AUX_DEPTH | Depth picture |
| 3 | AUX_TRANSPARENCY_MASK | Transparency mask |
| 4..127 |  | Reserved |
| 128..159 |  | Unspecified |
| 160..255 |  | Reserved |

# Conclusion

As presented here and in JVET-Y0104 [1] and JVET-Z0129 [2], transparent screens are an emerging technology for consumer electronics with several manufacturers already commercializing such display. It is to be expected that those screens will become progressively more popular as seen in recent tradeshows such as CES 2022 [6]. To support this market and enable a higher quality of experience, we recommend adopting the proposed metadata in the upcoming amendment of the VSEI specification, based on the option preferred by the group.

# References

- AHG9: Transparency information SEI for transparent screens, JVET-Y0104, E. Thomas, P. Andrivon, F. Le Leannec, M.-L. Champel (Xiaomi), Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022

- AHG9: SEI for transparency information as dedicated layer for transparent screens, JVET-Z0129, E.Thomas, P. Andrivon, F. Le Leannec, M. Radosavljević, M.-L. Champel (Xiaomi), Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022

- HEVC Video with Alpha, Apple’s Worldwide Developers Conference (WWDC) 2019, source:  https://developer.apple.com/videos/play/wwdc2019/506/

- Additional SEI messages for VSEI (Draft 6), Jill Boyce, Gary J. Sullivan, Ye-Kui Wang, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022

- Mi TV Lux: full specifications, photo | XIAOMI-MI.com, https://xiaomi-mi.com/tv-sets/mi-tv-lux/

- LG is bringing transparent TVs to CES | Mashable, December 27, 2021, https://mashable.com/article/lg-transparent-oled-displays

# Patent rights declaration(s)

Xiaomi may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).