| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0107-v1 |
| --- | --- |

| Title: | EE2-2.1: Regression based affine candidate derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yan Zhang Han Huang Vadim Seregin Muhammed Coban Marta Karczewicz | Tel: Email: | yzh@qti.qualcomm.com hanhuang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution presents results for EE2-2.1. In EE2-2.1, regression based affine merge candidate derivation methods are tested. A total of 3 simulations have been performed. In test a, the regression based affine merge candidates are derived and added to the affine merge. In test b, on top of test a, the number of affine candidates for ARMC is increased from 15 to 30, but the output list size is kept as 15. Finally in test c, the diversity criterion for ARMC sorting tested in EE2-2.5 are applied on top of test b.

Simulation result for tests a, b, and c are as following:

2.1a : RA: {-0.13%, -0.15%, -0.16%, xxx%, xxx%}; LB: {-0.xx%, -0.xx%, -0.xx%, xxx%, xxx%}.

2.1b : RA: {-0.21%, -0.12%, -0.18%, xxx%, xxx%}; LB: {-0.xx%, -0.xx%, -0.xx%, xxx%, xxx%}.

2.1c : RA: {-0.25%, -0.17%, -0.27%, xxx%, xxx%}; LB: {-0.xx%, -0.xx%, -0.xx%, xxx%, xxx%}.

# Introduction

During the VVC standardization progress, the Regression based Motion Vector Field (RMVF) derivation method was proposed in JVET-L0171 which provides a new variety of subblock-based merge candidate. The motion vectors and center positions from the neighboring subblocks of the current CU, as illustrated in Figure 1, are used as the input to the linear regression process to derive a set of linear model parameters.

Figure 1: Illustration of the neighboring 4 x 4 subblocks that are used for RMVF parameter derivation [1]. W and H are the width and height of the current CU

In JVET-Z0125 [2], regression based affine candidate derivation method was proposed. the subblock motion field from a previous coded affine CU and the motion vectors from the adjacent subblocks of current CU are used as the input for the regression process. Compares to the regression process in JVET-L0171, the difference is that the predicted CPMVs instead of the subblock motion field for current block are derived as output. It was decided to test the proposed method in EE2.

# Proposal

This contribution reports the EE test results of the proposed method on top of ECM-5.0 [3]. A total of 3 tests have been performed.

In test a, the regression based affine merge candidates are derived and added to the affine merge list. Subblock motion field from a previously coded affine CU and motion information from adjacent subblocks of a current CU are used as the input to the regression process to derive proposed affine candidates.

The previously coded affine CU can be identified from scanning through non-adjacent positions [4] and the affine HMVP table.

Adjacent subblock information of current CU is fetched from 4x4 sub-blocks represented by the grey zone as depicted in Figure 1. For each sub-block, given a reference list, the corresponding motion vector and center coordinate of the sub-block may be used.

For each affine CU, up to 2 affine candidates can be derived. One with adjacent subblock information and one without. All the linear-regression-generated candidates are pruned and collected into one candidate sub-group, TM cost based ARMC process is applied when ARMC is enabled. Afterwards, up to N linear-regression-generated candidates are added to the affine merge list when N affine CUs are found.

In test b, the number of affine candidates for ARMC is increased from 15 to 30, the output list size is kept as 15. Finally in test c, the diversity criterion for ARMC sorting tested in EE2-2.5 are applied on top of test b.

# Simulation results

The proposed tests are implemented on top of ECM-5.0 following the description in JVET-Z2024 [5] and the simulation is performed under common test conditions as described in JVET-Y2017 [6]. The BD-Rate impact is summarized in the tables below.

Test 2.1a:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.12% | -0.16% | 94% | 100% |
| Class A2 | -0.32% | -0.22% | -0.41% | 97% | 102% |
| Class B | -0.10% | -0.21% | -0.17% | 101% | 102% |
| Class C | -0.04% | -0.05% | 0.03% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.15% | -0.16% | 98% | 102% |
| Class D | -0.10% | -0.16% | -0.06% | 100% | 100% |
| Class F | -0.16% | -0.15% | -0.09% | 101% | 103% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.10% | -0.15% | 0.15% | 97% | 100% |
| Class E | -0.18% | -0.09% | -0.06% | 100% | 95% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.23% | 0.51% | 0.27% | 97% | 100% |
| Class F | -0.24% | -0.33% | -0.06% | 98% | 93% |

Test 2.1b:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.10% | -0.23% | 92% | 100% |
| Class A2 | -0.44% | -0.23% | -0.28% | 94% | 103% |
| Class B | -0.15% | -0.16% | -0.23% | 102% | 105% |
| Class C | -0.10% | 0.01% | -0.01% | 102% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.21% | -0.12% | -0.18% | 98% | 103% |
| Class D | -0.15% | -0.24% | -0.11% | 102% | 102% |
| Class F | -0.19% | -0.16% | -0.09% | 101% | 106% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.09% | -0.37% | -0.17% | 101% | 102% |
| Class E | -0.35% | -0.54% | -0.17% | 105% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.22% | 0.86% | 0.48% | 102% | 104% |
| Class F | -0.62% | -0.52% | -0.02% | 99% | 94% |

Test 2.1c:

|  | Over ECM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -0.04% | -0.25% | 94% | 102% |
| Class A2 | -0.54% | -0.36% | -0.45% | 92% | 103% |
| Class B | -0.20% | -0.21% | -0.25% | 101% | 104% |
| Class C | -0.09% | -0.08% | -0.16% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.25% | -0.17% | -0.27% | 98% | 103% |
| Class D | -0.06% | -0.32% | 0.12% | 101% | 101% |
| Class F | -0.17% | -0.09% | -0.04% | 101% | 103% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.20% | -0.24% | -0.02% | 99% | 105% |
| Class E | -0.36% | -0.05% | 0.27% | 99% | 98% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.34% | 0.05% | 0.02% | 98% | 106% |
| Class F | -0.69% | -0.80% | -0.38% | 98% | 94% |

# References

- R. Ghaznavi-Youvalarim, A. Aminlou and J. Lainema, “Regression-Based Motion Vector Field for Video Coding”, IEEE Transactions on Circuits and Systems for Video Technology  Volume: 31, Issue: 5, May 2021.

- Y. Zhang, H. Huang, V. Seregin, M. Coban and M, Karczewicz, “EE2-related: Regression based affine candidate derivation”, JVET-Z0125, Apr. 2022.

- ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-5.0.

- W. Chen, X. Xiu, H.-J. Jhu, C.-W. Kuo, X. Wang, K. Zhang, L. Zhang, Z. Deng, N. Zhang and Y. Wang “EE2-2.7, 2.8, 2.9: History-parameter-based affine model inheritance and Non-adjacent affine mode”, JVET-Z0139, Apr. 2022.

- V. Seregin, J. Chen, G. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-Z2024, Apr. 2022.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).