| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0115-v1 |
| --- | --- |

| Title: | EE1-1.6-related: ALF with Samples before Deep In-Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junru Li, Kai Zhang, Yue Li, Li Zhang | Tel: Email: | lijunru@bytedance.com zhangkai.video@bytedance.com yue.li@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

# Abstract

This contribution presents a loop filter (ALF) technique on top of the deep learning-based in-loop filtering method in EE1-1.6. ALF is proposed to take luma samples before CNN-based filtering as the additional input where filters with 3x3 and 5x5 diamond shapes are utilized to derive the filtered samples. Compared with VTM11.0_nnvc, the proposed method with 5x5 diamond shapes reportedly shows on average {x%, x%, x%}, { x%, x%, x%}, and {x%, x%, x%} BD-rate reductions for {Y, Cb, Cr} components, under AI, RA, and LDB configurations, respectively. With 3x3 diamond shapes, the proposed method reportedly bring on average {x%, x%, x%}, { x%, x%, x%}, and {x%, x%, x%} BD-rate reductions for {Y, Cb, Cr} components, under AI, RA, and LDB configurations, respectively.

# Introduction

In EE1-1.6 [1], a deep learning-based in-loop filtering method has been proposed to improve the coding efficiency. Combined with the CNN-based filtering, ALF takes the samples after CNN-based filtering as input. For luma component, a filter with 7x7 diamond shape is utilized to generate the filtered sample in ALF. In JVET-Z0146 [2], it is proposed to utilize samples before deblocking filter in ALF.

# Proposed method

It is proposed that samples before CNN-based filtering can be involved into the filtering process of ALF for luma component, while samples after CNN-based filtering are still utilized in ALF where the filtering process is kept same as VTM. Additional filter taps with a diamond shape with dimensions of 5×5 or 3×3 are used to perform the filtering of samples before CNN-based filtering. The coefficient signaling for additional filter taps is remained the same as that in VTM. The 5×5 and 3×3 diamond shapes are shown in Fig.1 and Fig. 2, respectively.

Fig. 1. The proposed 5x5 filter shape for samples before CNN-based filtering.

Fig. 2. The proposed 3x3 filter shape for samples before CNN-based filtering.

# Experimental results

The proposed method is tested according to the common test conditions defined in [3]. VTM-11.0_nnvc [4] and EE1-1.6 [1] under fixed point-based SADL [5] are used for comparison. The 5×5 and 3×3 filter shapes are involved in the experiments, respectively.

The test results with 5x5 diamond shape on top of VTM-11.0_nnvc are shown in Table 1 ~ Table 3. Under AI configurations, the proposed method can bring x%, x%, and x% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under RA configurations, the proposed method can bring x%, x%, and x% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under LDB configurations, the proposed method can bring x%, x%, and x% BD-rate reductions on average for Y, Cb, and Cr, respectively.

Table 1. Performance of the proposed method with 5×5 diamond shape on top of VTM-11.0_nnvc (RA)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Table 2. Performance of the proposed method with 5×5 diamond shape on top of VTM-11.0_nnvc (LDB)

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Table 3. Performance of the proposed method with 5×5 diamond shape on top of VTM-11.0_nnvc (AI)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Compared with the EE1-1.6, experimental results with 5x5 diamond are shown in Table 4 ~ Table 6. Under AI configurations, the proposed method can bring x%, x%, and x% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under RA configurations, the proposed method can bring x%, x%, and x% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under LDB configurations, the proposed method can bring x%, x%, and x% BD-rate reductions on average for Y, Cb, and Cr, respectively.

Table 4. Performance of the proposed method with 5×5 diamond shape on top of EE1-1.6 (RA)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.12% | 0.10% | -0.05% |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.08% | -0.38% | -0.12% |  |  |
| Class F |  |  |  |  |  |

Table 5. Performance of the proposed method with 5×5 diamond shape on top of EE1-1.6 (LDB)

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Table 6. Performance of the proposed method with 5×5 diamond shape on top of EE1-1.6 (AI)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.07% | -0.36% | -0.52% |  |  |
| Class E | -0.10% | 0.05% | 0.05% |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.01% | 0.68% | 0.53% |  |  |
| Class F |  |  |  |  |  |

The test results with 3×3 diamond shape on top of VTM-11.0_nnvc are shown in Table 7 ~ Table 9. Under AI configurations, the proposed method can bring x%, x%, and x% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under RA configurations, the proposed method can bring x%, x%, and x% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under LDB configurations, the proposed method can bring x%, x%, and x% BD-rate reductions on average for Y, Cb, and Cr, respectively.

Table 7. Performance of the proposed method with 3×3 diamond shape on top of VTM-11.0_nnvc (RA)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Table 8. Performance of the proposed method with 3×3 diamond shape on top of VTM-11.0_nnvc (LDB)

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Table 9. Performance of the proposed method with 3×3 diamond shape on top of VTM-11.0_nnvc (AI)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Compared with the EE1-1.6, experimental results with 3×3 diamond are shown in Table 10 ~ Table 12. Under AI configurations, the proposed method can bring x%, x%, and x% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under RA configurations, the proposed method can bring x%, x%, and x% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under LDB configurations, the proposed method can bring x%, x%, and x% BD-rate reductions on average for Y, Cb, and Cr, respectively.

Table 10. Performance of the proposed method with 3×3 diamond shape on top of EE1-1.6 (RA)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Table 11. Performance of the proposed method with 3×3 diamond shape on top of EE1-1.6 (LDB)

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Table 12. Performance of the proposed method with 3×3 diamond shape on top of EE1-1.6 (AI)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

# Conclusions

This contribution presents an improved ALF involving samples before CNN-based filtering. It is recommended to include the proposed method into the common code base of EE1.

# References

- Y. Li, K. Zhang, J. Li, L. Zhang, H. Wang, K. Reuze, A.M. Kotra, M. Karczewicz, Franck Galpin, K. Andersson, J. Ström, D. Liu, R. Sjöberg, EE1-1.6: Deep In-Loop Filter With Fixed Point Implementation, JVET-AA0113, July 2022.

- N. Hu, V. Seregin, M. Karczewicz, AHG12: Using samples before deblocking filter for adaptive loop filter, JVET-Z0146, April 2022.

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-Z2016, by teleconference, April 2022.

- https://vcgit.hhi.fraunhofer.de/jvet-w-ee1/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc

- F. Galpin, T. Dumas, P. Bordes, E. François, AHG11: SADL update, JVET-Z0161, by teleconference, April 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).