| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0121-v1 |
| --- | --- |

| Title: | Non-EE2: Template-based MIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhihuang Xie Yue Yu Haoping Yu Dong Wang Yang Liu Ming Li Junyan Huo Wenhan Qiao Xue Hao Yanzhuo Ma Fuzheng Yang | Tel: Email: | xiezhihuang@oppo.com jyhuo@mail.xidian.edu.cn |
| Source: | OPPO, Xidian University |  |  |

_____________________________

# Abstract

This contribution proposes a template-based MIP method to improve the coding efficiency of ECM. It is proposed to use the top and left templates of the current coding block to derive the MIP mode and the transpose flag. Moreover, DIMD is used to derive a traditional intra-prediction mode based on the MIP prediction samples, then both the LFNST transform set and LFNST transpose flag are determined by the derived traditional intra-prediction mode. Compared with the ECM-5.0 anchor, it is reported that the simulations on the proposed method showed the following coding performance results for {Y, U, V, EncT, DecT}:

AI:

RA:

# Introduction

The matrix weight intra-prediction (MIP) method is an intra-prediction tool in VVC and ECM[1]. MIP takes one left line of the reconstructed neighbouring boundary samples of the block and one above line of the reconstructed neighbouring boundary samples of the block as input to generate the prediction of a block. The generation of the prediction signal is based on the following three steps - averaging, matrix vector multiplication, and linear interpolation.

For each CU in the intra coding mode, a flag indicating whether an MIP mode is to be applied or not is sent. If an MIP mode is applied, the MIP mode and transposed flag are further signalled.

# Proposed method

This contribution proposes to use templates to derive the best MIP mode and transposed flag for the current block according to the SATD costs, which are calculated using the prediction samples and reconstruction sample in the templates, shown in Figure 1. For a block of width W and height H, the left template of the current block is 2 * H, and the above template of the current block is W * 2.

When predicting the samples in the templates, the reconstructed samples adjacent to the templates are used as input, to generate the prediction samples to fill the templates area. If this method is applied for the current block, both encoder and decoder will traverse all the allowed MIP modes with the transposed flag on and off. For each case of MIP mode combined with the transposed flag on or off, their costs are calculated both in the encoder and decoder, and the best two cases are selected to generate the prediction samples of the current block, and those prediction samples are further fused. In the fusion process, the weights of the best case and the second case are calculated by their costs, where the calculation is the same as DIMD and TIMD.

Figure 1. Templates of current block

A modification to LFNST and MIP was proposed in JVET-Z0048[2], which used DIMD on MIP prediction samples to derive a traditional intra prediction mode, then the derived intra prediction mode, instead of PLANAR mode, was used to select the LFNST transform and the transpose flag for MIP coded blocks. In this contribution, the same modification is also applied to the template-based MIP mode as well. In the DIMD derivation process, a horizontal gradient and a vertical gradient are calculated for each prediction sample of the current MIP block to build an HoG, as shown in Figure 2. Then the intra prediction mode with the largest histogram amplitude values is selected as the final output of the DIMD process in this method.

Figure 2. MIP prediction samples to build HoG in JVET-Z0048

# Simulation results

The proposed method was implemented on top of ECM-5.0, and the simulations were performed following the ECM common test condition (CTC)[3].

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.07% | -0.19% | 117% | 102% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.14% | -0.06% | -0.05% | 118% | 100% |
| Class C | -0.18% | -0.10% | 0.02% | 114% | 102% |
| Class E | -0.24% | 0.05% | -0.12% | 117% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.21% | 0.11% | -0.02% | 114% | 100% |
| Class F | -0.19% | -0.01% | -0.01% | 107% | 99% |

# Conclusion

This contribution proposes a template-based MIP method to improve the coding efficiency of ECM, which uses templates to derive the best MIP mode and transposed flag. With the combination of the modification to LFNST and MIP, it is reported that the simulations showed the following coding results, on top of ECM-5.0.

AI:

It is suggested to include this method in the next round of EE2 for further study.

# Reference

[1] ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] J. Huo, W. Qiao, X. Hao, Y. Ma, F. Yang, M. Li, and Y. Liu, “EE2-related: Modification of LFNST for MIP coded block”, JVET-Z0048, April 2022.

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, January 2022.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).