| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0138-v1 |
| --- | --- |

| Title: | EE2-related: On Gradient Linear Model (GLM) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xinwei Li Yan Ye Ru-Ling Liao Jie Chen | Tel: Email: | {sid.lxw, yan.ye, ruling.lrl, jiechen.cj }@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

In this contribution, a new GLM mode is proposed where the chroma sample is predicted based on both the gradient of luma samples and the reconstructed value of the down-sampled luma sample with different parameters. The model parameters are derived by the same adjacent samples as the CCLM modes in ECM-5.0 based on the LDL decomposition method used in EE2 Test 1.1. Multiple variants of the proposed method with different coding gain and complexity trade-offs are tested on top of ECM-5.0. The overall coding performance impact for {Y, U, V, EncT, DecT} are:

Test 1: two GLM modes with 16 gradient patterns

{-0.90%, -0.68%, -0.93%, xxx%, xxx%} for AI and {x.xx%, x.xx%, x.xx%, xxx%, xxx%} for RA;

Test 2: one GLM mode with 16 gradient patterns

{-0.88%, -0.58%, -0.87%, xxx%, xxx%} for AI and {x.xx%, x.xx%, x.xx%, xxx%, xxx%} for RA;

Test 3: two GLM modes with 4 gradient patterns

{x.xx%, x.xx%, x.xx%, xxx%, xxx%} for AI and {x.xx%, x.xx%, x.xx%, xxx%, xxx%} for RA;

Test 4: one GLM mode with 4 gradient patterns

{x.xx%, x.xx%, x.xx%, xxx%, xxx%} for AI and {x.xx%, x.xx%, x.xx%, xxx%, xxx%} for RA;

Introduction

In EE2 Test 1.2 [1], two GLM modes are supported. For the first GLM mode, the gradient of luma samples is used to predict a chroma sample as shown in Equation 1. For the second GLM mode, both the gradient and the reconstructed value of the down-sampled luma sample are used to predict the chroma sample with the same parameter as shown in Equation 2.

predCi,j=α∙Gi,j+β#( LISTNUM  \s 1 )

predCi,j=α∙Gi,j+recL'i,j+β# LISTNUM

where predCi,j represents the predicted value of a chroma sample, Gi,j represents the gradient of the corresponding reconstructed luma samples, recL'i,j represents the reconstructed value of the corresponding down-sampled luma sample, and the linear model parameters α and β are derived based on the same method as CCLM in ECM-5.0 [2], that is derived by one line reconstructed samples that are adjacent to the current block by the linear minimum mean square error (LMMSE) method.

The gradient can be calculated by one of the following 16 gradient patterns:

Figure 1. 16 gradient patterns for GLM

For signaling, when the CCLM_LT, CCLM_L or CCLM_T mode is enabled for the current CU, two flags are signaled separately for Cb and Cr components to indicate whether GLM is enabled for the component; if the GLM is enabled for one component, another flag is further signaled to indicate which of the two GLM modes is selected and one syntax element is further signaled to select one of the 16 gradient patterns in Figure 1 for gradient calculation.

Proposed method

In the second GLM mode in EE2 Test 1.2, the gradient and the down-sample luma value share the same parameter. However, based on general understanding, the gradient is a high-frequency signal, and the down-sample luma value is a low-frequency signal, which should not share the same parameter.

A new GLM mode is proposed in this contribution that a chroma sample is predicted based on both the gradient Gi,j of luma samples and the reconstructed value recL'i,j of the down-sampled luma sample with different parameters:

predCi,j=α0∙Gi,j+α1∙recL'i,j+α2∙midValue# LISTNUM  \s 3

where the model parameters α0, α1 and α2 are derived from the same adjacent samples as the CCLM modes in ECM-5.0 (one line) based on the LDL decomposition method used in EE2 Test 1.1 [3]. For 10-bit content, the midValue is set equal to 512.

In test 1, in order to improve the coding performance of the GLM in EE2 Test 1.2, the second GLM mode in EE2 Test 1.2 where Gi,j and recL'i,j share the same parameter is replaced by the proposed GLM mode where Gi,j and recL'i,j have different parameters. That is, the two GLM modes related to Equation 1 and Equation 3 are supported with 16 gradient patterns.

In order to reduce the encoding time of test1, two simplified methods can be used: reduce the number of GLM modes or reduce the number of gradient patterns. Two other tests are provided by using the two methods, respectively.

In test 2, the number of gradient patterns is reduced from 16 to 4, which are the first 4 gradient patterns in Figure 1. The two GLM modes related to Equation 1 and Equation 3 are supported with 4 gradient patterns.

In test 3, the number of GLM modes is reduced from 2 to 1, where only the proposed GLM mode related to Equation 3 is supported. That is, only one GLM mode related to Equation 3 is supported with 16 gradient patterns.

In order to further reduced the encoding time, both two simplified methods are used in test 4, that is, only the proposed GLM mode related to Equation 3 is supported with first 4 gradient patterns in Figure 1.

The other designs are kept unchanged with EE2 Test 1.2.

Simulation results

The proposed methods are implemented on top of ECM-5.0. The simulation results are tested under ECM test condition [3] with all intra and random access configurations. Four different variants listed in the following are tested:

Test 1: The first GLM mode in EE2 Test1.2 and the proposed GLM mode with 16 gradient patterns;

Test 2: The proposed GLM mode with 16 gradient patterns;

Test 3: The first GLM mode in EE2 Test1.2 and the proposed GLM mode with 4 gradient patterns;

Test 4: The proposed GLM mode with 4 gradient patterns.

The results are summarized in Table 1 and the details result of each test are shown in Table 2 to Table 5, respectively.

Table 1. Summary of the results for different variants

|  | All Intra Main10 |  |  |  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Test 1 | -0.90% | -0.68% | -0.93% |  |  |  |  |  |  |  |
| Test 2 | -0.88% | -0.59% | -0.88% |  |  |  |  |  |  |  |
| Test 3 |  |  |  |  |  |  |  |  |  |  |
| Test 4 |  |  |  |  |  |  |  |  |  |  |

Table 2. Simulation results of test 1

|  |  |  |  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over ECM-5.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  |  | DecT |  |  |
| Class A1 | -0.22% |  | 0.26% |  |  | -0.36% |  |  |  |  |  |  |  |
| Class A2 | -4.92% |  | -2.25% |  |  | -5.47% |  |  |  |  |  |  |  |
| Class B | -0.14% |  | -0.83% |  |  | -0.03% |  |  |  |  |  |  |  |
| Class C | 0.00% |  | -0.11% |  |  | 0.07% |  |  |  |  |  |  |  |
| Class E | -0.05% |  | -0.56% |  |  | 0.19% |  |  |  |  |  |  |  |
| Overall | -0.90% |  | -0.68% |  |  | -0.93% |  |  |  |  |  |  |  |
| Class D | 0.02% |  | 0.66% |  |  | 0.13% |  |  |  |  |  |  |  |
| Class F | -0.75% |  | -4.28% |  |  | -4.27% |  |  |  |  |  |  |  |
|  |  |  |  | Random access Main10 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Over ECM-5.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  |  | DecT |  |  |
| Class A1 | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class A2 | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class B | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class C | 0.03% |  | 0.10% |  |  | 0.08% |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class D | 0.05% |  | 0.35% |  |  | 0.57% |  |  |  |  |  |  |  |
| Class F | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |

Table 3. Simulation results of test 2

|  |  |  |  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over ECM-5.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  |  | DecT |  |  |
| Class A1 | -0.22% |  | 0.43% |  |  | -0.40% |  |  |  |  |  |  |  |
| Class A2 | -4.86% |  | -2.05% |  |  | -5.10% |  |  |  |  |  |  |  |
| Class B | -0.13% |  | -0.78% |  |  | 0.15% |  |  |  |  |  |  |  |
| Class C | 0.01% |  | -0.10% |  |  | 0.00% |  |  |  |  |  |  |  |
| Class E | -0.03% |  | -0.40% |  |  | 0.02% |  |  |  |  |  |  |  |
| Overall | -0.88% |  | -0.58% |  |  | -0.87% |  |  |  |  |  |  |  |
| Class D | 0.04% |  | 0.38% |  |  | 0.22% |  |  |  |  |  |  |  |
| Class F | -0.81% |  | -3.84% |  |  | -3.75% |  |  |  |  |  |  |  |
|  |  |  |  | Random access Main10 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Over ECM-5.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  |  | DecT |  |  |
| Class A1 | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class A2 | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class B | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class C | 0.03% |  | 0.10% |  |  | 0.08% |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class D | 0.05% |  | 0.35% |  |  | 0.57% |  |  |  |  |  |  |  |
| Class F | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |

Table 4. Simulation results of test 3

|  |  |  |  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over ECM-5.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  |  | DecT |  |  |
| Class A1 | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class A2 | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class B | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class C | 0.02% |  | -0.13% |  |  | -0.06% |  |  |  |  |  |  |  |
| Class E | -0.06% |  | -0.56% |  |  | 0.11% |  |  |  |  |  |  |  |
| Overall | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class D | 0.02% |  | 0.64% |  |  | -0.03% |  |  |  |  |  |  |  |
| Class F | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
|  |  |  |  | Random access Main10 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Over ECM-5.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  |  | DecT |  |  |
| Class A1 | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class A2 | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class B | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class C | 0.01% |  | 0.26% |  |  | 0.18% |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class D | 0.03% |  | 0.22% |  |  | 0.36% |  |  |  |  |  |  |  |
| Class F | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |

Table 5. Simulation results of test 4

|  |  |  |  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over ECM-5.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  |  | DecT |  |  |
| Class A1 | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class A2 | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class B | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class C | 0.00% |  | -0.18% |  |  | -0.08% |  |  |  |  |  |  |  |
| Class E | -0.04% |  | -0.42% |  |  | 0.05% |  |  |  |  |  |  |  |
| Overall | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class D | 0.04% |  | 0.04% |  |  | 0.07% |  |  |  |  |  |  |  |
| Class F | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
|  |  |  |  | Random access Main10 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Over ECM-5.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  |  | DecT |  |  |
| Class A1 | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class A2 | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class B | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class C | 0.02% |  | 0.04% |  |  | 0.06% |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |
| Class D | 0.03% |  | -0.17% |  |  | 0.11% |  |  |  |  |  |  |  |
| Class F | #VALUE! |  | #VALUE! |  |  | #VALUE! |  |  |  |  |  |  |  |

References

- C.-W. Kuo, et. al, “EE2-1.1b and 1.2: Filter-based linear model and gradient linear model”, JVET-AA0125, Jul. 2022

- ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- P. Astola, et. al, “EE2-1.1a: Convolutional cross-component intra prediction model”, JVET-AA0057, Jul. 2022.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Feb. 2022.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).