| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0148-v1 |
| --- | --- |

| Title: | Non-EE2: On MHP (Multi-Hypothesis Prediction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato Yue Yu Haoping Yu Dong Wang | Email: | kazushi.sato@oppo,com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution proposes to extend add_hyp_weight_idx for multi-hypothesis prediction currently in ECM. In particular, two more index values are added for add_hyp_weight_idx to signal additional α values equal to 1/8 and -1/4, respectively.

It is reported that the simulations showed the following results -- Y: xx %, U: xx %, V: xx% with EncT: xxx % and DecT: xxx % for RA CTC, and Y: xx %, U: xx %, V: xx% with EncT: xxx % and DecT: xxx % for LD CTC.

The gain is higher with LD, but with RA no significant loss is observed. With “Slide Editing” and “SlideShow” in LD CTC, gain by -0.73% and -1.04% are observed, respectively. Both in RA and LD no significant increase in EncT/DecT can be observed.

It is recommended that the proposed method be studied under the next round of EE.

# Introduction / Proposed Method

Multi-hypothesis prediction (MHP) originally proposed in [3] is included in the current ECM. The weighting factor α is specified by the syntax element add_hyp_weight_idx as shown in Table 1.1.

Table 1.1: add_hyp_weight_idx in the current ECM

| add_hyp_weight_idx | α |
| --- | --- |
| 0 | 1/4 |
| 1 | -1/8 |

This contribution proposes to extend Table 1.1 and add two more entries in the list. The proposed new indices for add_hyp_weight_idx and their corresponding values for weighting factor α are shown in Table 1.2.

Table 1.2: Proposed add_hyp_weight_idx

| add_hyp_weight_idx | α |
| --- | --- |
| 0 | 1/4 |
| 1 | 1/8 |
| 2 | -1/8 |
| 3 | -1/4 |

# Simulation

## Simulation Condition

The proposed method is tested with RA and LD conditions specified in [2].

ECM5.0 is the anchor. On top of the anchor software two simple modifications are implemented as follows:

In “CommonDef.h”

static const auto MULTI_HYP_PRED_NUM_WEIGHTS = 4;// 2;

In “Rom.cpp”

const int g_addHypWeight[MULTI_HYP_PRED_NUM_WEIGHTS] = { 2, 1, -1, -2 }; // { 2, -1 };

## Simulation Result

A summary of the results with RA and LD CTC is shown in Tables 2.2.1 and 2.2.2, respectively. The complete results are included in JVET-AA0148.xlsm.

As can be seen, the gain is higher with LD, but with RA no significant loss is observed. With “Slide Editing” and “SlideShow” in LD CTC, gain by -0.73% and -1.04% are observed, respectively. Both in RA and LD no significant increase in EncT/DecT can be observed.

Table 2.2.1: Summary of the Result with RA CTC

Table 2.2.2: Summary of the Result with LD CTC

# Conclusion

This contribution proposes to extend add_hyp_weight_idx for multi-hypothesis prediction currently in ECM. In particular, two more index values are added for add_hyp_weight_idx to signal additional α values equal to 1/8 and -1/4, respectively.

It is reported that the simulations showed the following results -- Y: xx %, U: xx %, V: xx% with EncT: xxx % and DecT: xxx % for RA CTC, and Y: xx %, U: xx %, V: xx% with EncT: xxx % and DecT: xxx % for LD CTC.

The gain is higher with LD, but with RA no significant loss is observed. With “Slide Editing” and “SlideShow” in LD CTC, gain by -0.73% and -1.04% are observed. Both in RA and LD no significant increase in EncT/DecT can be observed.

It is recommended that the proposed method be studied under the next round of EE.

# References

[1] “Algorithm description of Enhanced Compression Model 5 (ECM 5)”, JVET-Z2025, Apr 2022

[2] “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan 2022

[3] “CE10: Multi-hypothesis inter prediction (Test 10.1.2)”, JVET-M0425, Jan 2019

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).