| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0153-v1 |
| --- | --- |

| Title: | EE2-1.6b: Convolutional cross-component intra prediction model + weighted chroma prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pekka Astola Jani Lainema | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports results of EE2 Test 1.6b that is a combination test of EE2-1.1a convolutional cross component intra prediction model [1] and EE2-1.6 weighted chroma prediction [2]. The impact on coding efficiency and runtimes over ECM-5.0 is reportedly for {Y, U, V, EncT, DecT}:

AI { -1.42%, -3.11%, -3.01%, 104%, 106%}, RA { -0.80%, -2.14%, -2.60%, 103%, 100% }.

# Experimental results

Tables below summarize the coding efficiency and runtime impact of the proposed method in ECM 5.0 using the JVET common test conditions.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.26 % | -3.18 % | -5.11 % | 102 % | 107 % |
| Class A2 | -5.61 % | -4.53 % | -7.27 % | 104 % | 104 % |
| Class B | -0.61 % | -3.56 % | -1.85 % | 103 % | 105 % |
| Class C | -0.19 % | -1.16 % | -0.97 % | 105 % | 106 % |
| Class E | -0.37 % | -3.50 % | -1.31 % | 103 % | 106 % |
| Overall | -1.42 % | -3.11 % | -3.01 % | 104 % | 106 % |
| Class D | -0.13 % | -0.95 % | -0.82 % | 106 % | 107 % |
| Class F | -0.95 % | -3.51 % | -2.87 % | 103 % | 107 % |
| Class TGM | 0.00 % | 0.00 % | 0.00 % | 100 % | 100 % |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.16 % | -2.09 % | -4.22 % | 103 % | 100 % |
| Class A2 | -2.11 % | -2.22 % | -3.96 % | 103 % | 100 % |
| Class B | -0.37 % | -3.42 % | -2.36 % | 102 % | 99 % |
| Class C | -0.08 % | -0.51 % | -0.65 % | 103 % | 101 % |
| Class E |  |  |  |  |  |
| Overall | -0.80 % | -2.14 % | -2.60 % | 103 % | 100 % |
| Class D | -0.08 % | -0.94 % | -0.63 % | 103 % | 99 % |
| Class F | -1.25 % | -3.20 % | -3.20 % | 102 % | 99 % |
| Class TGM | 0.00 % | 0.00 % | 0.00 % | 100 % | 100 % |

# References

[1] P. Astola et al., “EE2-1.1a: Convolutional cross-component intra prediction model”, JVET-AA0057, July 2022.

[2] J.-Y. Huo et al., “EE2-1.6: Weighted chroma prediction”, JVET-AA0078, July 2022.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).