| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0159-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AA0058 (EE2-2.7: GPM adaptive blending (JVET-Z0059, JVET-Z0137) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Zhipin Deng | Tel: Email: | zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results of JVET-AA0058. The simulation results reportedly match those provided by the proponents.

# Verification

JVET-AA0058 [1] reports the results of EE2-2.7a and EE2-2.7b on geometric partitioning mode (GPM) adaptive blending. Specifically, besides the existing blending area (i.e., 2 samples on each side of the GPM partition split boundary), four extra blending area sizes, i.e., quarter, half, double, and quadrupole of the existing area size, are added for the GPM mode. The selected blending area size is signalled at CU-level from encoder to decoder. Additionally, block-size dependent signaling will be tested to reduce the signaling overhead and encoder runtime.

We verified that

- The source code implements match with what was described in the proposal.

- Simulation results match with those provided in the proposal.

The following two tests were arranged for the crosscheck.

- EE2-2.7a.: GPM adaptive blending

- EE2-2.7b: Test 2.7a + Block-size dependent signalling

The coding results of EE2-2.7a and EE2-2.7b under the ECM CTC are summarized in Table 1-2.

Table 1: Performance of EE2-2.7a over ECM-5.0 (CTC)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -0.41% | -0.39% | 101% | 96% |
| Class A2 | -0.07% | 0.06% | -0.03% | 100% | 94% |
| Class B | -0.20% | -0.44% | -0.41% | 102% | 104% |
| Class C | -0.42% | -0.27% | -0.42% | 101% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0.25% | -0.29% | -0.33% | 101% | 101% |
| Class D | -0.32% | -0.33% | -0.01% | 102% | 97% |
| Class F | -0.81% | -0.66% | -0.69% | 101% | 103% |
| Class TGM | -2.51% | -1.96% | -1.87% | 93% | 91% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.22% | -0.40% | -0.24% | 101% | 94% |
| Class C | -0.49% | -0.49% | -0.49% | 98% | 99% |
| Class E | -0.28% | -0.23% | -0.52% | 105% | 109% |
| Overall | -0.33% | -0.39% | -0.39% | 101% | 99% |
| Class D | -0.45% | 0.64% | 0.63% | 103% | 101% |
| Class F | -1.08% | -0.49% | -0.85% | 102% | 102% |
| Class TGM | -3.83% | -3.19% | -3.12% | 93% | 86% |

Table 2: Performance of EE2-2.7b over ECM-5.0 (CTC)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.22% | -0.32% | 100% | 96% |
| Class A2 | -0.08% | 0.04% | 0.06% | 98% | 94% |
| Class B | -0.13% | -0.24% | -0.21% | 101% | 104% |
| Class C | -0.28% | 0.00% | -0.12% | 99% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.17% | -0.12% | -0.16% | 100% | 100% |
| Class D | -0.28% | -0.22% | -0.18% | 100% | 96% |
| Class F | -0.69% | -0.50% | -0.47% | 101% | 102% |
| Class TGM | -2.40% | -1.82% | -1.73% | 92% | 90% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.14% | -0.45% | -0.33% | 102% | 95% |
| Class C | -0.33% | -0.59% | -0.34% | 100% | 101% |
| Class E | -0.24% | -0.66% | -0.14% | 102% | 98% |
| Overall | -0.23% | -0.55% | -0.29% | 101% | 98% |
| Class D | -0.47% | 0.09% | -0.23% | 101% | 99% |
| Class F | -0.91% | -0.62% | -0.42% | 101% | 102% |
| Class TGM | -3.58% | -3.00% | -2.95% | 92% | 88% |

The coding results of EE2-2.7a and EE2-2.7b by enabling the PLT for SCC sequences for both anchor and tests are summarized in Table 3-4.

Table 3: Performance of EE2-2.7a over ECM-5.0 with PLT ON

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 + PLT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.78% | -0.43% | -0.50% | 102% | 99% |
| Class TGM | -2.53% | -1.92% | -1.89% | 100% | 96% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-5.0 + PLT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.54% | -0.51% | -1.37% | 102% | 96% |
| Class TGM | -3.73% | -3.11% | -3.01% | 99% | 95% |

Table 4: Performance of EE2-2.7b over ECM-5.0 with PLT ON

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 + PLT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.65% | -0.43% | -0.49% | 101% | 99% |
| Class TGM | -2.53% | -1.94% | -1.92% | 99% | 96% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-5.0 + PLT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.52% | -0.71% | -0.78% | 101% | 97% |
| Class TGM | -3.55% | -2.85% | -2.86% | 99% | 96% |

The BD rate differences match with the results from the proposal [1]. Note that timing results are not accurate in this report, since the crosscheck tests and anchor test were launched in different time slots with different simulation environments.

# References

Y. Kidani, H. Kato, K. Unno, K. Kawamura (KDDI), Ning Yan, Xiaoyu Xiu, Wei Chen, Hong-Jheng Jhu, Che-Wei Kuo, Xianglin Wang (Kwai), “EE2-2.7: GPM adaptive blending (JVET-Z0059, JVET-Z0137)”, JVET-AA0058, July 2022.