| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0161-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AA0127 (EE2-1 related: Encoder optimization of EE2-1.3a) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Zhipin Deng | Tel: Email: | zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results of JVET-AA0127 EE2-1.3a with encoder optimization. The simulation results reportedly match those provided by the proponents.

# Verification

JVET-AA0127 [1] reports the results of EE2-1.3a with encoder optimization. In this test, two encoder optimizations are included: a software clean-up such as removing redundant operations in the gradient calculation process of the GLM at encoder, and an early R-D termination such as bypassing the SATD checking of the combination of the GLM with multiple directional linear model (MDLM) if the stand-alone SATD cost of the GLM is larger than that of CCLM.

We verified that

- The source code implements match with what was described in the proposal.

- Simulation results match with those provided in the proposal.

The following test was arranged for the crosscheck.

- EE2-1.3a with encoder optimization: The combination of CCCM and GLM with encoder optimization

The coding results of EE2-1.3a with encoder optimization under ECM CTC are summarized in Table 1.

Table 1: Performance of EE2-1.3a with encoder optimization over ECM-5.0 (CTC)

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.21% | -2.65% | -4.85% | 101% | 99% |
| Class A2 | -5.59% | -4.49% | -7.40% | 105% | 98% |
| Class B | -0.60% | -3.48% | -1.75% | 102% | 102% |
| Class C | -0.12% | -0.80% | -0.69% | 102% | 97% |
| Class E | -0.35% | -3.35% | -1.25% | 104% | 99% |
| Overall | -1.39% | -2.89% | -2.89% | 103% | 99% |
| Class D | -0.09% | -0.09% | -0.44% | 106% | 98% |
| Class F | -1.02% | -4.21% | -3.73% | 103% | 101% |
| Class TGM | -4.11% | -10.02% | -9.71% | 103% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.10% | -1.98% | -3.90% | 101% | 100% |
| Class A2 | -2.07% | -2.37% | -4.19% | 99% | 98% |
| Class B | -0.34% | -3.55% | -2.43% | 101% | 102% |
| Class C | -0.07% | -0.36% | -0.32% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.77% | -2.15% | -2.51% | 101% | 100% |
| Class D | -0.04% | -0.78% | -0.28% | 101% | 99% |
| Class F | -1.64% | -3.65% | -3.90% | 102% | 100% |
| Class TGM | -2.60% | -4.91% | -4.78% | 101% | 99% |

The BD rate differences match with the results from the proposal [1]. Note that timing results are not accurate in this report, since the crosscheck tests and anchor test were launched in different time slots with different simulation environments.

# References

C.-W. Kuo, H.-J. Jhu, X. Xiu, N. Yan, W. Chen, X. Wang, “EE2-1 related: Encoder optimization of EE2-1.2 and 1.3”, JVET-AA0127, July 2022.