| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0181-v1 |
| --- | --- |

| Title: | Cross-check of JVET-AA0111 (EE1-1.6: Deep In-Loop Filter With Fixed Point Implementation) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Kai Lin Chuanmin Jia Shanshe Wang | Email: | kailin@pku.edu.cn cmjia@pku.edu.cn sswang@pku.edu.cn |
| Source: | Peking University |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of EE1-1.6 described in JVET-Z2023 [1], which evaluates the effectiveness of the deep in-loop filter presented in JVET-Z0113 [2]. Compared with the simulation results in JVET-Z0113, the retrained models bring a BD-rate difference of -0.10%, -0.71%, -0.96% under AI configuration, and 0.40%, -0.17%, -0.73% under RA configuration. The simulation results reveal that the training process of JVET-Z0113 is reproducible. As for EE1.6.2, the BD-rate performance exactly matches the results provided by the proponents under the fixed-point calculation. While the results with the float-point calculation is very close to the proponent’s results.

Introduction

The EE test 1.6.1 is to study the training process and coding performance of the deep learning based in-loop filtering in JVET-Y-EE1.7 (8 res blocks, float32, Libtorch). The required training datasets for intra slice and inter slice were respectively generated under the instruction of the proponents. The models were retrained from scratch according to the training scripts. Specifically, the training process can be divided into three stages.

- At the first stage, we used DIV2K [5] to generate the training samples for intra slice filtering.

- Secondly, we integrated the intra filtering models and produced the training dataset for inter slice filtering by BVI-DVC [6].

- Thirdly, we integrated the inter models into the codec and repeated stage2. The deep learning based in-loop filtering for both intra slice and inter slice was enabled during the dataset generation.

After the training process of three stages, we selected the intra models of stage1 and inter models of stage3 to evaluate the coding performance. According to the arrangement of EE test 1.6.1, the test item is listed as following.

Test 1: an 8-residual block version implemented with Libtorch

This EE test 1.6.2 is to study the coding performance of JVET-Z0113 implemented with SADL library [3]. Both floating-point (float32) based and fixed-point (int16) based calculation during the inference were evaluated.  The crosscheck was conducted by cloning the EE1-1.6 branch (SHA: a7940b9e) from VTM repository. There are 2 test items in all as following,

Test 2: the fixed-point (int16) based calculation implemented with SADL library.

Test 3: the float-point (float32) based calculation implemented with SADL library.

Results

The source code was compiled using gcc 8.3.0 and tested according to the NN-CTC[4] on RedHat 7.3, CPU only. The results were compared with VTM11.0 + new MCTF according to the NN-CTC[4] for RA and AI, the detailed results could be seen in the attached excel files.

Test 1

|  |  |  |  |  |  |  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Over VTM-11.0_nnvc |  |  |  |  |  |  |
|  | Y |  | U |  |  |  |  |  |  | V | EncT | DecT |
| Class A1 | -9.48% |  |  | -17.90% |  |  |  |  | -20.07% |  | 260% | 55202% |
| Class A2 | -10.57% |  |  | -21.57% |  |  |  |  | -22.09% |  | 249% | 51106% |
| Class B | -9.37% |  |  | -24.81% |  |  |  |  | -23.95% |  | 262% | 55119% |
| Class C | -9.94% |  |  | -23.11% |  |  |  |  | -24.11% |  | 256% | 54403% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -9.78% |  |  | -22.33% |  |  |  |  | -22.84% |  | 257% | 54119% |
| Class D | -11.49% |  |  | -24.87% |  |  |  |  | -26.04% |  | 230% | 46332% |
| Class F | -4.52% |  |  | -13.41% |  |  |  |  | -13.45% |  | 335% | 25296% |
|  |  |  |  |  |  |  | All Intra Main10 |  |  |  |  |  |
|  |  |  |  |  |  |  | Over VTM-11.0_nnvc |  |  |  |  |  |
|  | Y |  |  |  | U |  |  | V |  |  | EncT | DecT |
| Class A1 | -6.56% |  |  |  | -18.49% |  |  | -18.95% |  |  | 251% | 40175% |
| Class A2 | -6.62% |  |  |  | -20.73% |  |  | -18.91% |  |  | 212% | 31788% |
| Class B | -6.57% |  |  |  | -21.25% |  |  | -21.79% |  |  | 199% | 28888% |
| Class C | -7.52% |  |  |  | -19.91% |  |  | -22.76% |  |  | 184% | 20128% |
| Class E | -9.90% |  |  |  | -21.15% |  |  | -21.37% |  |  | 202% | 33509% |
| Overall | -7.34% |  |  |  | -20.39% |  |  | -20.98% |  |  | 206% | 29335% |
| Class D | -7.45% |  |  |  | -20.13% |  |  | -22.11% |  |  | 175% | 16778% |
| Class F | -3.99% |  |  |  | -13.30% |  |  | -12.92% |  |  | 186% | 20846% |

Test 2

|  |  |  |  |  | Random access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Over VTM-11.0_nnvc |  |  |  |  |  |  |
|  | Y |  | U |  |  |  | V | EncT |  | DecT |  |
| Class A1 | -9.82% |  | -17.26% |  |  |  | -19.62% | 328% |  | 91562% |  |
| Class A2 | -10.98% |  | -21.45% |  |  |  | -18.38% | 314% |  | 88348% |  |
| Class B | -9.82% |  | -24.61% |  |  |  | -23.24% | 323% |  | 91893% |  |
| Class C | -10.38% |  | -23.14% |  |  |  | -23.86% | 270% |  | 82805% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -10.20% |  | -22.11% |  |  |  | -21.71% | 307% |  | 88612% |  |
| Class D | -11.88% |  | -24.79% |  |  |  | -24.96% | 268% |  | 78215% |  |
| Class F | -5.15% |  | -12.75% |  |  |  | -11.91% | 438% |  | 42300% |  |
|  |  |  |  |  |  | All Intra Main10 |  |  |  |  |  |  |
|  |  |  |  |  |  | Over VTM-11.0_nnvc |  |  |  |  |  |  |
|  | Y |  |  | U |  |  | V |  | EncT |  | DecT |  |
| Class A1 | -6.45% |  |  | -17.96% |  |  | -19.88% |  | 304% |  | 73316% |  |
| Class A2 | -6.50% |  |  | -20.46% |  |  | -17.82% |  | 236% |  | 59202% |  |
| Class B | -6.52% |  |  | -21.87% |  |  | -21.59% |  | 223% |  | 57286% |  |
| Class C | -7.40% |  |  | -18.86% |  |  | -21.76% |  | 190% |  | 39446% |  |
| Class E | -9.87% |  |  | -20.85% |  |  | -20.66% |  | 231% |  | 64682% |  |
| Overall | -7.26% |  |  | -20.14% |  |  | -20.56% |  | 230% |  | 56372% |  |
| Class D | -7.32% |  |  | -19.50% |  |  | -21.39% |  | 262% |  | 48766% |  |
| Class F | -4.06% |  |  | -11.76% |  |  | -10.93% |  | 194% |  | 38845% |  |

Test 3

|  |  |  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over VTM-11.0_nnvc |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -9.81% |  | -24.43% |  | -23.28% | 489% | 179685% |
| Class C | -10.38% |  | -23.05% |  | -23.97% | 390% | 160877% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -11.88% |  | -24.68% |  | -24.87% | 436% | 178023% |
| Class F | -5.10% |  | -12.68% |  | -11.85% | 733% | 84019% |

|  |  |  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over VTM-11.0_nnvc |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -6.49% |  | -21.90% |  | -21.61% | 271% | 105870% |
| Class C | -7.42% |  | -18.84% |  | -21.72% | 221% | 74239% |
| Class E | -9.87% |  | -20.81% |  | -20.61% | 298% | 124871% |
| Overall |  |  |  |  |  |  |  |
| Class D | -7.35% |  | -19.44% |  | -21.34% | 202% | 61142% |
| Class F |  |  |  |  |  |  |  |

References

[1] Exploration Experiment on Neural Network-based Video Coding (EE1), JVET-Z2023, E. Alshina, W. Chen, F. Galpin, Y. Li, Z. Ma, H. Wang, L. Wang.

[2] Y. Li, K. Zhang, L. Zhang, H. Wang, M. Coban, A.M. Kotra, M. Karczewicz, F. Galpin, K. Andersson, J. Ström, D. Liu, R. Sjöberg, EE1-1.7: Combined Test of EE1-1.6 and EE1-1.3, JVET-Z0113, by teleconference, April 2022

[3] F. Galpin, T. Dumas, P. Bordes, E. François, AHG11: SADL update, JVET-Z0161, by teleconference, April 2022.

[4] E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-Z2016, by teleconference, April 2022.

[5] https://data.vision.ee.ethz.ch/cvl/DIV2K/ R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

[6] Ma, Di, Fan Zhang, and David R. Bull. "BVI-DVC: A Training Database for Deep Video Compression." arXiv preprint arXiv:2003.13552 (2020).