| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0185-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AA0122 (EE1-1.3: On BaseQP adjustment in CNNLF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liqiang Wang | Email: | liqiangwang@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results for EE1-1.3 proposed in JVET-AA0122 [], which proposes a QP-adjustment method to improve the performance of the neural network based in-loop filter. The QP offset is signaled at the slice-level.

The source code provided by the proponent has been inspected and considered to be aligned with what is described in in the document.

The cross-check is conducted under LD and RA configurations. In terms of bitrate and PSNR, the test results exactly match for LD configuration, while mentioning some cases in RA may have a minor difference in PSNR probably caused by the rounding difference.

Introduction

The following two tests are included into this crosscheck.

Test 1.3.1: Test JVET-Z0128 as proposed and compare with original filter design (refer to JVET-Y-EE1-1.1)

Test 1.3.2: Tests the same for JVET-Y-EE1-1.2

The EE1-1.3 code based on Libtorch-1.9.0 is obtained from the following URL:

https://vcgit.hhi.fraunhofer.de/jvet-z-ee1/VVCSoftware_VTM/-/commits/EE1-1.3.

In addition, the following bugfix (line 253, EncCnnLoopFilter.cpp) is integrated.

Before:

for (int i = 1; i < MAX_NUM_CNN; i++)

After:

for (int i = 0; i < MAX_NUM_CNN; i++)

Crosscheck of EE1-1.3.1

The purpose of this test is to verify the performance of the proposed in the neural network-based filter with two models []. The macro configuration is set as follows:

Crosscheck of EE1-1.3.2

The purpose of this test is to verify the performance of the proposed in the neural network-based filter with one model []. The macro configuration is set as follows:

Results

All the two test results are provided as follows, which are built on the EE1 anchor [][]. The comparison basis is set to the test 1.1.1 in [2] and test 1.2.1 in [3] for EE1-1.3.1 and EE1-1.3.2, respectively. Noted that the running time is not so accurate.

Results of EE1-1.3.1

Table 1. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over JVET-Z0094 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.20% | 0.28% | -0.42% | 107% | 103% |
| Class A2 | -0.40% | -0.12% | 0.18% | 106% | 102% |
| Class B | -0.28% | -0.26% | -0.18% | 107% | 104% |
| Class C | -0.36% | -0.31% | -0.11% | 105% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.31% | -0.14% | -0.14% | 106% | 103% |
| Class D | -0.20% | -0.35% | 0.05% | 105% | 103% |
| Class F | -0.06% | 0.14% | 0.17% | 107% | 107% |

Table 2. Performance of the proposed method (LB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over JVET-Z0094 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.39% | -0.13% | -0.30% | 106% | 106% |
| Class C | -0.51% | -0.71% | -0.09% | 104% | 104% |
| Class E | -0.38% | 1.39% | 0.20% | 109% | 127% |
| Overall | -0.43% | -0.38% | -0.21% | 106% | 110% |
| Class D | -0.42% | -0.35% | 0.98% | 105% | 104% |
| Class F | 0.01% | -0.03% | 0.31% | 107% | 110% |

Results of EE1-1.3.2

Table 3. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over JVET-Z0091 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.19% | -0.33% | -0.43% | 107% | 102% |
| Class A2 | -0.36% | -0.41% | -0.25% | 106% | 101% |
| Class B | -0.27% | -0.61% | -0.34% | 107% | 103% |
| Class C | -0.44% | -0.93% | -0.68% | 105% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.32% | -0.60% | -0.43% | 106% | 103% |
| Class D | -0.39% | -0.72% | -0.38% | 106% | 105% |
| Class F | -0.12% | -0.12% | 0.05% | 108% | 105% |

Table 4. Performance of the proposed method (LB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over JVET-Z0091 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | -0.50% | -0.05% | -0.77% | 106% | 102% |
| Class B | -0.37% | 0.13% | 0.65% | 107% | 105% |
| Class C | -0.63% | -0.49% | 0.43% | 105% | 105% |
| Class E | -0.31% | 1.72% | 0.51% | 110% | 129% |
| Overall | -0.44% | -0.15% | 0.55% | 107% | 111% |
| Class D | -0.59% | -0.24% | -0.24% | 106% | 103% |
| Class F | -0.19% | -0.31% | 0.59% | 108% | 106% |

References

[] Z. Xie, Y. Yu, H. Yu, D. Wang. EE1-1.3: On BaseQP adjustment in CNNLF. JVET-AA0122, 27th Meeting, by teleconference, 13–22 July 2022.

[] Liqiang Wang, et al., “EE1-1.1: neural network based in-loop filter with 2 models”, JVET-Z0094, April. 2022.

[] Liqiang Wang, et al., “EE1-1.2: neural network based in-loop filter with a single model”, JVET-Z0091, April. 2022.

[] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc.

[] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-Z2016, 26th Meeting, by teleconference, 20–29 April 2022.