| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0204-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AA0043 (IntraTMP Adaptation for Camera Captured Contents) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Damian Ruiz Coll, Vikas Warudkar | Tel: Email: | duizcoll@ofinno.com, vwarudkar@ofinno.com, |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for JVET-AA0043, which proposes enabling IntraTMP for all CTC classes, not only for the classes F and TGM as required in the CTC. Specifically, it is proposed to process the template matching by subsampling the search range followed by iterative refinement to achieve a good trade-off between encoding time and efficiency.

# Introduction

In the current common test conditions, IntraTMP is only enabled for screen contents (class F and class TGM). For camera-captured contents, the IntraTMP computed at each position in the four search regions achieves a good coding gain but at the expense of a high encoding and decoding computing time.

Figure 1. Four search regions for IntraTMP in ECM software

To reduce the IntraTMP complexity, JVET-AA0043 [1] proposes the following method:

- Search range subsampling: subsample each of the search areas by a sampling factor “s=1<<i” (“s” is a variable of power 2). For example, if “s” equals two, only 1/4th of the template matching search is performed. The “s” is signaled to the decoder by signaling the value “i” value in SPS.

- Iterative refinement: after finding the best matching within the subsampled search area, the search is iteratively refined around the best match. The refinement is repeated with the sampling factor divided by 2, until the full pixel search.

For screen contents coding, the JVET-AA0043 proposes to keep unchanged the “s” value (s=1), where a full pixel search is performed without iterative refinement.

Cross-check of the source code and simulation results has been conducted. Inspected source code follows the description in the proposal. The simulation is performed on top of ECM-5.0. The cross-check results are provided in the tables below and match the results provided by the proponent. Note that coding time may not be accurate.

Below is the result for Test-1 and Test2 using a subsampling factor of s=4 and s=8, respectively:

Test-1 (s=4)

Table I. Proposed method (s=4) on top of ECM-5.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.01% | -0.07% | 104% | 101% |
| Class A2 | -0.29% | -0.50% | -0.46% | 103% | 104% |
| Class B | -0.29% | -0.38% | -0.33% | 101% | 100% |
| Class C | -0.09% | -0.11% | -0.02% | 105% | 106% |
| Class E | -0.44% | -0.50% | -0.45% | 108% | 109% |
| Overall | -0.23% | -0.30% | -0.26% | 104% | 104% |
| Class D | -0.02% | -0.03% | -0.10% | 102% | 105% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |

| Class A1 | 0.00% | 0.03% | -0.03% | 106% | 104% |
| --- | --- | --- | --- | --- | --- |
| Class A2 | -0.16% | -0.08% | -0.15% | 103% | 103% |
| Class B | -0.15% | -0.32% | -0.14% | 101% | 108% |
| Class C | -0.03% | 0.04% | -0.11% | 109% | 111% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.11% | -0.11% | 105% | 107% |
| Class D | 0.03% | -0.08% | 0.41% | 111% | 118% |
| Class F | 0.00% | 0.03% | -0.03% | 106% | 104% |
| Class TGM | -0.16% | -0.08% | -0.15% | 103% | 103% |

Test-1 (s=8)

Table II. The proposed method (s=8) on top of ECM-5.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.00% | 0.04% | 107% | 103% |
| Class A2 | -0.26% | -0.36% | -0.43% | 104% | 102% |
| Class B | -0.26% | -0.27% | -0.36% | 100% | 98% |
| Class C | -0.08% | -0.05% | -0.02% | 101% | 102% |
| Class E | -0.42% | -0.46% | -0.52% | 106% | 109% |
| Overall | -0.21% | -0.22% | -0.26% | 103% | 102% |
| Class D | -0.01% | -0.03% | -0.11% | 99% | 103% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.11% | -0.27% | -0.22% | 100% | 95% |
| Class C | -0.03% | 0.09% | -0.06% | 109% | 108% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -0.06% | 0.29% | 111% | 111% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

Results reported at the JVET-AA0043 contribution have been confirmed.

# Reference

[1] K. Naser, T. Poirier, F. Galpin, A. Robert, “IntraTMP Adaptation for Camera Captured Contents,” JVET-AA0043, July 2022