| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0217-v4 |
| --- | --- |

| Title: | Crosscheck of JVET-AA0088(EE1-1.5: neural network based in-loop filter with a single model) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhihuang Xie | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the cross-check results of EE1-1.5, which is reported in JVET-AA0088[1]. In EE1-1.5, a single-model scheme, neural network-based loop filter is proposed and implemented on top of VTM-NNVC software[2] with SADL[3].

This contribution both crosschecked the training and testing process of EE1-1.5.

The simulation includes all-intra, random access and low delay B configurations, and the results from crosschecker perfectly match proponent’s for all-inra and low delay B conditions in terms of bitrate and PSNR, while very slight difference(0.0001) was observed in random access configuration which may probably be due to the rounding difference.

Introduction

EE1-1.5 reports the results of JVET-Z0091[4], which proposed a single-model scheme of neura network-based loop filter. Compared with JVET-Z0091, EE1-1.5 updated the SADL library and well tuned the models.

The crosscheck includes testing and training, which are described as following:

Crosscheck of testing

In specific, the following tests are included into EE1-1.5:

Test 1.5.1: Design and performance are identical to JVET-Y-E1-1.2.2 (single model, SADL, float32). Note that the model should be replaced by the re-trained model from the cross-checker in the cross-check process of compression performance.

Test 1.5.2: Optimize the float SADL implementation based on JVET-Z0091.

Test 1.5.3: Optimize the integer SADL implementaion based on JVET-Z0091.

For test 1.5.1, the model was replaced by the re-trained model from the cross-checker in testing process. And the simulation results showed the very close coding performance compared with JVET-Z0091.

For test 1.5.3, the onnx model was converted to integer-16 and saved as SADL model using SADL quantization method, and its coding performance was similar to the test 1.5.2’s.

Additional test of 1.5.3 was conducted. The difference between this additional test and Test 1.5.3 is that, the precision of test 1.5.3 is int16 but saved as floating, while the precision of the additional test is int16 and saved as integer.

Crosscheck of training

Regarding the training process of EE1-1.5.1, it is confirmed that under the detailed guidance provided by the proponent of EE1-1.5, the cross-checker can reproduce the SADL model, which has similar coding performance compared with JVET-Z0091’s.

Training process

Stage1: train the model using compressed data generated by NNVC anchor;

Stage2: further train the model using compressed data generated by JVET-X0054, which has the similar performance to JVET-Z0091.

Finally, the best model was selected according to the evaluation curve observed by tensorboard, then it was converted to SADL format.

Training dataset

DIV2K, BVI-DVC(1080p) and TVD were compressed by NNVC anchor and used as training dataset of stage 1.

For stage 2, the compressed data of DIV2K generated by NNVC anchor was reused without any changes, but dataset of BVI-DVC(1080p) and TVD were replaced with the new ones generated by JVET-X0054.

Training environment

GPU Type: Tesla V100 32GB

Framework: Pytorch v1.9.0

Model selection

The model of 828th epoch was selected as the output of stage 1 and the pre-trained model of stage 2. As shown in the figure 1, the model of 828th epoch had large improvement compared with the previous one,  and both the luma and chroma had good performance in term of delta PSNR in valid dataset.

Using the same strategy described above, the model of 1224th epoch was selected as the best model of the whole training.

Figure 1. evaluation curve

Additional crosscheck of training

It is suggested and requested by proponent of EE1-1.5 to have an additional alternative training method in in training process stage 2.

The training process in stage 2, which had been described above, used the compressed data generated from JVET-X0054. Those compressed data could be well prepared before starting the training stage 2. Once the training stage 1 completed, the training stage 2 could be launched immediately. This could save some training-dataset preparation time. Although JVET-X0054 was a comparable codec that had similar coding performance to JVET-Z0091, it indeed was a pseudo-iteration training.

The additional crosscheck didn’t need to train model from training stage 1, but used 828th epoch model as pre-trained model in the training stage 2. Besides, the training dataset of the additional training stage 2 was generated by the pre-trained model(828th epoch in stage 1).

Additional test: further train the model using the compressed data generated by JVET-Z0091 with re-trained model(828th epoch) of stage1.

Simulation results

The simulation was conducted under NNVC-CTC[5] on Ubuntu, CPU only. The results were shown as the following tables.

Test 1.5.1 (model was re-trained by crosschecker)

The model used in this test was retrained by crosschecker.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11_NNVC-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -9.40% | -15.37% | -19.68% | 313% | 325553% |
| Class A2 | -9.25% | -18.07% | -15.09% | 306% | 291027% |
| Class B | -8.27% | -20.16% | -20.25% | 302% | 321857% |
| Class C | -8.30% | -22.51% | -21.81% | 239% | 274219% |
| Class E |  |  |  |  |  |
| Overall | -8.70% | -19.41% | -19.52% | 287% | 302941% |
| Class D | -9.15% | -20.75% | -21.55% | 248% | 279422% |
| Class F | -3.85% | -12.43% | -11.79% | 467% | 162639% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11_NNVC-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7.31% | -15.33% | -16.42% | 248% | 234022% |
| Class C | -8.01% | -19.67% | -17.14% | 217% | 219103% |
| Class E | -7.89% | -14.15% | -12.89% | 473% | 199726% |
| Overall | -7.69% | -16.88% | -16.87% | 279% | 220047% |
| Class D | -9.02% | -18.86% | -18.43% | 221% | 228576% |
| Class F | -4.33% | -11.29% | -10.39% | 366% | 113368% |

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11_NNVC-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -6.22% | -13.66% | -15.84% | 399% | 162267% |
| Class A2 | -5.76% | -14.61% | -12.37% | 268% | 130949% |
| Class B | -5.81% | -15.04% | -16.20% | 244% | 131132% |
| Class C | -6.40% | -16.32% | -17.79% | 183% | 96203% |
| Class E | -8.64% | -15.33% | -15.66% | 273% | 153733% |
| Overall | -6.47% | -15.07% | -15.77% | 257% | 130210% |
| Class D | -6.42% | -14.27% | -17.14% | 178% | 104971% |
| Class F | -4.18% | -11.06% | -11.62% | 173% | 108042% |

Test 1.5.2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11_NNVC-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -9.31% | -14.86% | -18.51% | 186% | 128003% |
| Class A2 | -9.56% | -19.63% | -14.53% | 179% | 119881% |
| Class B | -8.51% | -19.42% | -20.80% | 185% | 129614% |
| Class C | -8.36% | -20.57% | -20.45% | 158% | 114235% |
| Class E |  |  |  |  |  |
| Overall | -8.84% | -18.86% | -18.99% | 176% | 123072% |
| Class D | -9.53% | -19.62% | -20.32% | 159% | 123109% |
| Class F | -3.78% | -12.25% | -11.38% | 239% | 63083% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11_NNVC-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7.35% | -18.56% | -20.42% | 155% | 100621% |
| Class C | -7.89% | -20.13% | -20.85% | 139% | 90288% |
| Class E | -8.32% | -17.70% | -16.74% | 260% | 85659% |
| Overall | -7.77% | -19.26% | -20.61% | 170% | 93223% |
| Class D | -8.93% | -18.35% | -18.71% | 143% | 97812% |
| Class F | -4.18% | -10.74% | -8.52% | 204% | 47590% |

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11_NNVC-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -6.11% | -14.20% | -17.27% | 230% | 73570% |
| Class A2 | -5.74% | -15.94% | -12.97% | 169% | 58247% |
| Class B | -5.92% | -14.97% | -17.20% | 154% | 58005% |
| Class C | -6.37% | -16.30% | -17.94% | 130% | 43735% |
| Class E | -8.78% | -15.31% | -16.40% | 171% | 67751% |
| Overall | -6.50% | -15.35% | -16.54% | 164% | 58205% |
| Class D | -6.41% | -15.43% | -18.52% | 127% | 48579% |
| Class F | -4.04% | -11.92% | -11.72% | 121% | 50620% |

Test 1.5.3

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11_NNVC-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -9.33% | -16.11% | -19.27% | 185% | 128006% |
| Class A2 | -9.72% | -19.34% | -14.92% | 177% | 120695% |
| Class B | -8.55% | -18.65% | -20.50% | 184% | 128820% |
| Class C | -8.59% | -19.56% | -20.76% | 158% | 114117% |
| Class E |  |  |  |  |  |
| Overall | -8.95% | -18.52% | -19.21% | 175% | 122953% |
| Class D | -9.78% | -18.84% | -20.16% | 159% | 123221% |
| Class F | -4.04% | -11.50% | -12.23% | 232% | 56525% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11_NNVC-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7.21% | -18.07% | -19.67% | 176% | 116960% |
| Class C | -7.77% | -19.83% | -20.38% | 156% | 104677% |
| Class E | -8.17% | -16.70% | -17.24% | 297% | 97502% |
| Overall | -7.64% | -18.85% | -19.99% | 193% | 107701% |
| Class D | -8.77% | -17.44% | -18.73% | 159% | 112739% |
| Class F | -4.36% | -10.70% | -11.31% | 237% | 54445% |

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11_NNVC-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -5.98% | -14.23% | -16.76% | 258% | 84321% |
| Class A2 | -5.72% | -15.91% | -13.17% | 186% | 67680% |
| Class B | -5.91% | -15.19% | -16.99% | 174% | 67004% |
| Class C | -6.53% | -15.12% | -17.90% | 143% | 50401% |
| Class E | -8.88% | -14.81% | -16.26% | 188% | 77740% |
| Overall | -6.52% | -15.07% | -16.40% | 182% | 67103% |
| Class D | -6.56% | -13.32% | -17.23% | 138% | 54209% |
| Class F | -4.24% | -11.83% | -12.11% | 134% | 57471% |

Additional Test (real iteration of training process)

The model used in this test was retrained by crosschecker as well.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11_NNVC-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  | -9.43% |  | -14.92% |  | -18.69% |  | 285% |  | 246115% |  |
| Class A2 |  | -8.90% |  | -17.55% |  | -15.00% |  | 248% |  | 222761% |  |
| Class B | -8.16% | -18.62% | -20.55% | 352% | 366144% |
| Class C | -8.47% | -19.97% | -21.25% | 297% | 340145% |
| Class E |  |  |  |  |  |
| Overall |  | -8.64% |  | -18.03% |  | -19.26% |  | 301% |  | 300232% |  |
| Class D | -9.59% | -18.45% | -21.29% | 315% | 359868% |
| Class F | -3.73% | -11.80% | -11.51% | 473% | 156356% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11_NNVC-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  | -7.39% |  | -15.98% |  | -16.84% |  | 244% |  | 236565% |  |
| Class E |  | -6.84% |  | -15.88% |  | -14.44% |  | 560% |  | 183416% |  |
| Overall |  |  |  |  |  |
| Class D | -8.48% | -13.36% | -17.12% | 241% | 260192% |
| Class F |  | -3.67% |  | -8.97% |  | -9.40% |  | 431% |  | 110972% |  |

Please be kindly noted that two QP-22 jobs in LDB are not completed yet.

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11_NNVC-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  | -6.18% |  | -13.20% |  | -15.44% |  | 479% |  | 173670% |  |
| Class A2 |  | -5.55% |  | -15.14% |  | -12.49% |  | 313% |  | 141570% |  |
| Class B | -5.71% | -14.53% | -16..64% | 303% | 148816% |
| Class C | -6.22% | -14.15% | -17.06% | 228% | 123927% |
| Class E | -8.53% | -14.74% | -16.37% | 337% | 201293% |
| Overall |  | -6.35% |  | -14.36% |  | -15.80% |  | 314% |  | 152903% |  |
| Class D | -6.26% | -12.34% | -16.52% | 220% | 152903% |
| Class F |  | -4.03% |  | -10.97% |  | -11.27% |  | 208% |  | 138837% |  |

Test 1.5.3 (int16 SADL model)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11_NNVC-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -8.59% | -19.23% | -20.83% | 150% | 77263% |
| Class C | -8.50% | -20.77% | -20.72% | 134% | 61991% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -9.78% | -19.80% | -20.45% | 126% | 59419% |
| Class F | -8.59% | -19.23% | -20.83% | 150% | 77263% |

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11_NNVC-1.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -5.93% | -15.54% | -17.46% | 144% | 39400% |
| Class C | -6.41% | -16.42% | -18.17% | 125% | 27069% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -6.41% | -15.30% | -18.56% | 119% | 26445% |
| Class F |  |  |  |  |  |

Conclusion

The test results from cross-checker match the proponents’ in AI and LDB, and very slight difference(0.0001) was observed in RA configuration.

Reference

[1] L. Wang, et al. “EE1-1.4: Neural network based in-loop filter with a single model”, JVET-AA0088.

[2] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc.

[3] F. Galpin, et al. “AhG11: SADL update”, JVET-Z0161.

[4] L. Wang, et al. “EE1-1.1: neural network based in-loop filter with a single model”, JVET-Z0091.

[5] E. Alshina, et al. “JVET common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-Z2016.