| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0228-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AA0120 (Non-EE2: Template-based multiple reference line intra prediction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Keming Cao | Tel: Email: | kemicao@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for JVET-AA0120, which proposes a template-based multiple reference line intra prediction mode. The cross-checked source code follows the description in the proposal, and the results are matched with the results provided by proponent.

# Introduction

In JVET-AA0120, it proposed to combine reference line and prediction mode together and uses a template matching method to construct a list of candidate combinations. An index to the candidate combination list is coded to indicate which reference line and prediction mode is used in coding the current block. In this proposal, the regular multiple reference line (MRL) for the non-TIMD part is replaced by the proposed mode.

The proposed TMRL mode includes the following aspects:

- Construct initial combination list which is consisted of combination of the extended reference line index and the intra prediction mode of size 50.

- Prune the initial combination list to size 20 with SAD cost in template region and sort the pruned list to get the TMRL combination list.

- The TMRL combination list will go through encoder SATD check and the index of TMRL is signaled if chosen. A truncated Golomb-Rice coding with a divisor 4 is employed to code selected combinations from the combination list.

The proposal also proposes an encoder-side modifications which allow more rounds of full RDO check for intra blocks larger than 8x8. If the SATD cost of the selected intra prediction mode is smaller than a preset threshold, it will be added to full RDO check candidate list.

Cross-check of the source code and simulation results have been conducted. Inspected source code follows the description in the proposal. The simulation is performed on top of ECM-5.0. The cross-check results are provided in the tables below and match the results provide by the proponent.

Below is the result for the test a (without encoder-side modifications):

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.13% | 0.06% | 103% | 103% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.06% | -0.04% | -0.01% | 103% | 107% |
| Class C | -0.12% | -0.09% | -0.08% | 102% | 106% |
| Class E | -0.14% | 0.04% | -0.09% | 104% | 106% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.11% | 0.05% | -0.09% | 103% | 105% |
| Class F | -0.12% | 0.03% | -0.08% | 103% | 106% |
| Class TGM | 0.00% | -0.01% | -0.03% | 103% | 105% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.04% | 0.03% | -0.17% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.03% | -0.01% | 0.09% | 102% | 102% |
| Class F | 0.00% | -0.03% | -0.07% | 101% | 101% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Below is the result for the test b (with encoder-side modifications):

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.02% | -0.07% | 108% | 102% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.11% | -0.05% | -0.07% | 107% | 104% |
| Class C | -0.15% | -0.08% | -0.01% | 107% | 105% |
| Class E | -0.18% | -0.07% | -0.05% | 108% | 105% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.12% | 0.05% | -0.12% | 108% | 104% |
| Class F | -0.21% | -0.13% | -0.35% | 104% | 104% |
| Class TGM | -0.05% | -0.03% | -0.03% | 105% | 106% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.04% | -0.07% | -0.03% | 103% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Reference

[1] Luhang Xu, Yue Yu, Haoping Yu, Dong Wang, “Non-EE2: Template-based multiple reference line intra prediction”, JVET-AA0120, July, 2022