| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0246-v1 |
| --- | --- |

| Title: | Non-EE2: Combination of JVET-AA0120 and JVET-AA0137 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luhang Xu Yue Yu Haoping Yu Dong Wang Keming Cao Vadim Seregin Marta Karczewicz | Tel: Email: | xuluhang@oppo.com yue.yu@oppo.com v-yuhaoping@oppo.com wangdong7@oppo.com kemicao@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | OPPO, Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to combine the template-based multiple reference line intra prediction in JVET-AA0120[1] and the intra prediction fusion method proposed in JVET-0137 [2]. The proposed method was implemented on top of ECM-5.0 software, and it reportedly provides Y, U, V-BD rate reduction with encoder and decoder runtime as follows:

AI:

# Introduction

## Template-based multiple reference line intra prediction

The proposed template-based multiple reference line intra prediction (TMRL) includes the following aspects:

- the extended reference line candidate list and the intra-prediction-mode candidate list.

The extended reference line candidate list is {1, 3, 5, 7, 12}. The restriction on CTU top row is unchanged. The size of the intra-prediction-mode candidate list is 10. The construction of the intra-prediction-mode candidate list is similar to MPM. The only difference is that PLANAR mode is excluded from the proposed intra-prediction-mode candidate list.

- construction of the TMRL combination list.

There are 5x10=50 combinations of the extended reference line and the allowed intra-prediction modes for a block. Since the extended reference line starts from reference line 1, the area covered by reference line 0 is used for template matching. The SAD costs over the template area (see Figure-2) are calculated between the predictions (generated by 50 combinations) and the reconstructions. Only the first 20 combinations with the least cost are pruned from the combinations in an ascending order to form the combination candidate list.

Figure-1. Illustration of template area

- signaling of TMRL

In ECM or VTM, MRL is always combined with the non-planar intra-prediction modes from primary MPM. When an extended reference line is selected, up to 6 context-coded bins and 3 equi-probable (EP) bins are needed to represent the MRL and MPM index. In the proposed TMRL mode, a truncated Golomb-Rice coding with a divisor 4 is employed to code selected combinations from the combination list. The binarization process and the codewords are shown in Table-2.

## Intra Prediction Fusion

In JVET-AA0137, it is proposed to derive intra prediction by weighting intra predictions obtained from multiple reference lines represented as pfusion=w0pline+w1pline+1, where pline is the intra prediction from the default reference line and pline+1 is the prediction from the line above the default reference line. In the experiment, the weights are set as w0=3/4 and w1=1/4.

The proposed intra prediction fusion is applied to luma blocks when angular intra mode has non-integer slope and the block size is greater than 16, it is used with MRL and not applied for ISP coded blocks.

Figure-2. Reference lines for current CU

# Proposed method

In the contribution, it is proposed to combine the intra prediction fusion and the TMRL. In the combined test, the encoder-side changes in JVET-AA0120 are not applied.

# Results

The proposed method is implemented on top of ECM-5.0. The test simulations were conducted using the Common Test Conditions as defined in [3]. Test results for AI configuration are summarized in the following tables. More detailed results can be found in the attached spreadsheets.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.06% | -0.11% | 110% | 103% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.20% | -0.12% | -0.10% | 109% | 104% |
| Class C | -0.24% | -0.29% | -0.08% | 107% | 110% |
| Class E | -0.22% | -0.07% | -0.22% | 109% | 103% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.21% | -0.11% | -0.25% | 106% | 109% |
| Class F | -0.09% | -0.07% | -0.20% | 106% | 105% |

# Conclusions

The proposed combined test of TMRL and the intra prediction fusion shows an additive coding gain. It is proposed to study this method in EE2.

# References

- Luhang Xu, Yue Yu, Haoping Yu, Dong Wang “Non-EE2: Template-based multiple reference line intra prediction,” JVET-AA0120, July. 2022.

- Keming Cao, Vadim Seregin, Marta Karczewicz, “Non-EE2: Intra Prediction Fusion,” JVET-AA0137, July. 2022.

- Marta Karczewicz, Yan Ye, "Common test conditions and evaluation procedures for enhanced compression tool testing," JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).