| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA2018-v2 |
| --- | --- |

| Title: | Common test conditions for high bit depth and high bit rate video coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document from JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Adrian Browne Tomohiro Ikai Dmytro Rusanovskyy Xiaoyu Xiu Yue Yu | Tel: Email: | adrian.browne@sony.com ikai.tomohiro@sharp.co.jp dmytror@qti.qualcomm.com xiaoyuxiu@kwai.com yue.yu@oppo.com |
| Source: | JVET |  |  |

_____________________________

# Abstract

This document defines common test conditions (CTC) and software reference configurations to be used in the context of high bit depth and high bit rate video coding experiments after the 27th JVET meeting. These common test conditions are recommended for use in technical contributions to the 28th and subsequent JVET meetings.

# Introduction

Common test conditions (CTC) are desirable to conduct experiments in a well-defined environment and to ease the comparison of the outcome of experiments.

This document defines three test conditions for high bit depth and high bit rate content, reflecting intra-only, random-access, and low-delay settings:

- Intra, 12 and 16 bit

- Random access, 12 and 16 bit

- Low delay, 12 and 16 bit

A subset of these test conditions might be used for a particular experiment. For example, when testing an intra coding tool, only intra configurations might be used.

When using the VTM software: Version 17.2 of the software is expected to be used for most experiments. More recent versions are encouraged where applicable. The VTM software can be accessed using the Git version control system and is available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git.

When using the HM software: Version 16.25 of the software is expected to be used for most experiments. More recent versions are encouraged where applicable. The HM software can be accessed using the Git version control system and is available at https://vcgit.hhi.fraunhofer.de/jvet/HM.git.

The test conditions define tests at bit depths of 12 and 16 bits. For 12 bit tests, PQ, HLG and RGB sequences have been defined. For 16 bit tests, only RGB sequences are used.

Any input contributions to JVET related to high bit depth, high bit rate or high frame rate video coding should provide a set of results using these test conditions that is as complete as possible.

The following sections define the software configuration, test sequences, quantization parameter values, evaluation processes, and encoder configuration files to be used.

# Software configuration

HM shall be configured for high bit depth processing by defining the following macro in the source code (TypeDef.h)

#define RExt__HIGH_BIT_DEPTH_SUPPORT           1

VTM shall be configured for high bit depth processing by defining the following macro in the source code (TypeDef.h)

#define JVET_R0351_HIGH_BIT_DEPTH_ENABLED			1

It is noted that the above software configuration shall be applied in both low QP range testing and normal QP range testing defined in this document.

Regarding wPSNR reporting, enabling HDRLib functionality in the VTM software is shown in Annex A in JVET-Z2011 [2]

# Quantization parameter values

Two sets of quantization parameters shall be used: low QP range and normal QP range configurations.

## Low QP range for both 12 and 16 bit

Results shall be provided using two sets of six quantization parameter values, one for 12-bit test cases and the other for 16-bit test cases as follows:

- 12 bit: 12, 7, 2, -3, -8 and -13 (mandatory)

- 16 bit: -8, -13, -18, -23, -28, -33 (mandatory)

- 16 bit: 12, 7, 2 and -3 (optional)

These values define the initial QP values that are used to derive respective QP for I- and B-frames in a sequence.

The method for calculating six-point BD-rates described in JVET-H0030 [3] should be used.

## Normal QP range for 12 bit YUV 420 content

Results shall be provided using two sets of four quantization parameter values, one for 12-bit test cases as follows:

- 12 bit: 22, 27, 32, and 37 (mandatory)

These values define the initial QP values (input encoder’s parameter) that are used by encoder to derive respective QP for I- and B-frames in a sequence.

The method for calculating four points BD-rates described in [4] should be used.

# Test sequences

## Test sequences for low QP range

Table 1, Table 2, Table 3, define the sets of test sequences to be used for intra, random-access, and low-delay in both lossy and lossless experiment conditions in the low QP range testing. Original versions of the test sequences in Table 1, 2 and 3 are available on ftp://jvet@ftp.ient.rwth-aachen.de/ctc/hbd/. (Properly qualified participants of JVET may please contact the JVET chairs for login information).

All frames (as defined by frame count in the table) shall be encoded for all sequences in the random-access and low-delay test cases described below. For the intra configuration, the first of every eight frames shall be encoded starting with the first frame in the sequence for all sequences, and bit rates shall be calculated using the sequence frame rate (as defined by frame rate in the table) divided by eight. Please note that temporal sub-sampling may be enabled in the HM and VTM software using the parameter TemporalSubsampleRatio. For random access and low delay configurations, if there is no number in the “encoded frame count” column for the configuration, the test case is not required in the CTC.

Table 1: SVT RGB 4:4:4 content

| Class | Sequence | Frame rate | Encoded frame count |  |  | Input bit depth | Internal bit depth |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AI | RA | LDB |  |  |
| SVT RGB (HD) | OldTownCross | 50 | 100 | 97 | 50 | 16 | 12 and 16 |
|  | CrowdRun | 50 | 100 | 97 | 50 | 16 | 12 and 16 |
|  | ParkJoy | 50 | 100 |  |  | 16 | 12 and 16 |
|  | DucksTakeOff | 50 | 100 |  |  | 16 | 12 and 16 |
|  | InToTree | 50 | 100 |  |  | 16 | 12 and 16 |

RGB sequences are mandatory in both, 12 and 16 bit tests.

Table 2: PQ YUV 4:4:4 and 4:2:2 content

| Class | Sequence | Frame rate | Encoded frame count |  |  | Input bit depth | Internal bit depth |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AI | RA | LDB |  |  |
| PQ (HD) | BallonFestival | 24 | 100 | 97 | 50 | 12 | 12 |
|  | EBU_04_Hurdles | 50 | 100 | 97 | 50 | 12 | 12 |
|  | Market3Clip4000 | 50 | 100 |  |  | 12 | 12 |
|  | EBU_06_Starting | 50 | 100 |  |  | 12 | 12 |
|  | FireEater2Clip4000r1 | 25 | 100 |  |  | 12 | 12 |

PQ sequences are in both YUV 4:4:4 and YUV 4:2:2 format and are mandatory in 12 bit tests.

Table 3: HLG YUV 4:4:4 and 4:2:2 content

| Class | Sequence | Frame rate | Encoded frame count |  |  | Input bit depth | Internal bit depth |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AI | RA | LDB |  |  |
| HLG (HD) | DayStreet | 60 | 100 | 97 | 50 | 12 | 12 |
|  | PeopleInShoppingCenter | 60 | 100 | 97 | 50 | 12 | 12 |
|  | NightStreet | 60 | 100 |  |  | 12 | 12 |
|  | StainedGlass | 60 | 100 |  |  | 12 | 12 |

HLG sequences are in both YUV 4:4:4 and YUV 4:2:2 format and are mandatory in 12 bit tests.

Table 4 and Table 5 indicate which test configurations are required for each type of content. The column corresponding to each configuration is interpreted as follows:

- “M” indicates that the test sequence is mandatory in the CTC for the given configuration

- “O” indicates that the test sequence is optional (but encouraged) in the CTC for the given configuration

- “-” indicates that the test sequence is not required in the CTC for the given configuration

Table 4: Required test conditions for lossy coding

| Class | Chroma sub-sampling | 12 bit |  |  | 16 bit |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AI | RA | LDB | AI | RA | LDB |
| SVT RGB | 4:4:4 | M | M | M | M | O | M |
| PQ | 4:4:4 | M | M | M | - | - | - |
|  | 4:2:2 | M | O | O | - | - | - |
| HLG | 4:4:4 | M | M | M | - | - | - |
|  | 4:2:2 | M | O | O | - | - | - |

Table 5: Required test conditions for lossless coding

| Class | Chroma sub-sampling | 12 bit |  |  | 16 bit |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AI | RA | LDB | AI | RA | LDB |
| SVT RGB | 4:4:4 | O | O | O | O | O | O |
| PQ | 4:4:4 | O | O | O | - | - | - |
|  | 4:2:2 | O | O | O | - | - | - |
| HLG | 4:4:4 | O | O | O | - | - | - |
|  | 4:2:2 | O | O | O | - | - | - |

## Test sequences for normal QP range

For normal QP tests, 12 bit YUV 4:2:0 versions of the PQ and HLG test sequences described in Table 6 and 7 are to be used. The YUV 4:2:0 version of the PQ and HLG sequences are available to download from the JVET ftp site ( ftp://jvet@ftp.ient.rwth-aachen.de/ctc/hbd/ ).

Table 6: PQ YUV 4:2:0 content

| Class | Sequence | Frame rate | Encoded frame count |  |  | Input bit depth | Internal bit depth |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AI | RA | LDB |  |  |
| PQ (HD) | BallonFestival | 24 | 240 | 240 |  | 12 | 12 |
|  | EBU_04_Hurdles | 50 | 500 | 500 |  | 12 | 12 |
|  | Market3Clip4000 | 50 | 400 | 400 |  | 12 | 12 |
|  | EBU_06_Starting | 50 | 500 | 500 |  | 12 | 12 |

Table 7: HLG YUV 4:2:0 content

| Class | Sequence | Frame rate | Encoded frame count |  |  | Input bit depth | Internal bit depth |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AI | RA | LDB |  |  |
| HLG (HD) | DayStreet | 60 | 300 | 300 |  | 12 | 12 |
|  | PeopleInShoppingCenter | 60 | 300 | 300 |  | 12 | 12 |
|  | NightStreet | 60 | 300 | 300 |  | 12 | 12 |
|  | StainedGlass | 60 | 300 | 300 |  | 12 | 12 |

Table 8: Required test conditions for lossy coding

| Class | Chroma sub-sampling | 12 bit |  |  |
| --- | --- | --- | --- | --- |
|  |  | AI | RA | LDB |
| PQ (HD) | 4:2:0 | M | M | O |
| HLG (HD) | 4:2:0 | M | M | O |

# Configuration

## Configuration for low QP range

For low QP range testing, this document defines three test conditions reflecting intra-only, random-access, and low-delay configurations, with encoder parameters being specified by a respective configuration file provided for both HM and VTM software.

### HM

For HM software, the configuration files for the three test conditions are:

- “All Intra” (AI): cfg/encoder_intra_main_rext_subsample.cfg

- “Random access” (RA): cfg/encoder_randomaccess_main_rext.cfg

- “Low-delay B” (LB): cfg/encoder_lowdelay_main_rext.cfg

There are two depth specific parameters files which specify the internal bit depths and should be used:

- 12 bit: cfg/hbd/12-bit.cfg

- 16 bit: cfg/hbd/16-bit.cfg

Additional parameters are specified in the cfg/hbd/hbd.cfg file which enables extended precision processing and disables temporal filtering. This file should be used for both

Encoder parameters for lossless test configuration are specified in cfg/lossless/lossless.cfg.

### VTM

For VTM software, the corresponding file are following:

- “All Intra” (AI): cfg/encoder_intra_vtm.cfg

- “Random access” (RA): cfg/encoder_randomaccess_vtm_gop16.cfg

- “Low-delay B” (LDB): cfg/encoder_lowdelay_vtm.cfg

VTM software specifies additional parameters which are mandatory to use in this CTC and provided as respective configuration files.

The test data format specific encoder parameters:

- SVT RGB (both 12 bit and 16 bit): cfg/per-class/formatRGB.cfg

- PQ (both 4:4:4 and 4:2:2): cfg/per-class/classH1.cfg

- HLG (both 4:4:4 and 4:2:2): cfg/per-class/classH2.cfg

The bit depth specific encoder parameters files:

- 12 bit: cfg/hbd/12-bit.cfg

- 16 bit: cfg/hbd/16-bit.cfg

Complexity limiting encoder parameters are specified by the cfg/hbd/hbd.cfg file and include following settings:

- Disable LFNST, LMCS, BCW, MMVD, SMVD, Geo, BIO, DMVR, Affine, ISP, but encourage testing of tool-on configurations in contributions.

- Disable temporal filtering.

Encoder parameters for lossless test configuration are specified as following:

- SVT RGB 16 bit: cfg/lossless/lossless.cfg and cfg/lossless/losslessRGB.cfg, internal bit depth 16.

Note: internal bit depth 16 must be set by applying cfg/hbd/16-bit.cfg after cfg/lossless/lossless.cfg.

- SVT RGB 12 bit: cfg/lossless/lossless.cfg and cfg/lossless/losslessRGB.cfg, internal bit depth 12.

Note: internal bit depth 12 must be set by applying cfg/hbd/12-bit.cfg after cfg/lossless/lossless.cfg.

- HLG and PQ (both 4:4:4 and 4:2:2): cfg/lossless/lossless.cfg and cfg/lossless/lossless444.cfg.

The combination of configuration files to be used for individual tests is summarised in Table 9.

## Configuration for normal QP range

For normal QP range testing, this document defines two test conditions reflecting intra-only and random-access configurations, with encoder parameters being specified by a respective configuration file provided with HM and VTM software.

For HM software, the corresponding files are:

- “All Intra” (AI): cfg/encoder_intra_main_rext_subsample.cfg

- “Random access” (RA): cfg/encoder_randomaccess_main_rext.cfg

For VTM software, the corresponding files are:

- “All Intra” (AI): cfg/encoder_intra_vtm.cfg

- “Random access” (RA): cfg/encoder_randomaccess_vtm.cfg

The rest of HM and VTM encoder parameters in this configuration are defined by the common test conditions for HM and VTM testing with HDR/WCG data [2] with mandatory exception of:

- InternalBitDepth=12

This configuration setting is included in cfg/hbd/12-bit.cfg which should be used.

# Summary of configurations

The CTC configuration is summarized in Table 9.

Table 9: Summary of configurations

| Software | Content | NormalQP | LowQP | Lossless | Class | BitDepth |
| --- | --- | --- | --- | --- | --- | --- |
| HM | PQ | GOP 16 | GOP16 hbd.cfg | GOP16 hbd.cfg lossless.cfg |  | 12-bit.cfg |
|  | HLG |  |  |  |  |  |
|  | SVT RGB (12bit) |  |  |  |  |  |
|  | SVT RGB (16 bit) |  |  |  |  | 16-bit.cfg |
| VTM | PQ | GOP32 | GOP16 hbd.cfg | GOP16 hbd.cfg lossless.cfg lossless444.cfg | classH1.cfg | 12-bit.cfg |
|  | HLG |  |  |  | classH2.cfg |  |
|  | SVT RGB (12bit) |  |  | GOP16 hbd.cfg lossless.cfg losslessRGB.cfg | formatRGB.cfg |  |
|  | SVT RGB (16 bit) |  |  |  |  | 16-bit.cfg |

Note: lossless.cfg overwrites InternalBitDepth, thus for lossless tests the 12-bit.cfg or 16-bit.cfg configuration file shall be applied after lossless.cfg to ensure the correct bit depth is used for the test.

# Evaluation

Results reported for PQ sequences shall include the following metrics:

- PSNR for Y′, Cb and Cr

- wPSNR for Y’, Cb and Cr, as defined in HDR/WCG CTC [2]

Results reported for the HLG and SVT RGB sequences shall include the following metrics:

- PSNR for Y’, Cb and Cr

- PSNR for R, G and B

Three Excel templates for reporting test results are provided alongside this document.

- Template for normal QP test configuration (Anchor is VTM-17.0 or later if available)

- Template for low QP test configuration with VTM-17.0 anchor

- Template for low QP test configuration with HM-16.25 anchor

# MD5

## SVT

6004ff26cafcf2b066faa00c935a7621 CrowdRun_1920x1080_50_16bit_444.rgb

2ab943d7b1c6e3b26a2c6da416b69828 ParkJoy_1920x1080_50_16bit_444.rgb

d882efad39ae3d360896ec09d9213424 DucksTakeOff_1920x1080_50_16bit_444.rgb

6f7d413472f856741e95543217f9795d InToTree_1920x1080_50_16bit_444.rgb

ae5bab60c3dfb16399971b9afff120da OldTownCross_1920x1080_50_16bit_444.rgb

## PQ

### PQ 4:2:0

38216309e091dafabea8be6b4393fd7d BalloonFestival_1920x1080p_24_12b_pq_709_ct2020_420.yuv

3ea0125f8d34fd0e4ba1ed95c5a8cc01 Market3Clip4000r2_1920x1080p_50_12b_pq _709_ct2020_420.yuv

8836277e0e14e1ae86436cb84c2fe6da EBU_04_Hurdles_1920x1080p_50_12b_pq_709_ct2020_420.yuv

26773f9e00132ca318a4b45339ca02ee EBU_06_Starting_1920x1080p_50_12b_pq_709_ct2020_420.yuv

### PQ 4:2:2

b029dace9f10af78aa6c3363531b1768 BalloonFestival_1920x1080p_24_12b_pq_709_ct2020_422.yuv

716c1ec421a72c46011f31754077eec4 Market3Clip4000r2_1920x1080p_50_12b_pq _709_ct2020_422.yuv

b9f22e80fa8eb33e3c00633e6824844f EBU_04_Hurdles_1920x1080p_50_12b_pq_709_ct2020_422.yuv

a480ed6fc9e9f4e6b84b1ab6e7bc4f1b EBU_06_Starting_1920x1080p_50_12b_pq_709_ct2020_422.yuv

### PQ 4:4:4

1664a322cf91934872afd07035819a05 BalloonFestival_1920x1080p_24_12b_pq_709_ct2020_444.yuv

564904c4001957472483a3d79b55f17d Market3Clip4000r2_1920x1080p_50_12b_pq_709_ct2020_444.yuv

544b1b8816d0003e69ce6a28bbd01810 EBU_04_Hurdles_1920x1080p_50_12b_pq_709_ct2020_444.yuv

f6761039392e76336493effdc257bf1a EBU_06_Starting_1920x1080p_50_12b_pq_709_ct2020_444.yuv

## HLG

### HLG 4:2:0

ae1af2b4d594315089186d0e6650aaca DayStreet_1920x1080_60p_12bit_420_hlg.yuv

2c3059fbcb4b2711500f905b4c49df23 PeopleInShoppingCenter_1920x1080_60p_12bit_420_hlg.yuv

c8c3d00e7cc9f3a06b60d13522bf4de0 NightStreet_1920x1080_60p_12bit_420_hlg.yuv

803b5fda1d99c1c236cb67fef68a6ac6 StainedGlass_1920x1080_60p_12bit_420_hlg.yuv

### HLG 4:2:2

ba3c3b0576f98bc923773d48db3f4416 DayStreet_1920x1080_60p_12bit_422_hlg.yuv

1dbc59e747b08e0abe4d7a7c574c691b PeopleInShoppingCenter_1920x1080_60p_12bit_422_hlg.yuv

271ae806359e070d1347750299dba4cb NightStreet_1920x1080_60p_12bit_422_hlg.yuv

dde95ad0188511e050b9cc23a5491f26 StainedGlass_1920x1080_60p_12bit_422_hlg.yuv

### HLG 4:4:4

1e9702b1aea62a4a00224bd0398238ca DayStreet_1920x1080_60p_12bit_444_hlg.yuv

2fab7d501cd6232f0e9e80b21ebb1eb8 PeopleInShoppingCenter_1920x1080_60p_12bit_444_hlg.yuv

d54aa384b79b1b10f8160b63529d1df2 NightStreet_1920x1080_60p_12bit_444_hlg.yuv

daf169ed0811a929fbc1927a0a6a9709 StainedGlass_1920x1080_60p_12bit_444_hlg.yuv

# References

[1] F. Bossen, X. Li, V. Seregin, K. Sharman, K. Sühring , “VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video ”, JVET-Y2010, JVET, 25th Meeting: by teleconference, 12-21 Jan. 2022

[2] A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy , “VTM and HM common test conditions and evaluation procedures for HDR/WCG video”, JVET-Z2011, JVET, 26th Meeting: by teleconference, 20–29 April 2022

[3] A. M. Tourapis, D. Singer, Y. Su, K. Mammou, “BD-Rate/BD-PSNR Excel extensions”, JVET-H0030, JVET, 8th Meeting: Macao, CN, 18–25 Oct. 2017

[4] K. Andersson, F. Bossen, J.-R. Ohm, A. Segall, R. Sjöberg, J. Ström, G. J. Sullivan, A. Tourapis “Summary information on BD-rate experiment evaluation practices,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 output document JVET-R2016, 18th JVET meeting, by teleconference, April 2020 (available online at https://jvet-experts.org/).