| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0063-v1 |
| --- | --- |

| Title: | Cross-check of JVET-AB0053 (EE1-1.2: NN intra model without attention and partitioning strength) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Maria Santamaria Francesco Cricri Nokia Technologies Tampere, Finland | Email: | maria.santamaria_gomez@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports the cross-check (training and inference) results of JVET-AB0053 (EE1-1.2) related to a NN-based luma loop filter. The training was cross-checked by means of using the models trained by the cross-checker during inference. It is reported that, compared to the proponents’ results, the overall BD-rate PSNR deviates up to 0.1%.

# Simulation results

JVET-AB0053 [1] describes two luma in-loop filters. Both the training and inference codes are available at [2].

## Training

The training environment has Ubuntu 20.04.3 LTS, an NVIDIA GPU A100-SXM-80GB, CUDA Toolkit 11.3.1 and PyTorch 1.12.1.

Two luma filters were trained: original and simplified. The latter is a simplification of the former. Specifically, the simplified model does not use the partitioning data as input, and it removes the spatial attention.

For both models the training consisted of two stages:

- Train from scratch for 372 epochs using learning rate 1e-4 and L1 loss.

- Fine-tune the model from step 1 for 7 epochs using learning rate 1e-5 and MSE loss.

After the training was done, both models were converted to SADL format (float32).

The training of the simplified model was faster than the training of the original model. The time reduction was about 14%.

## Inference

The cross-check simulations ran on a cluster with CentOS Linux (Core), Intel Xeon Gold 6154 CPUs and GCC 9.3.1.

Two anchors are used for the evaluation:

- VTM 11.0 NNVC 2.0

- NCS 1.0 with filter set 1 enabled.

Notice the anchor simulations ran in a different computing environment.

## Original model

|  | Random access Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -8,87 % | -16,01 % | -18,76 % |
| Class A2 | -10,22 % | -20,31 % | -17,35 % |
| Class B | -9,05 % | -22,95 % | -21,70 % |
| Class C | -9,90 % | -21,68 % | -22,38 % |
| Class E |  |  |  |
| Overall | -9,47 % | -20,69 % | -20,43 % |
| Class D | -11,49 % | -23,60 % | -23,83 % |
| Class F | -4,79 % | -11,67 % | -10,66 % |

|  | Low delay B Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class B | -7,82 % | -15,98 % | -14,10 % |
| Class C | -9,15 % | -15,18 % | -15,28 % |
| Class E | -8,77 % | -16,92 % | -17,27 % |
| Overall | -8,50 % | -15,63 % | -14,62 % |
| Class D | -10,88 % | -18,69 % | -18,35 % |
| Class F | -4,88 % | -8,81 % | -5,89 % |

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -6,57 % | -17,73 % | -20,00 % |
| Class A2 | -6,87 % | -20,18 % | -17,32 % |
| Class B | -6,82 % | -21,66 % | -21,62 % |
| Class C | -7,62 % | -18,68 % | -21,86 % |
| Class E | -10,24 % | -20,82 % | -20,59 % |
| Overall | -7,53 % | -19,96 % | -20,51 % |
| Class D | -7,49 % | -19,03 % | -21,08 % |
| Class F | -4,75 % | -11,76 % | -10,91 % |

|  | Random access Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NCS 1.0 filter set 1 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | 0,01 % | 0,12 % | 0,03 % |
| Class A2 | -0,08 % | -0,06 % | -0,02 % |
| Class B | -0,07 % | -0,03 % | 0,15 % |
| Class C | -0,07 % | 0,04 % | 0,03 % |
| Class E |  |  |  |
| Overall | -0,06 % | 0,01 % | 0,06 % |
| Class D | -0,07 % | -0,24 % | -0,09 % |
| Class F | -0,15 % | -0,01 % | -0,07 % |

|  | Low delay B Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NCS 1.0 filter set 1 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class B | -0,03 % | 0,23 % | -0,34 % |
| Class C | 0,01 % | 0,14 % | -0,14 % |
| Class E | -0,28 % | -0,19 % | 0,16 % |
| Overall | -0,08 % | 0,19 % | -0,25 % |
| Class D | -0,12 % | -0,41 % | -0,44 % |
| Class F | 0,09 % | 0,21 % | 0,41 % |

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NCS 1.0 filter set 1 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0,16 % | 0,30 % | -0,29 % |
| Class A2 | -0,41 % | 0,39 % | 0,58 % |
| Class B | -0,35 % | 0,24 % | -0,06 % |
| Class C | -0,22 % | 0,21 % | -0,17 % |
| Class E | -0,41 % | -0,01 % | 0,02 % |
| Overall | -0,31 % | 0,23 % | 0,00 % |
| Class D | -0,15 % | 0,46 % | 0,32 % |
| Class F | -0,67 % | -0,03 % | -0,02 % |

## Simplified model

|  | Random access Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -8,88 % | -15,92 % | -18,70 % |
| Class A2 | -10,22 % | -20,29 % | -17,36 % |
| Class B | -9,07 % | -22,96 % | -21,71 % |
| Class C | -9,88 % | -21,70 % | -22,40 % |
| Class E |  |  |  |
| Overall | -9,48 % | -20,68 % | -20,42 % |
| Class D | -11,47 % | -23,56 % | -23,77 % |
| Class F | -4,92 % | -11,63 % | -10,58 % |

|  | Low delay B Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class B | -7,78 % | -15,96 % | -14,19 % |
| Class C | -9,18 % | -15,26 % | -15,08 % |
| Class E | -8,81 % | -17,12 % | -17,35 % |
| Overall | -8,50 % | -15,65 % | -14,59 % |
| Class D | -10,81 % | -18,34 % | -18,00 % |
| Class F | -5,02 % | -9,04 % | -6,31 % |

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -6,55 % | -18,07 % | -20,39 % |
| Class A2 | -6,76 % | -20,35 % | -17,59 % |
| Class B | -6,75 % | -21,71 % | -21,63 % |
| Class C | -7,55 % | -18,80 % | -21,94 % |
| Class E | -10,18 % | -20,81 % | -20,62 % |
| Overall | -7,47 % | -20,08 % | -20,65 % |
| Class D | -7,44 % | -19,32 % | -21,28 % |
| Class F | -5,13 % | -11,76 % | -10,92 % |

|  | Random access Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NCS 1.0 filter set 1 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | 0,01 % | 0,22 % | 0,10 % |
| Class A2 | -0,08 % | -0,05 % | -0,03 % |
| Class B | -0,10 % | -0,05 % | 0,14 % |
| Class C | -0,05 % | 0,04 % | 0,02 % |
| Class E |  |  |  |
| Overall | -0,06 % | 0,03 % | 0,06 % |
| Class D | -0,04 % | -0,20 % | -0,04 % |
| Class F | -0,29 % | 0,04 % | 0,01 % |

|  | Low delay B Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NCS 1.0 filter set 1 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class B | 0,01 % | 0,29 % | -0,43 % |
| Class C | -0,02 % | 0,02 % | 0,07 % |
| Class E | -0,33 % | -0,38 % | 0,14 % |
| Overall | -0,08 % | 0,17 % | -0,21 % |
| Class D | -0,04 % | -0,04 % | -0,04 % |
| Class F | -0,06 % | -0,07 % | -0,09 % |

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NCS 1.0 filter set 1 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0,14 % | -0,12 % | -0,73 % |
| Class A2 | -0,30 % | 0,17 % | 0,25 % |
| Class B | -0,29 % | 0,18 % | -0,06 % |
| Class C | -0,15 % | 0,07 % | -0,27 % |
| Class E | -0,34 % | 0,01 % | -0,01 % |
| Overall | -0,24 % | 0,08 % | -0,16 % |
| Class D | -0,10 % | 0,14 % | 0,07 % |
| Class F | -1,07 % | -0,04 % | -0,02 % |

## Comparison against proponent’s results

While there are differences between the cross-checker’s results and the proponent’s results reported in JVET-AB0053, overall, these go up to 0.1%.

Notice that computing environments are different.

|  | Original RA Main10 |  |  |
| --- | --- | --- | --- |
|  | JVET-AB0053 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0,02 % | -0,01 % | -0,05 % |
| Class A2 | 0,00 % | -0,10 % | 0,04 % |
| Class B | 0,02 % | 0,05 % | 0,06 % |
| Class C | -0,01 % | 0,09 % | -0,06 % |
| Class E |  |  |  |
| Overall | 0,00 % | 0,02 % | 0,00 % |
| Class D | -0,06 % | 0,02 % | 0,09 % |
| Class F | 0,12 % | 0,01 % | -0,02 % |

|  | Original LDB Main10 |  |  |
| --- | --- | --- | --- |
|  | JVET-AB0053 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class B | -0,04 % | -0,17 % | 0,18 % |
| Class C | 0,02 % | -0,17 % | 0,08 % |
| Class E | -0,06 % | -0,03 % | 0,22 % |
| Overall | -0,03 % | -0,17 % | 0,13 % |
| Class D | -0,08 % | 0,11 % | -0,25 % |
| Class F | 0,02 % | 0,36 % | 0,73 % |

|  | Original AI Main10 |  |  |
| --- | --- | --- | --- |
|  | JVET-AB0053 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0,04 % | 0,33 % | -0,11 % |
| Class A2 | -0,01 % | 0,40 % | 0,61 % |
| Class B | -0,04 % | -0,09 % | -0,08 % |
| Class C | -0,08 % | -0,06 % | -0,03 % |
| Class E | -0,04 % | -0,03 % | 0,00 % |
| Overall | -0,04 % | 0,08 % | 0,05 % |
| Class D | -0,04 % | 0,10 % | -0,02 % |
| Class F | 0,28 % | 0,00 % | 0,01 % |

|  | Simplified RA Main10 |  |  |
| --- | --- | --- | --- |
|  | JVET-AB0053 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | 0,00 % | 0,02 % | 0,07 % |
| Class A2 | 0,00 % | -0,06 % | 0,02 % |
| Class B | 0,00 % | 0,04 % | 0,18 % |
| Class C | 0,03 % | 0,01 % | -0,03 % |
| Class E |  |  |  |
| Overall | 0,01 % | 0,01 % | 0,07 % |
| Class D | 0,00 % | -0,01 % | 0,24 % |
| Class F | 0,10 % | 0,03 % | 0,11 % |

|  | Simplified LDB Main10 |  |  |
| --- | --- | --- | --- |
|  | JVET-AB0053 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class B | 0,04 % | 0,19 % | -0,10 % |
| Class C | 0,00 % | -0,26 % | 0,00 % |
| Class E | -0,16 % | -0,47 % | 0,57 % |
| Overall | -0,02 % | -0,01 % | -0,06 % |
| Class D | -0,01 % | 0,84 % | -0,05 % |
| Class F | -0,08 % | -0,16 % | -0,49 % |

|  | Simplified AI Main10 |  |  |
| --- | --- | --- | --- |
|  | JVET-AB0053 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | 0,00 % | -0,30 % | -0,48 % |
| Class A2 | 0,09 % | -0,13 % | -0,22 % |
| Class B | 0,07 % | 0,18 % | 0,17 % |
| Class C | 0,06 % | -0,12 % | -0,09 % |
| Class E | 0,04 % | 0,01 % | -0,03 % |
| Overall | 0,05 % | -0,05 % | -0,10 % |
| Class D | 0,02 % | -0,19 % | -0,22 % |
| Class F | 0,15 % | 0,00 % | 0,00 % |

# References

- J. Ström, D. Liu, K. Andersson, P. Wennersten, M. Damghanian, R. Yu (Ericsson). EE1-1.2: NN intra model without attention and partitioning strength. JVET-AB0053. October 2022.

- https://vcgit.hhi.fraunhofer.de/jvet-aa-ee1/VVCSoftware_VTM/-/tree/EE1-1.2

# Patent rights declaration(s)

Nokia does not have any current or pending patent rights relating to the technology described in this contribution.