| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0065-v3 |
| --- | --- |

| Title: | Non-EE2: Adaptive reference region DIMD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zheming Fan Yukinobu Yasugi Tomohiro Ikai | Email: | fan.zheming@sharp.co.jp yasugi.yukinobu@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to introduce new mode for DIMD, named adaptive reference region DIMD. The proposed DIMD consists of DIMD_TL (top and left), DIMD_L (left) and DIMD_T (top) where a new syntax element is signalled to select neighboring reconstructed pixels (reference region) used to derive a prediction direction (DIMD intra mode). The experimental results reportedly show that the average luma BDrate gain of the proposal is 0.1 % and 0.06 %, under AI and RA respectively.

Introduction

The Decoder-side Intra Mode Derivation (DIMD) [1] is an effective method used for intra prediction. In DIMD method, the intra prediction mode is derived by using previously reconstructed neighboring pixels through a gradient analysis. Meanwhile, the reference region used in DIMD is fixed as left, top and top-left neighborhoods and the number of lines in the reference area is fixed at 3.

Proposal

In this contribution, we add two new modes DIMD_T and DIMD_L to select different neighborhoods (reference region). In the DIMD_T we use top-left, top, and top-right neighborhoods as reference region, and in DIMD_L mode we use top-left, left, and left-bottom neighborhoods as reference region. Moreover, we set the number of lines in the reference area in DIMD_T and DIMD_L modes to 4. The reference region used in these two modes are shown in Figure 1. The original DIMD mode is referred as DIMD_TL.

Figure 1. Reference region of DIMD, DIMD_T and DIMD_L.

In each CU, a new flag named cu_dimd_mode is signalled after the DIMD enabled flag (cu_dimd_flag) to determine which DIMD mode is used. The binarization of cu_dimd_mode are summarized in Table 1.

Table 1 The binaryzation of cu_dimd_mode

| cu_dimd_mode | Binarization | Name |
| --- | --- | --- |
| 0 | 0 | DIMD_TL |
| 1 | 10 | DIMD_T |
| 2 | 11 | DIMD_L |

Experimental Results

The test results are shown in Table2.

Table2. Test Results.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.12% | -0.04% | 116% | 100% |
| Class A2 | -0.10% | -0.01% | 0.00% | 114% | 100% |
| Class B | -0.09% | 0.06% | 0.07% | 117% | 100% |
| Class C | -0.08% | 0.08% | 0.00% | 115% | 100% |
| Class E | -0.07% | 0.07% | -0.07% | 117% | 100% |
| Overall | -0.10% | 0.02% | 0.00% | 116% | 100% |
| Class D | -0.07% | 0.18% | -0.03% | 116% | 100% |
| Class F | -0.06% | 0.01% | 0.09% | 109% | 100% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.03% | -0.04% | 104% | 104% |
| Class A2 | -0.06% | -0.01% | -0.05% | 104% | 110% |
| Class B | -0.06% | 0.14% | -0.29% | 102% | 100% |
| Class C | -0.04% | -0.03% | -0.10% | 98% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | 0.03% | -0.14% | 101% | 101% |
| Class D | -0.05% | 0.12% | 0.05% | 114% | 103% |
| Class F | -0.05% | -0.03% | -0.41% | 101% | 79% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Conclusion

The experiment proves that the new DIMD proposed modes shows 0.1% Bd-rate gain on luma compared with the original DIMD mode. It is recommended to study this technique in EE.

Acknowledgements

These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Reference

[1] M. Abdoli, T. Guionnet, E. Mora, et. al, “Non-CE3: Decoder-side Intra Mode Derivation with Prediction Fusion Using Planar”, JVET-O0449, Gothenburg, July 2019.