| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0079-v1 |
| --- | --- |

| Title: | EE2-2.2: Template matching based BCW index derivation for merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao Jie Chen Yan Ye Xinwei Li | Tel: Email: | {ruling.lrl, jiechen.cj, yan.ye, sid.lxw}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution reports the simulation results of template matching based BCW index derivation for merge mode, which is studied in EE2 Test 2.2. In Test 2.2, the BCW index for merge coded CUs is derived based on template matching cost instead of inferring from neighboring blocks. Given a selected merge candidate, the TM cost values are calculated with different bi-predicted weights, and then, the bi-predicted weight with minimum TM cost value is used to predict the merge CU. In addition, the bi-predicted weights for merge mode are extended. It is reported that on top of ECM-6.0, the overall coding performance impact for {Y, U, V, EncT, DecT} are:

- Without negative bi-predicted weights:
{-0.11%, -0.13%, -0.19%, 101%, 100%} for RA and {-0.08%, -0.04%, 0.21%, 100%, 100%} for LDB.

- With negative bi-predicted weights:
{-0.11%, -0.14%, -0.20%, 100%, 100%} for RA and {-0.10%, -0.31%, 0.18%, 100%, 99%} for LDB.

Introduction

This contribution presents the simulation results of EE2 Test 2.2 [1] which studies the template matching based BCW index derivation for merge mode.

Proposed methods

In Test 2.2, it is proposed to derive BCW index for merge coded CUs according to template matching (TM) cost and extend bi-predicted weights for merge mode.

Firstly, the bi-predicted weights for merge mode is extended from {-2, 3, 4, 5, 10} to {-4, -3, -2, -1, 1, 2, 3, 4, 5, 6, 7, 9, 10, 11, 12} if negative bi-predicted weights are supported. When negative bi-predicted weights are not supported, the bi-predicted weights for merge mode is extended from {-2, 3, 4, 5, 10} to {1, 2, 3, 4, 5, 6, 7}. In addition, the negative bi-predicted weights for non-merge mode is replaced with positive weights, that is, the weights {-2, 10} is replaced with {1, 7}.

Secondly, for a CU coded in one of regular merge, template matching, adaptive decoder-side motion vector refinement and MMVD modes, its bi-predicted weight is derived based on TM cost. When calculating TM cost for bi-predicted weights, the following rules are applied:

- Since the inherited bi-predicted weight is likely to have higher accuracy than others, only the two neighboring bi-predicted weights (i.e. ±1) and the inherited bi-predicted weight are considered. For example, the inherited bi-predicted weight is 4, only three weights {3, 4, 5} are involved in TM cost calculation.

- The TM cost of the inherited BCW index is multiplied with 0.90625, that is, the cost is reduced by 3/32.

- The TM cost of the equal weight is multiplied with 0.90625 since bi-predicted samples are beneficial for BDOF and BDOF is only applied to CU with equal weight.

Simulation results

The proposed methods are implemented on top of ECM-6.0 and are tested under CTC for enhanced compression tool testing [2] with random access and low delay B configurations. Two tests: 1) without negative weights supported and 2) with negative weights supported are evaluated, and the results are shown in Table 1 and Table 2, respectively.

Table 1. Proposed method without negative weights

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.24% | -0.26% | 102% | 100% |
| Class A2 | -0.12% | -0.04% | -0.17% | 102% | 101% |
| Class B | -0.11% | -0.09% | -0.29% | 100% | 99% |
| Class C | -0.05% | -0.17% | -0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.13% | -0.19% | 101% | 100% |
| Class D | 0.02% | -0.35% | -0.04% | 100% | 101% |
| Class F | -0.24% | -0.26% | -0.27% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.10% | -0.10% | -0.19% | 102% | 100% |
| Class C | -0.09% | -0.20% | 0.50% | 99% | 99% |
| Class E | -0.03% | 0.29% | 0.49% | 101% | 100% |
| Overall | -0.08% | -0.04% | 0.21% | 100% | 100% |
| Class D | 0.04% | -0.31% | 0.29% | 100% | 100% |
| Class F | -0.01% | -0.19% | -0.69% | 99% | 98% |

Table 2. Proposed method with negative weights

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.32% | -0.21% | 102% | 100% |
| Class A2 | -0.09% | -0.03% | -0.19% | 101% | 101% |
| Class B | -0.13% | -0.12% | -0.32% | 100% | 99% |
| Class C | -0.04% | -0.12% | -0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.14% | -0.20% | 100% | 100% |
| Class D | 0.01% | -0.28% | -0.13% | 100% | 101% |
| Class F | -0.24% | -0.27% | -0.25% | 99% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.09% | -0.37% | 0.00% | 102% | 100% |
| Class C | -0.10% | -0.12% | 0.44% | 99% | 98% |
| Class E | -0.12% | -0.46% | 0.13% | 101% | 99% |
| Overall | -0.10% | -0.31% | 0.18% | 100% | 99% |
| Class D | -0.06% | 0.02% | 0.51% | 101% | 102% |
| Class F | -0.21% | -0.35% | -0.67% | 98% | 98% |

Conclusion

This contribution reports the simulation results of EE2 Test 2.2. Experimental results show that {0.11%, 0.08%} luma bitrate saving can be achieved in RA and LB conditions when negative bi-predicted weights are not supported. When negative bi-predicted weights are supported, {0.11%, 0.10%} luma bitrate saving can be obtained in RA and LB conditions, respectively. Thus, it is recommended to adopt the proposed method to ECM.

References

- V. Seregin and et al., “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AA2024, July 2022.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Feb. 2022.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).