| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0089-v2 |
| --- | --- |

| Title: | Crosscheck of JVET-AB0052 (EE1-1.5: One luma model with IPB and/or skip for filtering intra and inter luma slices) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Liqiang Wang | Email: | liqiangwang@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results for EE1-1.5, which proposes one luma model with block type information and/or block skip information to perform the filtering process for both intra luma and inter luma slices.

It is reported that the full test results are not an exact match, while mentioning small differences on BD-rate caused by the different GCC versions. GCC-8.1.0 is used in the crosscheck tests while GCC-7.4.0 is used in EE1-1.5 tests.

The comparison in the similar GCC versions (GCC-8.1.0 and GCC-8.2.0) is further conducted in terms of Class D for all the EE1-1.5 tests, then it can exactly match in AI and LD while mentioning some cases in RA still have minor differences which are probably due to the rounding difference, because the similar difference can also be observed in the anchor comparison.

Introduction

The following four EE1-1.5 tests [] are included into this crosscheck.

- Test 1.5.1 tests the model proposed in JVET-AA0090 which has two additional inputs IPB and skip.

- Test 1.5.2 tests the model with one additional input IPB.

- Test 1.5.3 tests the model with one additional input skip.

- Test 1.5.4 tests the original inter luma model in JVET-AA0111, i.e., the model in Test 1.5.1 without IPB or skip.

SADL is used as the inference library, and the model precision is float32.

Results

All the experiments are performed based on the common test conditions in []. The full test results generated by GCC-8.1.0 are shown in following tables.

Results of EE1-1.5.1

Table 1. Performance of EE1-1.5.1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -8.68% | -16.18% | -18.70% | 281% | 100154% |
| Class A2 | -9.86% | -20.24% | -17.37% | 266% | 93823% |
| Class B | -9.09% | -23.04% | -21.84% | 272% | 95543% |
| Class C | -10.61% | -21.57% | -22.39% | 221% | 84090% |
| Class E |  |  |  |  |  |
| Overall | -9.57% | -20.72% | -20.46% | 258% | 92881% |
| Class D | -12.66% | -23.29% | -23.98% | 210% | 76727% |
| Class F | -5.41% | -11.72% | -10.68% | 379% | 44176% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -7.08% | -12.40% | -17.00% | 277% | #NUM! |
| Class A2 | -8.09% | -18.21% | -15.25% | 270% | 93821% |
| Class B | -7.25% | -15.90% | -13.88% | 263% | 90899% |
| Class C | -9.13% | -14.61% | -14.63% | 219% | 80715% |
| Class E | -8.74% | -16.40% | -17.58% | 515% | 76943% |
| Overall | -8.25% | -15.33% | -14.21% | 293% | 83803% |
| Class D | -10.98% | -18.52% | -17.89% | 207% | 72469% |
| Class F | -5.27% | -8.87% | -6.20% | 380% | 48379% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.51% | -17.96% | -19.76% | 243% | 68202% |
| Class A2 | -6.68% | -20.34% | -17.57% | 185% | 54612% |
| Class B | -6.66% | -20.94% | -20.93% | 172% | 50864% |
| Class C | -7.43% | -18.65% | -21.65% | 144% | 34176% |
| Class E | -10.02% | -20.81% | -20.63% | 181% | 57325% |
| Overall | -7.37% | -19.81% | -20.29% | 179% | 50474% |
| Class D | -7.33% | -19.32% | -21.24% | 135% | 30460% |
| Class F | -5.11% | -11.77% | -10.93% | 132% | 41462% |

Results of EE1-1.5.2

Table 2. Performance of EE1-1.5.2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -8.73% | -16.13% | -18.70% | 269% | 92816% |
| Class A2 | -9.85% | -20.32% | -17.25% | 255% | 87526% |
| Class B | -8.99% | -23.03% | -21.81% | 260% | 87964% |
| Class C | -10.51% | -21.63% | -22.35% | 212% | 77706% |
| Class E |  |  |  |  |  |
| Overall | -9.52% | -20.73% | -20.42% | 247% | 85936% |
| Class D | -12.66% | -23.35% | -23.96% | 202% | 70987% |
| Class F | -5.36% | -11.70% | -10.63% | 361% | 38166% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -7.17% | -12.89% | -17.31% | 265% | 94191% |
| Class A2 | -8.06% | -18.05% | -15.18% | 258% | 86558% |
| Class B | -7.19% | -15.97% | -14.12% | 252% | 83885% |
| Class C | -9.09% | -14.73% | -14.81% | 210% | 74756% |
| Class E | -8.42% | -16.84% | -18.22% | 487% | 68144% |
| Overall | -8.13% | -15.42% | -14.43% | 280% | 76637% |
| Class D | -11.01% | -19.17% | -17.48% | 200% | 66939% |
| Class F | -5.18% | -8.79% | -5.78% | 360% | 42426% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.60% | -17.59% | -19.68% | 236% | 63939% |
| Class A2 | -6.72% | -20.23% | -17.38% | 180% | 51174% |
| Class B | -6.66% | -21.12% | -21.14% | 169% | 47228% |
| Class C | -7.38% | -18.58% | -21.69% | 142% | 32097% |
| Class E | -10.07% | -20.80% | -20.64% | 177% | 53696% |
| Overall | -7.39% | -19.77% | -20.31% | 175% | 47201% |
| Class D | -7.32% | -19.09% | -21.04% | 133% | 28382% |
| Class F | -5.06% | -11.77% | -10.93% | 130% | 38332% |

Results of EE1-1.5.3

Table 3. Performance of EE1-1.5.3

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -8.50% | -16.04% | -18.70% | 270% | 93405% |
| Class A2 | -9.67% | -20.27% | -17.37% | 256% | 87519% |
| Class B | -8.92% | -23.06% | -21.78% | 262% | 88894% |
| Class C | -10.38% | -21.67% | -22.41% | 213% | 78330% |
| Class E |  |  |  |  |  |
| Overall | -9.38% | -20.73% | -20.45% | 248% | 86530% |
| Class D | -12.56% | -23.17% | -23.86% | 203% | 71688% |
| Class F | -5.20% | -11.69% | -10.64% | 363% | 39840% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -7.31% | -12.58% | -17.10% | 265% | #NUM! |
| Class A2 | -8.10% | -18.09% | -15.26% | 259% | #NUM! |
| Class B | -7.34% | -15.72% | -13.84% | 252% | 83883% |
| Class C | -9.08% | -14.71% | -14.70% | 211% | 74963% |
| Class E | -8.52% | -17.26% | -17.86% | 487% | 67120% |
| Overall | -8.21% | -15.27% | -14.22% | 280% | 76418% |
| Class D | -10.93% | -19.11% | -18.22% | 200% | 67688% |
| Class F | -5.21% | -9.21% | -6.06% | 361% | 43422% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.59% | -17.61% | -19.72% | 237% | 63595% |
| Class A2 | -6.67% | -20.27% | -17.47% | 181% | 51221% |
| Class B | -6.65% | -21.54% | -21.34% | 169% | 47580% |
| Class C | -7.38% | -18.63% | -21.69% | 142% | 32039% |
| Class E | -9.97% | -20.83% | -20.62% | 178% | 53926% |
| Overall | -7.36% | -19.91% | -20.38% | 175% | 47278% |
| Class D | -7.31% | -19.21% | -21.15% | 134% | 28596% |
| Class F | -5.07% | -11.77% | -10.92% | 130% | 39177% |

Results of EE1-1.5.4

Table 4. Performance of EE1-1.5.4

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -8.15% | -16.18% | -18.80% | 252% | 82167% |
| Class A2 | -9.48% | -20.19% | -17.32% | 239% | 77560% |
| Class B | -8.65% | -23.08% | -21.81% | 245% | 77947% |
| Class C | -10.23% | -21.55% | -22.35% | 201% | 69074% |
| Class E |  |  |  |  |  |
| Overall | -9.14% | -20.72% | -20.45% | 232% | 76199% |
| Class D | -12.49% | -23.32% | -23.95% | 192% | 63377% |
| Class F | -5.23% | -11.75% | -10.67% | 335% | 35008% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -7.08% | -12.60% | -17.29% | 247% | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -7.17% | -15.85% | -14.20% | 236% | 74419% |
| Class C | -8.95% | -14.93% | -14.89% | 198% | 66060% |
| Class E | -8.65% | -17.39% | -17.86% | 443% | 60397% |
| Overall | -8.13% | -15.44% | -14.51% | 261% | 67884% |
| Class D | -10.89% | -19.19% | -17.75% | 189% | 59550% |
| Class F | -5.23% | -9.21% | -5.46% | 335% | 37378% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.50% | -17.94% | -19.79% | 224% | 56466% |
| Class A2 | -6.67% | -20.32% | -17.52% | 174% | 45542% |
| Class B | -6.63% | -21.14% | -21.12% | 164% | 42266% |
| Class C | -7.36% | -18.62% | -21.70% | 139% | 28636% |
| Class E | -9.97% | -20.80% | -20.61% | 171% | 48174% |
| Overall | -7.33% | -19.85% | -20.34% | 169% | 42098% |
| Class D | -7.30% | -19.18% | -21.12% | 132% | 25531% |
| Class F | -5.10% | -11.77% | -10.94% | 128% | 35085% |

Reference

[] D. Liu, J. Ström, M. Damghanian, P. Wennersten, K. Andersson. EE1-1.5: One luma model with IPB and/or skip for filtering intra and inter luma slices. JVET-AB0052, 28th Meeting, Mainz, DE, 20–28 October 2022.

[] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AA2016, 27th Meeting, by teleconference, 13–22 July 2022.