| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0096-v1 |
| --- | --- |

| Title: | AHG9: Resolution Change Information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon Monzastr. 4c 63225 Langen, Germany Kiyofumi Abe Tadamasa Toma | Tel: Email: | +49 6103-5996407 virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

Dynamic resolution encoding can be used for dynamically adapting the resolution of a video on a scene-by-scene basis to reach the desired bitrate while preserving the quality. However, many receivers have been developed with the assumption that the picture resolution within a bitstream stays constant, for example within an MPEG-DASH Representation.

It is suggested to consider signalling of dynamic resolution encoding in bitstreams, together with the resolutions used. The authors of the present document believe that such a signalling would simplify implementation of receivers in environments where bitstreams may use dynamic resolution encoding.

This input is a follow-up of JVET-AA0091 attempting to address the comments made at the last meeting not to make any promises and guarantees regarding the bitstream within an SEI message.

# Introduction: Dynamic Resolution Encoding

Using a dynamic resolution can improve the coding efficiency of some video content, as JVET has observed with EE1-related experiments using RPR (Reference Picture Resampling) dynamically (see JVET-Z0065 [1]). Streaming services, such as Netflix, are starting to explore the advantages of selecting the resolution to encode a content based on the properties of that content [2]. Since most real-life content is made of different scenes with different video signal characteristics, dynamic resolution encoding may become beneficial to achieve a constant bitrate for video content, while preserving the quality, whereby complex scenes may use a down-sampled resolution, while static scenes may use the maximum resolution. Dynamic resolution encoding on a scene-by-scene basis can be used with older video coding standards, such as AVC and HEVC, by inserting an IDR picture and changing the resolution signalled in the Sequence Parameter Set (SPS). VVC is even more flexible due to RPR (Reference Picture Resampling) which allows changing the resolution on a picture-by-picture basis. While it is likely that dynamic resolution encoding will be used on a scene-by-scene basis rather than on a picture-by-picture basis for streaming, the capabilities of VVC and general awareness of this possibility may increase its usage in coming streaming (and potentially even broadcast) services.

A decoder can deal with any video resolution, as long as the bitstreams follow the level that the decoder supports. However, in the streaming and broadcast industries, the decoder itself is only one component of a receiver. Post-processing modules in many current implementations tend to assume that the pictures coming out of the Decoded Picture Buffer all have the same resolution.

In streaming protocols, such as MPEG-DASH, a piece of content is typically encoded several times with different bitrates and resolutions. Each encoded bitstream is a Representation made of Segments that are aligned across all Representations, so that switching between Segments is possible in order for an MPEG-DASH client to adapt to the available bandwidth. Due to that inherent capability, MPEG-DASH receivers are often designed to deal with changes of resolutions at the output of the decoder. Unfortunately, this capability has often been implemented by MPEG-DASH receivers signalling to the post-processing and display modules that a switch between different Representations has occurred and what the ratio of the resolution change is, as illustrated in Figure 1. Many MPEG-DASH receivers therefore rely on the fact that the resolution stays constant within one Representation. However, it may not be the case in future streaming services if dynamic resolution encoding is applied within a Representation to improve coding efficiency.

Figure 1: Possible implementation of an MPEG-DASH receiver

When dynamic resolution encoding is used, some form of signalling of which resolutions are to be expected in a bitstream are necessary for a receiver to be able to schedule post-processing resources, in particular for up-sampling. For example, DVB has restricted the resolution changes allowed within a VVC bitstream (including within a DVB-DASH Representation), as documented in DVB BlueBook A001 [3] clause 5.15.2.3.2:

Encoding:			VVC Bitstreams may use reference picture resampling signalled by setting sps_ref_pic_resampling_enabled_flag equal to 1 and sps_res_change_in_clvs_allowed_flag equal to 1 in the referred SPS. 

The resolution of a VVC Bitstream may be modified at a VVC DVB_RAP without the use of reference picture resampling. 

If reference picture resampling is used in a VVC Bitstream or if the resolution of a VVC Bitstream is changed at a VVC DVB_RAP, only changes between the following resolutions shall be used:
-	3840x2160 and 2560x1440
-	3840x2160 and 1920x1080
-	2560x1440 and 1920x1080
-	2560x1440 and 1280x720
-	1920x1080 and 1280x720
-	1920x1080 and 960x540

If reference picture resampling is used in the VVC Bitstream, the values of syntax elements pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples in referred PPSs shall only change at a VVC DVB_RAP.

The interval between two successive changes of pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples in referred PPSs shall be greater than or equal to 5 seconds.

However, the authors of the present document believe that the industry would benefit from a more general signalling mechanism for dynamic resolution encoding.

Compared to JVET-AA0091 [4], the semantics of the proposed Resolution Change Information SEI message has been re-written in the present document to make sure that no promises are made about the bitstream. Some information, such as the exact location of each resolution used within the bitstream, has also been removed from the version proposed in the present document, as such information may not always be easily available to an encoder in advance and would make too many promises about the bitstream.

# Proposed Resolution Change Information SEI message syntax

The authors of the present document suggest considering a Resolution Change Information SEI message that would signal the following characteristics for a whole bitstream:

- A flag to signal if the picture resolution is expected to be constant within the bitstream or not

- Resolutions in luma samples that may be present in the bitstream, possibly including a conformance cropping window

- Chroma format of the resolutions that may be present in the bitstream

An SEI message is easy to find in a bitstream and easy to parse which makes the information that it conveys more readily available than the information signalled across several parameters sets within the bitstream. The following Resolution Change Information SEI message is proposed for HEVC and VVC.

| resolution_change_information( payloadSize ) { | Descriptor |
| --- | --- |
| rci_constant_resolution_flag | u(1) |
| if( rci_constant_resolution_flag == 0 ) { |  |
| rci_number_of_resolutions | ue(v) |
| for( i = 0; i < rci_number_of_resolutions; i++ ) { |  |
| rci_width_in_luma_samples[ i ] | ue(v) |
| rci_height_in_luma_samples[ i ] | ue(v) |
| rci_chroma_format_idc[ i ] | u(2) |
| rci_conformance_window_flag[ i ] | u(1) |
| if ( rci_conformance_window_flag[ i ] ) { |  |
| rci_conf_win_left_offset[ i ] | ue(v) |
| rci_conf_win_right_offset[ i ] | ue(v) |
| rci_conf_win_top_offset[ i ] | ue(v) |
| rci_conf_win_bottom_offset[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

# Proposed Resolution Change Information SEI message semantics

The resolution change information SEI message conveys information about a bitstream being expected (i.e. likely) to use dynamic resolution encoding or not as well as a list of picture resolutions that are expected (i.e. likely) to be present in the encoded bitstream when the bitstream is expected to use dynamic resolution encoding. There is no guaranty that the picture resolutions listed in the resolution change information SEI message are indeed present within the encoded bitstream.

The content of a resolution change information SEI message may, for example, be used by transport-layer or systems-layer processing elements to determine whether the bitstream is suitable for delivery to a receiving system, based on whether the receiving system can properly process the bitstream to enable an adequate user experience or whether the bitstream satisfies the application needs.

When a resolution change information SEI message is present in any access unit of a bitstream, a resolution change information SEI message shall be present in the first access unit of any CVS of the bitstream. When there are multiple resolution change information SEI messages present in a bitstream, they shall have the same content.

rci_constant_resolution_flag equal to 1 specifies that the picture resolution is expected (i.e. likely) to stay constant within the bitstream and that no other syntax elements are signalled in the resolution change information SEI message. rci_constant_resolution_flag equal to 0 specifies that more than one picture resolution are expected be used (although not guaranteed to be used) within the bitstream and that information about resolutions that are expected to be present in the bitstream is signalled in the resolution change information SEI message.

rci_number_of_resolutions specifies the number of resolutions of coded pictures for which information is provided in the resolution change information SEI message.

rci_width_in_luma_samples[ i ] specifies the width, in units of luma samples, of the i-th resolution for which information is provided in the resolution change information SEI message.

rci_height_in_luma_samples[ i ] specifies the height, in units of luma samples, of the i-th resolution for which information is provided in the resolution change information SEI message.

The resolutions indicated by the pair of values (rci_width_in_luma_samples[ m ], rci_height_in_luma_samples[ m ]) and (rci_width_in_luma_samples[ n ], rci_height_in_luma_samples[ n ]) shall not be identical when m is not equal to n.

rci_chroma_format_idc[ i ] specifies the chroma sampling relative to the luma sampling, for the i-th resolution for which information is provided in the resolution change information SEI message, as specified in the Table below:

| sps_chroma_format_idc | Chroma format | SubWidthC | SubHeightC |
| --- | --- | --- | --- |
| 0 | Monochrome | 1 | 1 |
| 1 | 4:2:0 | 2 | 2 |
| 2 | 4:2:2 | 2 | 1 |
| 3 | 4:4:4 | 1 | 1 |

rci_conformance_window flag[ i ] equal to 1 indicates that the conformance cropping window offset parameters for the i-th resolution for which information is provided follow next in the resolution change information SEI message. rci_conformance_window_flag equal to 0 indicates that the conformance cropping window offset parameters are not present for the i-th resolution for which information is provided in the resolution change information SEI message.

rci_conf_win_left_offset[ i ], rci_conf_win_right_offset[ i ], rci_conf_win_top_offset[ i ] and rci_conf_win_bottom_offset[ i ] specify the cropping window that is applied, in terms of a rectangular region that is output, for the i-th resolution for which information is provided in the resolution change information SEI message. When rci_conformance_window_flag[ i ] is equal to 0, the values of rci_conf_win_left_offset[ i ], rci_conf_win_right_offset[ i ], rci_conf_win_top_offset[ i ] and rci_conf_win_bottom_offset[ i ] are inferred to be equal to 0 for the i-th resolution for which information is provided in the resolution change information SEI message.

Pictures with resolution i output from the Decoded Picture Buffer (i.e. after conformance cropping) are expected to have a width in luma samples equal to rci_width_in_luma_samples[ i ] – SubWidthC * (rci_conf_win_left_offset[ i ] + rci_conf_win_right_offset[ i ]), and a height in luma samples equal to rci_height_in_luma_samples[ i ] * SubHeightC * (rci_conf_win_top_offset[ i ] + rci_conf_win_bottom_offset[ i ]).

# Proposed text changes for VVC

[Ed.: Add the following rows to the table specifying sei_payload() in subclause D.2.1.]

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| […] |  |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| […] |  |
| else if( payloadType = = 212 [Ed.: or next available number] ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| resolution_change_information( payloadSize ) |  |
| […] |  |
| } |  |

# Proposed text changes for VSEI

[Ed.: In clause 8.1, add the following row to the table on persistence scope of SEI messages.]

| SEI message | Persistence scope |
| --- | --- |
| Resolution change information | The remainder of the bitstream |

[Ed.: Add new subclause 8.30 as follows.]

8.30	Resolution change information SEI message

8.30.1	Resolution change information SEI message syntax

[Ed.: Copy the table with the syntax from section 2 of the present document.]

8.30.2	Resolution change information SEI message semantics

[Ed.: Copy the text from section 3 of the present document]

# Proposed text changes for HEVC

[Ed.: Add the following rows to the table specifying sei_payload() in subclause D.2.1.]

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| […] |  |
| else if( payloadType = = 212 [Ed: or next available number] ) |  |
| resolution_change_information( payloadSize ) |  |
| […] |  |
| } |  |

[Ed.: Add new clause D.2.49 as follows and renumber current subclause D.2.49 (reserved SEI message syntax) as D.2.50.]

D.2.49	Resolution change information SEI message syntax

[Ed.: Copy the table with the syntax from section 2 of the present document.]

[Ed.: In clause D.3.1, add the following row to the table on persistence scope of SEI messages.]

| SEI message | Persistence scope |
| --- | --- |
| Resolution change information | The remainder of the bitstream |

[Ed.: Add new clause D.3.49 as follows and renumber current subclause D.3.49 (reserved SEI message semantics) as D.3.50.]

D.3.49	Resolution change information SEI message semantics

[Ed.: Copy the text from section 3 of the present document]

# Conclusion

The present document points out the current challenges for many existing receivers when using dynamic resolution encoding in bitstreams, including within a DASH Representation. A Resolution Change Information SEI message is proposed to signal that a bitstream is expected (i.e. likely) to use dynamic resolution encoding, together with a list of different resolutions that may be present within a bitstream. The Resolution Change Information SEI message is proposed for inclusion in the HEVC and VVC video coding standards.

# References

- J. Nam, S. Yoo, J. Lim, S. Kim, “EE1-2.1: RPR encoder with multiple scale factors”, JVET-Z0065, 26th Meeting, by teleconference, 20–29 April 2022

- Anne Aaron, Zhi Li, Megha Manohara, Jan De Cock and David Ronca, “Per-Title Encode Optimization”, December 2015, https://netflixtechblog.com/per-title-encode-optimization-7e99442b62a2

- DVB BlueBook A001r20, “Specification for the use of Video and Audio Coding in Broadcast and Broadband Applications”, July 2022, https://dvb.org/wp-content/uploads/2022/02/A001r20_Use-of-Video-and-Audio-Coding-in-Broadcast-and-Broadband-Applications_Draft_TS_101-154-v281_July-2022.pdf

- V. Drugeon, K. Abe, T. Toma, “AHG9: Resolution Change Information SEI message”, JVET-AA0091, 27th meeting, by teleconference, 13–22 July 2022

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).