| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0106-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AB0077 (EE1-2.2: CNN Filter for Super-Resolution with RPR functionality in VVC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Renjie Chang | Email: | renjiechang@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results for EE1-2.2, which proposes the convolutional neural network (CNN)-based super-resolution (SR) filter with existing RPR functionality in VVC. Only one crosscheck test for class A1 and A2 was carried out to test the performance of CNN-based super resolution compared to VTM-11.0_NNVC-2.0.

It is reported that the test results can match very well for AI configuration. For RA configuration, the test results can match with a minor difference of 0.000x, and this is probably due to float inference.

Introduction

The JVET-AA-EE1-2.2 described in JVET-AB0077 [] study a CNN-based super-resolution filter with existing RPR functionality in VVC, where both VTM-11.0_NNVC-2.0 and JVET-Z0065 are used as anchors for comparison. In this crosscheck report, the performance of proposed method in [1] was tested compare to VTM-11.0 NNVC-2.0.

Results

All the experiments are performed following the common test conditions in [], and the crosscheck results are shown in the following tables. The inference framework is libtorch1.8.1 and the model uses float precision.

Table 1. Performance of EE1-2.1 for AI

|  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0_nnvc-2.0 (QP 22, 27, 32, 37, 42) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Cass A1 | Tango2 | -9.45% | -17.31% | -15.16% | 227% | 31% |
|  | FoodMarket4 | -4.48% | -6.66% | -6.73% |  |  |
|  | Campfire | -12.89% | 149.67% | -23.80% |  |  |
| Class A2 | CatRobot1 | -7.72% | -11.63% | -9.81% | 139% | 31% |
|  | DaylightRoad2 | -2.71% | -22.70% | -19.24% |  |  |
|  | ParkRunning3 | -13.80% | 21.27% | -0.95% |  |  |
| Average on A1 |  | -8.94% | 41.90% | -15.23% | 227% | 31% |
| Average on A2 |  | -8.08% | -4.35% | -10.00% | 139% | 31% |
| overall |  | -8.51% | 18.77% | -12.62% | 183% | 31% |

Table 2. Performance of EE1-2.1 for RA

|  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0_nnvc-2.0 (QP 22, 27, 32, 37, 42) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Cass A1 | Tango2 | -7.04% | -25.37% | -15.49% | 130% | 27% |
|  | FoodMarket4 | -6.03% | -9.59% | -11.33% |  |  |
|  | Campfire | -17.36% | 38.67% | -23.00% |  |  |
| Class A2 | CatRobot1 | 3.91% | -10.72% | -2.95% | 120% | 26% |
|  | DaylightRoad2 | 13.44% | -18.60% | -15.16% |  |  |
|  | ParkRunning3 | -10.29% | 58.25% | 13.38% |  |  |
| Average on A1 |  | -10.14% | 1.24% | -16.61% | 130% | 27% |
| Average on A2 |  | 2.35% | 9.65% | -1.58% | 120% | 26% |
| overall |  | -3.89% | 5.44% | -9.09% | 125% | 27% |

Reference

- S. Huang, C. Jung, Y. Liu, M. Li. EE1-2.2: CNN Filter for Super-Resolution with RPR functionality in VVC. JVET-AB0077, 28th Meeting, Mainz, DE, 20–28 October 2022.

- E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AA2016, 27th Meeting, by teleconference, 13–22 July 2022.