| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB109-v1 |
| --- | --- |

| Title: | AHG11: Content-adaptive NN post-filter with new QP normalisation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Maria Santamaria Francesco Cricri Miska M. Hannuksela Nokia Technologies Tampere, Finland | Email: | {maria.santamaria_gomez, francesco.cricri, miska.hannuksela}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes a change in the normalisation of the slice QP used in the neural-network post-filter in JVET-Z0082. Compared to VTM 11.0 NNVC 1.0, in the RA configuration, it is reported that the coding gains for class B are -5.42% (Y), -21.75% (U), -17.93% (V); and for class C are -4.00% (Y), -19.5% (U), -16.44 (V) compared to VTM 11.0 NNVC 1.0. Compared to the JVET-Z0082, in the RA configuration, it is reported that the BD-rates for class B are -0.09% (Y), -0.66% (U), -0.58% (V); and for class C are -0.01% (Y), 0.05% (U) and 0.28% (V).

# Introduction

This contribution proposes changing the normalisation of the slice QP utilised in the neural-network (NN) post-filter described in JVET-Z0082 [1]. Before, the normalisation was defined as 2(sliceQP – 42) ÷ 6. Now, the normalisation is defined as sliceQP ÷ 63.

JVET-Z0082 describes a content-adaptive NN post-filter via NN over-fitting. For this contribution, the base models are the same as those in JVET-Z0082 and the content-adaptive models are re-overfitted using the new normalisation strategy.

# NN over-fitting

| Network Information for NN-based Video Coding Tool Testing in Training Stage |  |  |
| --- | --- | --- |
| Network Information in Training Stage |  |  |
| Mandatory | GPU Type | GPU NVIDIA A100-SXM-80GB |
|  | Framework: | TensorFlow 2.3.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 200 |
|  | Batch size: | 64 |
|  | Loss function: | Weighted-MSE |
|  | Training time: | Class B: 5.5 hours Class C: 1 hour |
|  | Training data information: | JVET CTC RA mandatory sequences |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size |  |
|  | Learning rate: | 1,00E-03 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | Convert 144x144 YUV420 signal to 6 72x72 blocks. Normalize to 0~1 |
|  | Mini-batch selection process: |  |
|  | Other information: |  |
| Network Information for NN-based Video Coding Tool Testing in Inference Stage |  |  |
| Network Information in Inference Stage |  |  |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | TensorFlow 2.3.1 C++ API |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 109068 |
|  | Total Number of Parameters (All Models) | 436272 |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 1,664245605 |
|  | Multiply Accumulate (kMAC/pixel) | 34 |
| Optional |  |  |
|  | Total Conv. Layers | 35 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size |  |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling | Use padding samples if necessary |
|  | Other information: |  |

# Simulation results

The performance is measured against two anchors, using the JVET RA CTC for NNVC [2]:

- VTM 11.0 NNVC 1.0.

- JVET-Z0082.

|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class B | -5,42 % | -21,75 % | -17,93 % | -3,68 % | -21,68 % | -17,58 % | 134 % | 5422 % | 0 % |
| Class C | -4,00 % | -19,53 % | -16,44 % | -2,92 % | -18,37 % | -16,25 % | 126 % | 4678 % | 1 % |

|  | BD-rate Over JVET-Z0082 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class B | -0,09 % | -0,66 % | -0,58 % | -0,15 % | -0,36 % | -0,55 % | 102 % | 107 % | 0 % |
| Class C | -0,01 % | 0,05 % | 0,28 % | -0,01 % | 0,04 % | 0,43 % | 99 % | 100 % | 0 % |

# References

- M. Santamaria, R. Yang, F. Cricri, J. Lainema, R. G. Youvalari, H. Zhang, G. Rangu, H. R. Tavakoli, H. Afrabandpey, M. M. Hannuksela. AHG11: Content-adaptive neural network post-filter. JVET-Z0082. April 2022.

- E. Alshina, R.-L. Liao, S. Liu, A. Segall. Common Test Conditions and evaluation procedures for neural network-based video coding technology. JVET-X2016. October 2021.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).