| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0111-v3 |
| --- | --- |

| Title: | Non-EE2: Intra prediction fusion with PMPM list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gihwa Moon Dohyeon Park Yong-Uk Yoon Jae-Gon Kim (KAU) | Tel: Email: | +82-2-300-0414 ghmoon@kau.kr dhpark@kau.kr yuyoon@kau.kr jgkim@kau.ac.kr |
| Source: | Korea Aerospace University (KAU) |  |  |

_____________________________

- Abstract

This contribution proposes a intra prediction fusion method that blends the prediction of a primary MPM mode and the prediction of its proceeding mode for all intra modes in the primary MPM list except the planar mode. The proposed method was implemented on top of ECM-6.0 software, and the experimental results are summarized as follows:

- AI: x.x%, x.x%, x.x%, x% (EncT), x% (DecT)

- RA: x.x%, x.x%, x.x%, x% (EncT), x% (DecT)

Introduction

In ECM-6.0 [1], 65 angular intra prediction modes and wide-angle intra prediction modes are supported as well as the Planar and DC mode as in VVC. A general MPM list with 22 mode entries is constructed first, and then the first 6 entries in this general MPM list constitutes the primary MPM (PMPM) list. The first entry in the general MPM list is the Planar mode. The remaining entries consist of the intra modes of neighbouring blocks. The secondary MPM list is also constructed based on the remining entries from the general MPM list. This contribution introduces a new fusion-based prediction mode that combines the prediction of an MPM mode and the prediction of its proceeding mode.

Proposed method

This contribution proposes to generate additional intra prediction by blending adjacent angular modes’ predictions, as follows:

Pfusion=PPMPM*w0+PPMPM-1*w1≫12,

where PPMPM is the intra prediction using the current PMPM mode, and PPMPM-1 is the intra prediction using the its proceeding mode. In the experiments, the both predictions are averaged with w0=12,  w1=12.

The proposed intra prediction fusion is applied to all intra modes in the PMPM list except the Planar mode that does not have proceeding mode. The fusion is applied to luma blocks, and it is not applied for MRL and ISP coded blocks.

A flag is added in the mode signaling to indicate whether the current CU is a fusion mode or not. That is, for the luma blocks, if the MPM flag is equal to ‘1’, and the current block is neither ISP nor MRL coded block, a fusion flag is signaled to indicate whether a fusion mode or not. If the fusion flag equals to ‘1’, then a PMPM index is signaled to indicate which entry of the PMPM list is selected without planar flag.

Experimental results

The proposed intra prediction fusion method was implemented on top of ECM6.0, and evaluated with ECM CTC configurations [2]. The experiments were conducted on the platform of Intel Xeon Gold 6256 (3.6GHz) and 256GB RAM with GCC 9.4.0 compiler under Ubuntu 20.04.4 LTS. The experimental results for AI and RA configurations are summarized in Table 1.

Table 1. Experimental results of the proposed method over ECM6.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.12% | -0.03% | 112% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.10% | 0.04% | 0.10% | 116% | 100% |
| Class C | -0.10% | -0.01% | -0.05% | 116% | 100% |
| Class E | -0.17% | -0.10% | -0.13% | 115% | 98% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.07% | 0.00% | -0.06% | 116% | 96% |
| Class F | -0.06% | -0.05% | 0.10% | 107% | 100% |
| Class TGM | -0.02% | 0.03% | -0.04% | 106% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

Conclusion

This contribution proposes an intra prediction fusion method that blends the prediction of a intra mode in the primary MPM list and the prediction of its proceeding mode. It is reported that Y: x.x%, U: x.x%, V: x.x% with EncT: xxx% and DecT: xxx% for AI CTC. It is recommended to include the proposed method into the next round of EE2 for further investigation.

References

- ECM-6.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-6.0.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Feb. 2022.

Patent rights declaration(s)

Korea Aerospace University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).