| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0119-v1 |
| --- | --- |

| Title: | Non-EE2: Gradient and location based convolutional cross-component model (GL-CCCM) for intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ramin G. Youvalari Pekka Astola Jani Lainema | Email: | ramin.ghaznavi-youvalari@nokia.com, pekka.astola@nokia.com, jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes a gradient and location based convolutional cross-component model (GL-CCCM) for chroma prediction to improve compression efficiency of ECM. The proposed 7-tap convolutional filter maps luma values into chroma values when a GL-CCCM prediction mode is activated by a PU level flag. The filter input consists of one spatial luma sample, two gradient values, two location information, a nonlinear term, and a bias term. Filter coefficients are derived for each chroma block separately using regression based MSE minimization (i.e., the same solver as CCCM) on reference samples in the PU’s neighborhood. The impact on coding efficiency and runtimes over ECM-6.0 is reportedly {for Y, U, V, EncT, DecT}:

AI { -0.06%, -0.77%, -0.80%, 10x%, 10x%}, RA { -0.xx%, -0.xx%, -0.xx%, 10x%, 10x%}

Furthermore, the tool has been extended to support different templates as in EE2-1.13 using full template, top-only template, and left-only template. The results on top of EE software show further coding performance increase of the EE proposals.

Results on top of EE2-1.13. A:

AI { -0.08%, -1.53%, -1.58%, 10x%, 10x%}, RA { -0.xx%, -0.xx%, -0.xx%, 10x%, 10x%}

Results on top of EE2-1.13. B:

AI { -0.09%, -1.73%, -1.69%, 10x%, 10x%}, RA { -0.xx%, -0.xx%, -0.xx%, 10x%, 10x%}

# Studied approach

## Introduction

Convolutional cross-component model (CCCM) [1] predicts chroma samples from reconstructed luma samples using a 7-tap filter. Also, similarly to CCLM, there is an option of using a single model or multi-model variant of CCCM. The multi-model variant uses two models, one model derived for samples above the average luma reference value and another model for the rest of the samples (following the spirit of the CCLM design). Multi-model CCCM mode can be selected for PUs which have at least 128 reference samples available.

The CCCM filter consist of a 5-tap plus sign shape spatial component, a nonlinear term and a bias term. The input to the spatial 5-tap component of the filter consists of a center (C) luma sample which is collocated with the chroma sample to be predicted and its above/north (N), below/south (S), left/west (W) and right/east (E) neighbors as illustrated below.

Figure 1: Spatial part of the convolutional filter

The nonlinear term P is represented as power of two of the center luma sample C and scaled to the sample value range of the content:

P = ( C*C + midVal ) >> bitDepth

That is, for 10-bit content it is calculated as:

P = ( C*C + 512 ) >> 10

The bias term B represents a scalar offset between the input and output (similarly to the offset term in CCLM) and is set to middle chroma value (512 for 10-bit content).

Output of the filter is calculated as a convolution between the filter coefficients ci and the input values and clipped to the range of valid chroma samples:

predChromaVal = c0C + c1N + c2S + c3E + c4W + c5P + c6B

## Proposed Gradient and Location based convolutional cross-component model (GL-CCCM)

The proposed GL-CCCM method uses gradient and location information instead of the 4 spatial neighbor samples in the CCCM filter. The GL-CCCM filter for the prediction is:

predChromaVal = c0C + c1Gy + c2Gx + c3Y + c4X + c5P + c6B

Where Gy and Gx are the vertical and horizontal gradients, respectively, and are calculated as:

Gy = (2N + NW + NE) – (2S + SW + SE)

Gx = (2W + NW + SW) – (2E + NE + SE)

Moreover, the Y and X parameters are the vertical and horizontal locations of the center luma sample and they are calculated with respect to the top-left coordinates of the block.

The rest of the parameters are the same as CCCM tool. The reference area for the parameter calculation is the same as CCCM method.

Figure 2: Spatial samples used for GL-CCCM

## Bitstream signalling

Usage of the mode is signalled with a CABAC coded PU level flag. One new CABAC context was included to support this. When it comes to signalling, GL-CCCM is considered a sub-mode of CCCM. That is, the GL-CCCM flag is only signalled if original CCCM flag is true.

## Encoder operation

The encoder performs two new RD checks in the chroma prediction mode loop, one for checking single model GL-CCCM mode and one for checking multi-model GL-CCCM mode.

# Experimental results

Tables below summarize the coding efficiency and runtime impact of the proposed method in ECM 6.0 using the JVET common test conditions [2].

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12 % | -1.15 % | -1.20 % |  |  |
| Class A2 | -0.05 % | -1.23 % | -1.31 % |  |  |
| Class B | -0.05 % | -0.87 % | -0.87 % |  |  |
| Class C | -0.03 % | -0.34 % | -0.48 % |  |  |
| Class E | -0.06 % | -0.35 % | -0.19 % |  |  |
| Overall | -0.06 % | -0.77 % | -0.80 % |  |  |
| Class D | -0.01 % | -0.25 % | -0.42 % |  |  |
| Class F | -0.13 % | -0.75 % | -0.94 % |  |  |

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class B | -0.03 % | -0.43 % | -0.66 % |  |  |
| Class C | -0.02 % | -0.19 % | -0.10 % |  |  |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | 0.00 % | -0.30 % | 0.05 % |  |  |
| Class F | -0.08 % | -0.47 % | -0.70 % |  |  |

# Experiments on top on EE2-1.13

EE2-1.13 [3] proposes two extensions for CCCM:

EE2-1.13. A: Extend the CCLM template selection to CCCM to use left, top or top and left templates for model derivation.

EE2-1.13. B: MMLM fusion is replaced with multi-model CCCM fusion. More specifically, the predictors of a non-CCLM chroma mode can be fused with the predictors of multi-model CCCM mode. The derivation of fusion weights is kept unchanged.

Similar to EE2-1.13. A, GL-CCCM is extended to also cover different template selections. To this end, 6 modes are defined for GL-CCCM method in which the tool can use top or left or top and left templates in both single-model and multi-model cases.

Tables below summarize the results of the proposed method on top of EE2-1.13. A:

All intra results over ECM-6.0

|  | EE2-1.13A |  |  |  |  | Proposed on top of EE2-1.13A |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.03 % | -1.14 % | -0.90 % | 101 % | 101 % | -0.11 % | -1.93 % | -2.03 % |  |  |
| Class A2 | -0.03 % | -1.58 % | -1.86 % | 102 % | 100 % | -0.08 % | -2.50 % | -2.83 % |  |  |
| Class B | -0.05 % | -1.21 % | -1.06 % | 102 % | 100 % | -0.10 % | -1.88 % | -1.84 % |  |  |
| Class C | -0.03 % | -0.39 % | -0.45 % | 102 % | 98 % | -0.06 % | -0.72 % | -0.84 % |  |  |
| Class E | -0.04 % | -0.52 % | -0.39 % | 101 % | 100 % | -0.07 % | -0.67 % | -0.45 % |  |  |
| Overall | -0.04 % | -0.96 % | -0.92 % | 101 % | 100 % | -0.08 % | -1.53 % | -1.58 % |  |  |
| Class D | -0.02 % | -0.23 % | -0.47 % | 102 % | 96 % | -0.02 % | -0.67 % | -0.80 % |  |  |
| Class F | -0.06 % | -0.85 % | -0.76 % | 99 % | 100 % | -0.17 % | -1.28 % | -1.37 % |  |  |

Random access results over ECM-6.0

|  | EE2-1.13A |  |  |  |  | Proposed on top of EE2-1.13A |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.06 % | -0.38 % | -0.66 % | 101 % | 100 % |  |  |  |  |  |
| Class A2 | -0.04 % | -0.87 % | -1.00 % | 101 % | 101 % |  |  |  |  |  |
| Class B | -0.04 % | -0.91 % | -1.08 % | 99 % | 99 % |  |  |  |  |  |
| Class C | -0.02 % | -0.34 % | -0.26 % | 100 % | 99 % |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.04 % | -0.64 % | -0.76 % | 100 % | 100 % |  |  |  |  |  |
| Class D | 0.01 % | -0.39 % | -0.70 % | 99 % | 98 % |  |  |  |  |  |
| Class F | -0.01 % | -0.71 % | -0.62 % | 99 % | 99 % |  |  |  |  |  |

Tables below summarize the results of the proposed method on top of EE2-1.13. B:

All intra results over ECM-6.0

|  | EE2-1.13B |  |  |  |  | Proposed on top of EE2-1.13B |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.05 % | -1.33 % | -1.02 % | 100 % | 99 % | -0.11 % | -2.05 % | -2.03 % |  |  |
| Class A2 | -0.02 % | -1.75 % | -2.06 % | 101 % | 102 % | -0.08 % | -2.68 % | -2.97 % |  |  |
| Class B | -0.07 % | -1.36 % | -1.22 % | 100 % | 100 % | -0.11 % | -2.17 % | -1.92 % |  |  |
| Class C | -0.04 % | -0.33 % | -0.45 % | 101 % | 100 % | -0.06 % | -0.64 % | -0.84 % |  |  |
| Class E | -0.04 % | -0.99 % | -0.95 % | 100 % | 100 % | -0.07 % | -1.17 % | -0.81 % |  |  |
| Overall | -0.05 % | -1.13 % | -1.11 % | 101 % | 100 % | -0.09 % | -1.73 % | -1.69 % |  |  |
| Class D | -0.03 % | -0.42 % | -0.49 % | 102 % | 101 % | -0.03 % | -0.62 % | -0.71 % |  |  |
| Class F | -0.09 % | -0.64 % | -0.45 % | 99 % | 99 % | -0.21 % | -1.35 % | -1.60 % |  |  |

Random access results over ECM-6.0

|  | EE2-1.13B |  |  |  |  | Proposed on top of EE2-1.13B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.05 % | -0.63 % | -0.59 % | 101 % | 101 % |  |  |  |  |  |
| Class A2 | -0.02 % | -0.95 % | -1.10 % | 100 % | 101 % |  |  |  |  |  |
| Class B | -0.04 % | -0.95 % | -1.11 % | 99 % | 99 % |  |  |  |  |  |
| Class C | -0.02 % | -0.24 % | -0.08 % | 99 % | 99 % |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.03 % | -0.69 % | -0.73 % | 100 % | 100 % |  |  |  |  |  |
| Class D | -0.06 % | -0.25 % | -0.48 % | 99 % | 99 % |  |  |  |  |  |
| Class F | -0.12 % | -0.43 % | -0.56 % | 98 % | 98 % |  |  |  |  |  |

# Conclusion

This contribution proposes the gradient and location based CCCM method as extensions to the CCCM tool. The results show that the gain of CCCM can be further improved using the GL-CCCM method. It is suggested to study the proposed method in the next EE round.

# References

- P. Astola, J. Lainema, R. G. Youvalari, A. Aminlou, and K. Panusopone, “EE2-1.1a: Convolutional cross-component intra prediction model,” JVET-AA0057, Jul. 2022.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

- Y.-J. Chang, V. Seregin, and M. Karczewicz, “EE2-1.13: On CCCM improvement,” JVET-ABxxx, Oct. 2022.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).