| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0128-v1 |
| --- | --- |

| Title: | EE2-related: CCCM template selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P.Bordes, K.Naser, E.Francois, F.Galpin | Email: | philippe.bordes@interdigital.com |  |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In EE2 test 1.13 described in document JVET-AA2024, three types of templates for CCLM and CCCM to derive the CC-models are defined: full template, top-only template, and left-only template. It is proposed to extend the template selection to six.

It is reported that the results of the proposed method implemented on top of EE2-test-1.13b in AI are - 0.06% (Y), -1.40% (U), -1.39% (V) compared to ECM-6.0 anchor, and -0.02% (Y), -0.28% (U), -0.28% (V), 100% (EncT), 99% (DecT) compared to EE2-test-1.13b.

# Introduction

In EE2 test 1.13a, the CCLM template used to derive the CC-model can be selected among left, top or top-and-left templates, as depicted in grey in Figure 1. Test 1.13b combines test 1.13a with CCCM fusion instead of MMLM fusion [1].

Figure 1: samples used for the derivation of CCCM model.

# Proposed method

To provide more diversity in the selection of CC-models, it is proposed to extend the number of templates to 6 (Figure 2). When one of the 3 extended templates is selected, the CC-model is derived selecting the samples of the template with reconstructed luma value within the range of the reconstructed current CU luma values. If CCCM mode is selected, a flag is coded to indicate whether regular or extended templates is used.

Figure 2: 3 extended templates used for the derivation of CCCM model.

# Experimental results

The proposed method is tested on top of EE2-1.13a and EE2-1.13b software under the common test conditions [2] in AI configuration.

Table 1: performance of the proposed method over ECM-6.0 (left) and over EE2-test1.13a (right)

| AI | Proposal in EE2-1.13a vs ECM-6.0 |  |  | gain vs EE2-1.13a |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -0,05% | -1,36% | -1,48% | -0,03% | -0,22% | -0,59% | 98% | 93% |
| Class A2 | -0,06% | -1,90% | -2,42% | -0,03% | -0,32% | -0,58% | 98% | 98% |
| Class B | -0,06% | -1,54% | -1,44% | -0,01% | -0,33% | -0,38% | 102% | 99% |
| Class C | -0,03% | -0,58% | -0,62% | 0,00% | -0,19% | -0,17% | 101% | 100% |
| Class E | -0,04% | -0,72% | -0,69% | -0,01% | -0,20% | -0,30% | 105% | 107% |
| Overall | -0,05% | -1,22% | -1,30% | -0,01% | -0,26% | -0,39% | 101% | 99% |
| Class D | -0,02% | -0,44% | -0,48% | 0,00% | -0,21% | -0,01% | 103% | 105% |
| Class F | -0,09% | -1,06% | -0,82% | -0,03% | -0,20% | -0,06% | 97% | 97% |
| TGM | -0,64% | -1,26% | -1,32% | -0,23% | -0,44% | -0,44% | 97% | 96% |
| all SC | -0,25% | -0,92% | -0,87% | -0,09% | -0,28% | -0,17% | 99% | 99% |

Table 2: performance of the proposed method over ECM-6.0 (left) and over EE2-test1.13b (right)

| AI | Proposal in EE2-1.13b vs ECM-6.0 |  |  | gain vs EE2-1.13b |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -0,07% | -1,63% | -1,46% | -0,03% | -0,31% | -0,44% | 100% | 98% |
| Class A2 | -0,06% | -2,10% | -2,56% | -0,04% | -0,36% | -0,51% | 98% | 99% |
| Class B | -0,07% | -1,74% | -1,59% | -0,01% | -0,38% | -0,38% | 97% | 97% |
| Class C | -0,06% | -0,53% | -0,54% | -0,02% | -0,20% | -0,09% | 101% | 100% |
| Class E | -0,05% | -1,07% | -0,94% | -0,01% | -0,08% | 0,01% | 102% | 100% |
| Overall | -0,06% | -1,40% | -1,39% | -0,02% | -0,28% | -0,28% | 100% | 99% |
| Class D | -0,02% | -0,63% | -0,61% | 0,01% | -0,22% | -0,12% | 103% | 102% |
| Class F | -0,12% | -1,14% | -1,27% | -0,03% | -0,50% | -0,82% | 93% | 89% |
| TGM | -0,63% | -1,26% | -1,34% | -0,21% | -0,47% | -0,46% | 98% | 101% |
| all SC | -0,26% | -1,01% | -1,08% | -0,08% | -0,39% | -0,47% | 98% | 97% |

# Conclusion

This contribution proposes an extension on template selection for CCCM. The method implemented on top of EE2-test-1.13 shows that the gain of CCCM can be further improved. It is suggested to study the proposed method in the next EE round.

# References

- V. Seregin, J. Chen, G. Li, K. Naser, J. Ström, M. Wiken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AA2024, Jul. 2022.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).