| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0130 |
| --- | --- |

| Title: | EE2-1.14: IntraTMP adaptation for camera-captured content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Naser T. Poirier F. Galpin A. Robert | Email: | karam.naser@interdigital.com |  |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution presents EE2-1.14 test results of the IntraTMP adaptation for camera-captured contents. On top of ECM-6.0, simulation results are reported as follows:

- AI:  -0.26%	 -0.32%	 -0.30%	 with 102% EncT and 99% DecT.

- RA: -0.12%	 -0.09%	 -0.17%	 with 103% EncT and 104% DecT.

# Introduction

IntraTMP was adopted in ECM-1.0 and activated only for class F and TGM. For a predefined search range, the encoder searches for the most similar template to the current template in a reconstructed part of the current frame and uses the corresponding block as a prediction block. Four search regions are defined as in Figure 1.

Figure 1 Intra template matching search areas used.

# Proposed method

To adapt IntraTMP for camera-captured contents, a speed-up is proposed for this mode (as described in JVET-AA0043). Specifically, the search region (R1 to R4 in Figure 1) are sub-sampled by a factor of 2. This reduces the template matching search by a factor of 4. After finding the best match, a refinement process is performed in which another template matching search is performed around the best match with a reduced search range. The refined search range is defined as min(w,h)/2, where w and h are the current CU width and height.

# Experimental results

The proposed method is tested on top of ECM-6.0 software under the common test conditions [1] in AI and RA configurations. The following results were obtained:

Table 1: performance of the proposed method over ECM-6.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6,0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.06% | 0.06% | 104% | 100% |
| Class A2 | -0.32% | -0.51% | -0.44% | 106% | 108% |
| Class B | -0.34% | -0.30% | -0.37% | 100% | 99% |
| Class C | -0.11% | -0.14% | -0.12% | 102% | 95% |
| Class E | -0.48% | -0.63% | -0.67% | 101% | 98% |
| Overall | -0.26% | -0.32% | -0.30% | 102% | 99% |
| Class D | -0.02% | -0.05% | -0.23% | 102% | 102% |
| Class F | 0.04% | -0.08% | 0.16% | 95% | 95% |
| Class TGM | 0.06% | 0.09% | 0.05% | 89% | 79% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-6,0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.04% | 0.01% | 104% | 105% |
| Class A2 | -0.19% | -0.23% | -0.24% | 102% | 100% |
| Class B | -0.18% | -0.04% | -0.36% | 102% | 102% |
| Class C | -0.04% | -0.14% | 0.00% | 103% | 109% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.09% | -0.17% | 103% | 104% |
| Class D | 0.01% | 0.06% | 0.02% | 104% | 110% |
| Class F | 0.03% | 0.04% | -0.02% | 102% | 109% |
| Class TGM | 0.06% | 0.08% | 0.12% | 98% | 106% |

# Conclusion

It is proposed to adopt the method in ECM-7.0 reference software.

# References

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).