| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0136-v1 |
| --- | --- |

| Title: | AHG11: Complexity Reduction on Neural-Network Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jay N. Shingala, Shireesh Kadaramandalgi, Ajay Shyam Tong Shao, Arjun Arora, Peng Yin, Fangjun Pu, Taoran Lu, Sean McCarthy | Email: | {jay.shingala, shireesh.kadaramandalgi }@ittiam.com {tshao, pyin}@dolby.com |
| Source: | Ittiam Systems, Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

JVET-AA0080 proposes to use CP decomposition plus fusing adjacent 1x1 convolution to reduce the complexity of NNLF. In addition, it also proposes to split architecture for luma and chroma components between input network and output network. Simulation shows that using JVET-X0140 low complexity model (EE1-1.4.1) as baseline, whose worse case block level complexity is 33.6 KMAC/Pixel, the proposal can achieve the AI luma gain about 5% with worst case complexity of 17.7 KMAC/pixel for (24L,8C) split. This contribution applies the same techniques proposed in JVET-AA0080 and investigates the RA case. The same baseline model shows the BD-rate saving of 5.78%, 6.80% and 5.03% (Y, Cb, Cr, respectively) for RA compared to EE1 VTM anchor 1. For the proposed complexity reduction techniques with first pass training, when compared against EE1 VTM anchor 1, the BD-Rate saving for RA (Y, Cb Cr, respectively) is:

- CP decomposition with fusing: 5.06%, 7.08%, 3.20%, with worst case block level complexity of 16.2 KMAC/Pixel

- (24L, 8C) + CP decomposition with fusion: 5.31%, 9.39%, 7.80%, with worst case block level complexity of 17.7 KMAC/Pixel

# Introduction

JVET-AA0080 [1] proposes to use CP decomposition plus fusing adjacent 1x1 convolution to reduce the complexity of NNLF. In addition, it also proposes to split architecture for luma and chroma components between input network and output network. The proposed techniques shows that in AI case the luma gain is about 5% with worst case complexity of 17.7 KMAC/pixel for (24L,8C) split. The architecture retains majority of the luma gain and achieve higher chroma gain with worst case complexity further reduced to 14.2 KMAC/Pixel for (20L,8C) split and 11.3 KMAC/Pixel for (16L,8C) split, respectively.

In this contribution, further investigation is performed for RA case. The same baseline model JVET-X0140 low complexity model (EE1-1.4.1) [2] is chosen for our trial. This model has a good tradeoff on performance versus complexity. The block-level worst case complexity is 33.6 KMAC/Pixel. The simulation results indicate that EE1-1.4.1 achieved 5.78%, 6.80% and 5.03% BD-rate saving for RA and 5.16%, 5.93%, 5.19% BD-rate saving for AI, for Y, Cb and Cr components, respectively.

Figure 1 shows the baseline JVET-X0140 NNLF architecture. Figure 2 shows the split architecture combined with CP decomposition and fusing of adjacent 1x1 conv layer. The block-level complexity in KMACs/Pixel is shown in Table 1.

3x3 conv 3x3x10xM

Leaky ReLu

3x3 conv 3x3xKxL

1x1 conv 1x1xMxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

1x1 conv 1x1xKxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

BS info

QStep

Yx4+U+V 72x72x10

NxN

1x1 conv 1x1xKxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

Y’x4+U’+V‘ 64x64x6

N’xN’

n Hidden Layers

Figure 1 JVET-X0140 Baseline Model

Ky

NxN

KC

Channel Concat

LY

NxN

LC

3x3 conv 3x3x10xM

Leaky ReLu

3x3 conv 3x3xKYxLY

1x1 conv 1x1xMxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

Leaky ReLu

3x3 conv 3x3xKYxKY

1x1 conv 1x1xMYxKY

BS info

QStep

Yx4+U+V 72x72x10

NxN

1x1 conv 1x1xKYxMY

Leaky ReLu

3x3 conv 3x3xKYxKY

1x1 conv 1x1xMYxKY

Y’x4+U’+V‘ 64x64x6

N’xN’

nY Hidden Layers (Luma)

nC Hidden Layers (Chroma)

1x1 conv 1x1xMCxKC

1x1 conv 1x1xKCxMC

3x3 conv 3x3xKCxKC

1x1 conv 1x1xMCxKC

3x3 conv 3x3xKCxKC

1x1 conv 1x1xKCxMC

Leaky ReLu

3x3 conv 3x3xKCxLC

Leaky ReLu

1x1 conv 1x1xKYxMY

Layers approximated with CP decomposition

Separable Convolution layers

3x3 conv 3x3xKYxKY

Rank RY CP decomposition

1x1 conv 1x1xKYxRY

3x1 Sep conv 3x1xRYxRY

1x1 conv 1x1xRYxKY

1x3 Sep conv 1x3xRYxRY

Illustration of CP decomposition in Luma layer hidden unit

Figure 2  Split luma and chroma model + CP Decomposition + fusion of 1x1 conv layer

Table 1 Block-level complexity measure in KMacs/Pixel for split luma and chroma model + CP + fusion

| Split luma+chroma model | complexity in KMAC/Pixel |
| --- | --- |
| CP + fusion | 16.2 |
| (24L, 8C) | 17.7 |
| (20L, 8C) | 14.2 |
| (16L, 8C) | 11.3 |

# Simulation results

Simulations are performed based on the common test conditions for neural-network-based video coding technology [4] for RA condition. For I picture, the model in JVET-AA0080 is used.

Table 2 shows our proposal CP decomposition + fusion of adjacent 1x1 conv compared against VTM EE1 VTM anchor 1. Table 3 shows our proposal CP decomposition + fusion of adjacent 1x1 conv compared against JVET-X0140 EE1-1.4.1 baseline model.

Table 2  Simulation results of CP decomposition + fusion compared against EE1 VTM Anchor 1

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0+V0056 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -5.51% | -4.32% | -1.85% |  |  |
| Class A2 | -5.35% | -5.88% | -2.77% |  |  |
| Class B | -4.70% | -7.80% | -3.77% |  |  |
| Class C | -4.94% | -9.17% | -3.84% |  |  |
| Class E |  |  |  |  |  |
| Overall | -5.06% | -7.08% | -3.20% |  |  |
| Class D | -6.13% | -9.07% | -4.27% |  |  |

Table 3  Simulation results of CP decomposition + fusion compared against JVET-X0140 EE1-1.4.1

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over JVET-X0140 EE1-1.4.1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.46% | 0.14% | 2.17% |  |  |
| Class A2 | 0.64% | 0.04% | 0.87% |  |  |
| Class B | 0.85% | -0.68% | 1.72% |  |  |
| Class C | 0.99% | -0.52% | 2.66% |  |  |
| Class E |  |  |  |  |  |
| Overall | 0.77% | -0.33% | 1.89% |  |  |
| Class D | 1.14% | -0.80% | 2.32% |  |  |

The simulation for combined split luma and chroma model + CP decomposition + fusion of adjacent 1x1 conv compared to EE1 VTM anchor is shown in Table 4 for setting 1 (24L, 8C).

Table 4  Simulation results of split luma and chroma model + CP decomposition + fusion compared using (24L, 8C) against EE1 VTM Anchor 1

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0+V0056 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -5.32% | -6.39% | -5.06% |  |  |
| Class A2 | -5.80% | -9.56% | -5.71% |  |  |
| Class B | -5.00% | -9.67% | -8.97% |  |  |
| Class C | -5.34% | -11.15% | -9.96% |  |  |
| Class E |  |  |  |  |  |
| Overall | -5.31% | -9.39% | -7.80% |  |  |
| Class D | -6.67% | -11.69% | -11.17% |  |  |

# Training and inference information

The models are trained based on BVI-DVC [3] for RA. The raw video data was compressed using RA configurations to create the training examples where Intra model is from JVET-AA0080. As in JVET-X0140 EE1-1.4.1, joint models are used for luma and chroma components. 2 models are used for Intra and 2 modes are used for Inter.

More information related to the training & inference of models can be found in the tables below.

Table 5  Training information of split luma and chroma model + CP decomposition + fusion

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Nvidia GeForce RTX3090 |
|  | Framework: | Tensorflow 2.8.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 50 |
|  | Batch size: | 16 |
|  | Training time: | 124h |
|  | Training data information: | BVI-DVC |
|  | Training configurations for generating compressed training data (is different to VTM CTC): | QP=20,25,30,35,40,45 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 72x72 (4 luma and 2 chroma planes) |
|  | Learning rate: | Epoch : LR, 0-30 1E-4, 30 to 40 1E-5, >40 1E-6 |
|  | Optimizer: | ADAM |
|  | Loss function: | 12-1-1 Weighted L2 |
|  | Preprocessing: | Convert 144x144 YUV420 signal to 6 72x72 blocks. Normalize to 0~1 |
|  | Other information: |  |

Table 6  Inference information of split luma and chroma model + CP decomposition + fusion

| Network Information in Inference Stage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Split LC (16L, 8C) | Split LC (20L, 8C) | Split LC (24L, 8C) |  |
| Mandatory | HW environment: |  |  |  |  |
|  | GPU Type | CPU only |  |  |  |
|  | Framework: | Tensorflow 2.8.0 |  |  |  |
|  | Number of GPUs per Task | 0 |  |  |  |
|  | Number of Parameters (Each Model) | 36974 | 46066 |  | 57014 |
|  | Total Parameter Number | 73948 | 92132 |  | 114028 |
|  | Parameter Precision (Bits) | Floating (32bits) |  |  |  |
|  | Memory Parameter (MB) | 0.282 | 0.368 |  | 0.435 |
|  | MAC/Pixel (Kilo) | 11.3 | 14.2 |  | 17.7 |
| Optional |  |  |  |  |  |
|  | Total Conv. Layers | 88 |  |  |  |
|  | Total FC Layers | 0 |  |  |  |
|  | Total Memory (MB) |  |  |  |  |
|  | Batch size: |  |  |  |  |
|  | Patch size | 72x72 (4 luma and 2 chroma planes ) |  |  |  |
|  | Changes to network configuration or weights required to generate rate points |  |  |  |  |
|  | Peak Memory Usage (Total) |  |  |  |  |
|  | Peak Memory Usage (per Model) |  |  |  |  |
|  | Other information: |  |  |  |  |

# Conclusion

This contribution shows the potential of using split luma and chroma model combining with CP decomposition plus fusion of 1x1 convolutional layer to achieve computational complexity reduction by approximately half compared to the JVET-X0140 low complexity model (EE1-1.4.1). With one pass training, when EE1 VTM Anchor 1 is used, the RA luma gain is about 5.06% with worst case complexity of 16.2 KMAC/Pixel for CP decomposition fused model. When applying split luma-chroma architecture plus CP decomposition fused model, the RA luma gain is about 5.31% with worst case complexity of 17.7 KMAC/pixel for (24L,8C) split. The contribution proposes to further study the complexity reduction framework in EE.

# References

- Y J. N. Shingala, S. Kadaramandalgi, A. Shyam, T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, S. McCarthy, “AHG11: Complexity reduction on neural-network loop filter”, JVET-AA0080, teleconference, Oct 2021

- H. Wang, J. Chen, K. Reuze, A.M. Kotra, M. Karczewicz, “EE1-1.4: Tests on Neural Network-based In-Loop Filter with Constrained Computational Complexity”, JVET-X0140, teleconference, Oct 2021

- Di Ma, Fan Zhang, and David, R. Bull, "BVI-DVC: A Training Database for Deep Video Compression", arXiv preprint arXiv:2003.13552 (2020)

- S. Liu, A. Segall, E. Alshina, R.-L. Liao, “Common Test Conditions and evaluation procedures for neural network-based video coding technology”, JVET-W2016, July 2021

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ittiam Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).