| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0149-v1 |
| --- | --- |

| Title: | Non-EE1: neural network-based intra prediction with learned mapping to VVC intra prediction modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Thierry Dumas, Franck Galpin, Philippe Bordes thierry.dumas@interdigital.com franck.galpin@interdigital.com philippe.bordes@interdigital.com |  |  |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

This contribution proposes a neural network-based intra prediction mode with learned mapping to the VVC intra prediction modes. Thanks to this learned mapping, for the current luma CB predicted with the neural network-based intra prediction mode, the neural network-based intra prediction mode can be represented by a VVC intra prediction mode, and this VVC mode index can be involved in the generation the MPM list of a luma CB being neighbor of the current luma CB.

A low-complexity neural network-based intra prediction mode is proposed with a complexity of 7.7kMAC/pixel (worst-case). The inference uses 16 bits integer model.

On top of VTM-11-NNVC, this neural network-based intra prediction, using the proposed learned mapping yields -3.61%, -3.16%, -3.27% and -1.75%, -0.75%, -1.08% of average BD-rate gains in AI and RA respectively.

On top of NCS-1.0 (with the NN-filter-set 0), the method yields -xxx%, -xxx%, -xxx% and -xxx%, -xxx%, -xxx% of average BD-rate gains in AI and RA respectively.

# Introduction

During the 25th JVET meeting, a single additional neural network-based intra prediction inside VTM-11-NNVC has been studied in EE1 [1]. The average BD-rate gains of VTM-11-NNVC including the low-complexity neural network-based intra prediction mode, the neural networks featuring 16-bit signed integer parameters and intermediate representations, with respect to VTM-11-NNVC are -2.87% -2.32% -2.37% and -1.38% -0.55% -0.82% in AI and RA respectively.

But, when generating the MPM list used to signal the index of a VVC intra prediction mode selected to predict a given luma CB, the index of the single neural network-based intra prediction mode being inside the MPM list is problematic as this index does not represent any intra prediction characteristic. This contribution introduces a learned mapping to solve this issue.

# Proposal

## Learned mapping to the VVC intra prediction modes

For the current W×H block Y to be predicted by the neural network-based intra prediction mode, the context X made of na rows of decoded pixels above Y and nl columns of decoded pixels on the left side of Y is first preprocessed as described in [1], yielding the preprocessed context X, see Figure 1. Then, the neural network dedicated to the prediction of W×H blocks, denoted fh,w. ; θh,w, takes X to return three outputs. The first output Y undergoes the postprocessing detailed in [1], yielding the prediction Y of Y. The second output gathers two indices grpIdx1 and grpIdx2, grpIdxi denoting the index characterizing the LFNST kernel index and whether the primary transform coefficients resulting from the application of the DCT-2 horizontally and the DCT-2 vertically to the residue of the neural network prediction are transposed when lfnstIdx=i, i∈1, 2, see [1]. The third output corresponds to the index repIdx of the VVC intra prediction mode (PLANAR or DC or directional intra prediction mode) whose prediction of Y from the decoded reference samples surrounding Y best represents  Y.

Figure 1: prediction of the current W×H block Y from the context X of decoded pixels around  Y via the neural network-based intra prediction mode.

When creating the MPM list of a given luma CB, if the “left” luma CB is predicted via the neural network-based mode, the neural network-based mode index can be replaced by the repIdx returned during the prediction of the “left” luma CB and become a candidate index to be put into the MPM list. Similarly, if “above” luma CB is predicted via the neural network-based mode, the neural network-based mode index can be replaced by the repIdx returned during the prediction of the “above” luma CB and become a candidate index to be inserted into the MPM list.

## Extension of the context of the block for neural network-based intra prediction

The neural network-based intra prediction mode contains 7 neural networks, each predicting blocks of a different size in 4×4, 8×4, 16×4, 32×4, 8×8, 16×8, 16×16.

For the current W×H block Y to be predicted via the neural network-based intra prediction mode, if W≤8, the context X of Y is extended towards the above-right side of the block by setting ew=4. Otherwise, ew=0. If H≤8, the context X of Y is extended towards the bottom-left side of the block by setting eH=4. Otherwise, eH=0.

# AHG11 template

| Network information in training stage |  |  |
| --- | --- | --- |
| Mandatory | GPU type | GPU: Tesla-P100-16GB |
|  | Framework | Tensorflow 2.6 |
|  | Number of GPUs per task | 1 |
|  | Epoch | 15 |
|  | Batch size | 128 |
|  | Training time | 8 hours |
|  | Training data information | The set of images used to create the training sets combines frames extracted from the BVI-DVC dataset, the TVD dataset, the UVG dataset, and the RGB images in the ILSVCR2012 dataset set and DIV2-K, converted into YCbCr. |
|  | Training configurations for generating compressed training data | VTM-8.0; QP {22, 27, 32, 37} |
|  | Patch size | The neural network predicting w×h blocks is trained on pairs of a w×h block and its context. |
|  | Learning rate | 1e-4 |
|  | Optimizer | ADAM |
|  | Loss function | Mean-squared error. A l1-norm weight decay term is added to the loss function for training the neural network. |
|  | Preprocessing | See [1]. |

| Network information (inference) |  |  |  |
| --- | --- | --- | --- |
| Parameter number | block size | Low-complexity (sparse weights) | High-complexity |
|  | 4×4 8×4 16×4 32×4 8×8 16×8 16×16 | 148599 158817 175629 221081 189222 230097 394196 | 1735338 1793722 1891034 2124570 2124506 2319130 4071515 |
| Parameter precision | 16-bit signed integer. |  |  |
| MAC/pixel | block size | Low-complexity (sparse weights) | High-complexity |
|  | 4×4 8×4 16×4 32×4 8×8 16×8 16×16 | 7773 4254 2405 1560 2624 1625 1411 | 108300 55974 29507 16577 33155 18097 15627 |
| Total conv. layers |  |  |  |
| Total FC layers | 4 cascaded fully-connected layers for the neural network predicting 16×16 blocks 3 cascaded fully-connected layers for the others. |  |  |
| Mem (MB) | 5.3 MB for the whole low-complexity neural network-based mode. 31261 MB for the whole high-complexity neural network-based mode. |  |  |
| GPU Type | No GPU |  |  |
| CPU Type | Xeon 2.4GHz |  |  |
| Framework | SADL (C++ with a single thread) |  |  |

# Experimental results

The test results are presented in Table 1, Table 2, Table 3, and Table 4.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | DecT GPU | bit DIFF |
| Class A1 | -4.31% | -3.67% | -4.03% | -4.54% | -3.52% | -3.86% | 153% | 636% |  | 4% |
| Class A2 | -2.82% | -2.35% | -2.77% | -2.84% | -1.87% | -2.25% | 169% | 755% |  | 3% |
| Class B | -3.24% | -2.79% | -3.00% | -3.07% | -2.43% | -2.31% | 168% | 827% |  | 3% |
| Class C | -3.20% | -2.78% | -3.01% | -3.25% | -2.25% | -2.23% | 165% | 993% |  | 3% |
| Class E | -4.89% | -4.58% | -3.82% | -4.85% | -4.25% | -2.32% | 157% | 875% |  | 5% |
| Overall | -3.61% | -3.16% | -3.27% | -3.61% | -2.78% | -2.54% | 163% | 820% |  | 4% |
| Class D | -3.00% | -2.68% | -2.88% | -2.87% | -2.04% | -2.30% | 165% | 1020% |  | 3% |
| Class F | -2.21% | -1.77% | -1.88% | -2.18% | -1.46% | -1.50% | 130% | 498% |  | 2% |

Table 1: BD-rate gains of VTM-11-NNVC (without postfilter) with the low-complexity neural network-based intra prediction mode, 16-bit signed integer, with respect to VTM-11-NNVC (without postfilter).

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | DecT GPU | bit DIFF |
| Class A1 | -2.26% | -0.84% | -1.62% | -2.02% | -0.52% | -1.45% | 103% | 179% |  | 2% |
| Class A2 | -1.36% | -0.28% | -0.84% | -1.29% | 0.44% | -0.27% | 103% | 155% |  | 1% |
| Class B | -1.76% | -0.67% | -0.72% | -1.57% | -0.28% | 0.46% | 103% | 181% |  | 2% |
| Class C | -1.64% | -1.14% | -1.30% | -1.49% | -0.89% | -0.55% | 107% | 163% |  | 2% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.75% | -0.75% | -1.08% | -1.58% | -0.35% | -0.34% | 104% | 170% |  | 2% |
| Class D | -1.36% | -0.66% | -0.84% | -1.34% | -0.18% | 0.13% | 109% | 186% |  | 1% |
| Class F |  |  |  |  |  |  |  |  |  |  |

Table 2: BD-rate gains of VTM-11-NNVC (without postfilter) with the low-complexity neural network-based intra prediction mode, 16-bit signed integer, with respect to VTM-11-NNVC (without postfilter).

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | DecT GPU |  | bit DIFF |  |
| Class A1 | -5.14% | -4.55% | -4.88% | -5.38% | -4.23% | -4.58% | 255% | 1847% |  |  | 5% |  |
| Class A2 | -3.67% | -3.31% | -3.79% | -3.72% | -2.67% | -3.14% | 292% | 2338% |  |  | 4% |  |
| Class B | -4.13% | -3.73% | -3.96% | -4.00% | -3.36% | -3.20% | 289% | 2724% |  |  | 4% |  |
| Class C | -4.06% | -3.76% | -4.02% | -4.09% | -3.07% | -3.23% | 290% | 2985% |  |  | 4% |  |
| Class E | -6.26% | -6.33% | -5.33% | -6.35% | -5.77% | -3.74% | 260% | 2692% |  |  | 6% |  |
| Overall | -4.56% | -4.23% | -4.33% | -4.60% | -3.73% | -3.52% | 279% | 2535% |  |  | 4% |  |
| Class D | -3.75% | -3.44% | -3.53% | -3.60% | -2.83% | -2.55% | 281% | 3272% |  |  | 4% |  |
| Class F | -2.93% | -2.65% | -2.75% | -3.10% | -2.15% | -2.24% | 180% | 1461% |  |  | 3% |  |

Table 3: BD-rate gains of VTM-11-NNVC (without postfilter) with the high-complexity neural network-based intra prediction mode, 16-bit signed integer, with respect to VTM-11-NNVC (without postfilter).

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | DecT GPU | bit DIFF |
| Class A1 | -2.77% | -1.11% | -2.01% | -2.51% | -0.58% | -1.68% | 112% | 337% |  | 3% |
| Class A2 | -1.79% | -0.54% | -1.20% | -1.78% | 0.39% | -0.45% | 108% | 274% |  | 2% |
| Class B | -2.23% | -0.87% | -0.91% | -2.04% | -0.38% | 0.23% | 111% | 326% |  | 2% |
| Class C | -2.07% | -1.45% | -1.65% | -1.90% | -0.76% | -0.75% | 116% | 359% |  | 2% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.21% | -1.01% | -1.39% | -2.05% | -0.37% | -0.55% | 112% | 325% |  | 2% |
| Class D | -1.76% | -0.89% | -1.21% | -1.78% | -0.47% | -0.28% | 118% | 391% |  | 2% |

Table 4: BD-rate gains of VTM-11-NNVC (without postfilter) with the high-complexity neural network-based intra prediction mode, 16-bit signed integer, with respect to VTM-11-NNVC (without postfilter).

# Conclusion

A low-complexity neural network-based intra prediction mode gives -3.61%, -3.16%, -3.27% and -1.75%, -0.75%, -1.08% of average BD-rate gains in AI and RA respectively. The model has a worst-case complexity of 7.7kMAC/pixel.

For comparison, in the previous EE1 contribution [1], the neural network-based intra prediction mode provided -2.87% -2.32% -2.37% and -1.38% -0.55% -0.82% of average BD-rate gains in AI and RA respectively. It is suggested to study this neural network-based intra prediction mode in EE1.

# References

[1] T. Dumas, F.Galpin, P. Bordes, E. François, “EE1-3.1: Intra prediction using neural networks”, 25th JVET meeting, 12–21 January 2022, JVET-Y0082.

# Patent rights declaration(s)

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).