| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0160-v2 |
| --- | --- |

| Title: | [AHG11] Combination of chroma order adjustment and residual adjustment for NNLF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhenyu Dai Yue Yu Haoping Yu Dong Wang | Tel: Email: | daizhenyu@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution proposes a combination method of chroma order adjustment (from JVET-AB0158[1]) and residual adjustment (from JVET-AB0159[2]) based on JVET-AA0088[3] to further improve the coding performance. Compared with JVET-AA0088, the proposed method demonstrates {-0.08%, -1.97%, -0.90%} and {-0.70%, -3.74%, -1.95%} BD-rate savings with the RA and LDB configurations, respectively.

# Introduction

In JVET-AA0088[3], a neural network based in-loop filter with constrained memory size and low complexity was presented and was adopted as one of the two NNLF sets in the AHG11 reference software.

The network structure of JVET-AA0088 is shown in Fig. 1. Along with the reconstructed image (rec_yuv), additional side information is also fed into the network, such as the prediction image (pred_yuv), slice QP, base QP and slice type.

Fig. 1: Network architecture of JVET-AA0088

# Proposed method

## Chroma order adjustment

In the current neural network-based in-loop filter design, the input tensors are constructed by concatenting the video colour components and corresponding information in a fixed order defined in the training stage. Similarly, the output video colour components are extracted from the output tensor in a fixed order. In JVET-AA0088, the input order is {rec_y, rec_u, rec_v, pred_y, pred_u, pred_v, slice QP, base QP, slice type}, while the output order is {output_y, output_u, output_v}. The same fixed orders for input and output are followed in application of the NNLF (i.e. in inference).

The first aspect of this contribution proposes that, instead of following a fixed order of input/output information, a frame level chroma order adjustment can be made to switch the input/output order of U and V components during the NNLF inference stage. When chroma order adjustment is enabled, the input order is modified to {rec_y, rec_v, rec_u, pred_y, pred_v, pred_u, slice QP, base QP, slice type}, and the output order is modified to {output_y, output_v, output_u}.

A frame-level flag is signaled to indicate whether the chroma order adjustment method is used.

## Residual offset

The second aspect of this contribution proposes that a frame level residual offset can be used to adjust the output of the NNLF in the inference stage.

More specifically, the residual is adjusted by reducing the magnitude of the residual at each pixel by a small offset value. In this contribution, the offset candidates are{1, 2}. For example, if residual adjustment is performed with an offset value of 1, all positive residual values are decremented by 1, while all negative residual values are incremented by 1. An example of residual adjustment with offset value 1 is shown in Fig. 2.

Fig. 2: an example of residual offset

Two frame level flags are signalled to indicate whether the residual offset method is enabled for luma and chroma, respectively. If residual offset is enabled, another frame level flag is signalled to indicate what offset value is used for adjusting the corresponding residual colour component(s).

## Combination of Chroma order adjustment and residual offset

This contribution proposes to combine these two methods together, the chroma order adjustment method will go through first, and followed by the residual offset method. More specifically, one frame level flag is signalled to indicate if chroma order adjustment is used or not. Two more frame level flags are signalled to indicate if residual offset is enabled for luma and chroma, respectively. If residual offset is enabled, another frame level flag is signalled to indicate what offset value is used for adjusting the corresponding residual colour component(s).

# Experiment results

The proposed method has been implemented on top of the AHG11 reference software with the macro NnlfOption=1, which indicates the activation of the NNLF set from JVET-AA0088. The simulations were performed by following the NNVC common test condition (CTC)[4]. Since the proposed method is only used for B-Slice, only Random Access and Low Delay B configurations were tested.

Table 1. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over AHG11 reference software (NnlfOption=1) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -3.36% | -1.18% | 108% | 101% |
| Class A2 | -0.10% | -1.68% | -0.55% | 107% | 99% |
| Class B | -0.11% | -1.92% | -0.80% | 109% | 103% |
| Class C | -0.09% | -1.20% | -1.07% | 106% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -1.97% | -0.90% | 108% | 102% |
| Class D | -0.04% | -0.70% | -1.97% | 106% | 103% |
| Class F | -0.04% | -0.44% | -0.70% | 111% | 112% |

Table 2. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over AHG11 reference software (NnlfOption=1) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.63% | -3.91% | -1.33% | 107% | 108% |
| Class C | -0.70% | -3.52% | -2.71% | 106% | 108% |
| Class E | -0.81% | -0.50% | -1.14% | 112% | 134% |
| Overall | -0.70% | -3.74% | -1.95% | 108% | 114% |
| Class D | -0.40% | -3.34% | -5.51% | 106% | 103% |
| Class F | -0.18% | -3.65% | -3.86% | 112% | 113% |

# Conclusions

This contribution proposes a combination method of chroma order adjustment and residual adjustment for the NNLF set based on JVET-AA0088 in AHG11 reference software to further improve the coding performance. This method can provide good performance on top of JVET-AA0088 without increasing decoder-side complexity. It is suggested to adopt this combination method into the NNLF set from JVET-AA0088 in the AHG11 reference software.

# References

- [AHG11] On chroma order adjustment in NNLF, JVET-AB0158, Z. Dai, Y. Yu, H. Yu, D. Wang (OPPO)

- [AHG11] On adjustment of residual for NNLF, JVET-AB0159, Z. Dai, Y. Yu, H. Yu, D. Wang (OPPO)

- EE1-1.5: neural network based in-loop filter with a single model , JVET-AA0088, L. Wang, S. Lin, X. Xu, S. Liu (Tencent), F. Galpin (InterDigital)

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-AA2016, E. Alshina, R.-L. Liao, S. Liu, A. Segall

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).