| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE 20–28 October 2022 | Document: JVET-AB0165 |
| --- | --- |

| Title: | EE2-1.16: Picture-level geometry transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Jia1, Kai Zhang1, Yang Wang2, Tianliang Fu2, Yue Li1, Li Zhang1 18910 University Center Lane, San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | wei.jia@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This proposal reports the performance of EE2-1.16, in which a method of picture-level geometry transform is proposed. Three geometry transforms are supported at picture level: horizontal flip, vertical flip and 180°-rotation. It is signaled in the slice header to indicate whether the geometry transform is applied, and which transform is used. On top of ECM-6.0, simulation results of EE2-1.16 are reported as below:

AI: {-0.20%, -0.20%, -0.25%; 101%, 99%};

RA: {}.

# Introduction

In the current design of video/image coding scheme, the orientation of the input picture is fixed. A method of adaptive picture flipping coding was proposed in [1], in which an image was coded with different orientations.

# Proposed method

A method of picture-level geometry transform was proposed in JVET-AA0142 and established as EE2-1.16. Three geometry transforms are supported at picture level: horizontal flip, vertical flip, and 180°-rotation. It is signaled in the picture header to indicate whether the geometry transform is applied, and which transform is used.

The best geometry transform is selected via rate distortion optimization (RDO) at encoder. To accelerate the encoder, a simplified encoding process is used in RDO and only the selected geometry transform is encoded with the original encoding process. The encoder will disable geometry transform with resolution smaller than 1080p.

When a geometry transform is selected, the picture will be restored to the original orientation after reconstruction.

# Simulation results

The proposed method is implemented on top of ECM-6.0 [2]. Simulations are performed following the common test conditions of EE2 [3]. The simulation results of the proposed method compared to ECM-6.0 are shown in Table 1.

Table 1: Coding performance

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.16% | 0.02% | 102% | 100% | -0.07% | -0.08% | -0.04% | 100% | 100% |
| Class A2 | -0.52% | -0.43% | -0.34% | 102% | 101% |  |  |  |  |  |
| Class B | -0.29% | -0.38% | -0.71% | 102% | 100% | -0.14% | -0.41% | -0.72% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 97% | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 98% |  |  |  |  |  |
| Overall | -0.20% | -0.20% | -0.25% | 101% | 99% |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 98% | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class F | -0.06% | -0.10% | 0.00% | 106% | 99% | -0.03% | -0.06% | -0.04% | 100% | 98% |

# Conclusions

This proposal reports the EE test results for the method of picture-level geometry transform proposed based on ECM-6. Simulation results indicate 0.2%/.       % coding gain in AI/RA configurations with 101%/99% and .  %/.   % computational complexity It is recommended to adopt EE2-1.16.

# Reference

- T. Murakami, I. Karube, H. Ito and M. Takahashi, “Adaptive picture flipping coding for enhancing H.264/AVC,” Proc. 26th Picture Coding Symp. (No. 1027), Nov. 2007.

- ECM-6.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-6.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).