| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0172-v1 |
| --- | --- |

| Title: | Comments on SMPTE liaison on ITP-PQ-C2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposed action |  |  |
| Author(s) or Contact(s): | W. Husak | Email: | wjh@dolby.com |
| Source: | Dolby Labs |  |  |

_____________________________

# Abstract

SMPTE is in the process of standardizing a color representation used by streaming platforms to deliver HDR content to consumers.  The representation is a variation of the ITP color format that was developed for print applications.  ITP has a distinct characteristic where the hue is perceptually linear across a wide range of luminance values.   The hue linearity from ITP coupled with the perceptual luminance linearity from the PQ (BT.2100) transfer function results in ITP-PQ-C2 being an ideal color volume for image processing - especially when expanding or contracting a color volume.

- Introduction

ST 2128 describes two conversions based on two different sets of color primaries:

- XYZ → ITP-PQ-C2

- BT.2100 → ITP-PQ-C2

The vast majority of available content originates from studio masters using BT.2100 primaries.  The ICtCp color representation defined in BT.2100 is the color difference form of ITP-PQ-C2. A code point describing the conversion from BT.2100 to ITP-PQ-C2 is requested by SMPTE.

- Proposal

The following is proposed for resolving the issues:

- Add appropriate equations in the semantics under matrix_coeffs.

- Add an additional code point in Table E.5.

The detailed proposed text changes are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts):

The following example is for Rec. ITU-T H.265 | ISO/IEC 23008-2.  Similar additions can be added to other VUI parameter sets such as Rec. ITU-T H.273 | ISO/IEC 23001-8.

matrix_coeffs semantics

An additional code point needs to be added.  For the purposes of this document, the assumption is the next code point is 15.  The process of conversion is a two step process where the underlying R, G, and B values are first converted to LMS and then coverted to ITP-PQ-C2.  The current conversion to LMS are described in Equations [4] through [E7].

The following describe the modifications for conversion from LMS to IPT.

Otherwise (matrix_coeffs is equal to 15), the following equations apply:

–

E′Y = ( 1 638 * E′L + 1 638 * E′M +  820 * E′S ) ÷ 4 096	 (E-68)

E′PB = ( 18 248 * E′L − 19 870 * E′M + 1 622 * E′S ) ÷ 4 096	(E-69)

E′PR = ( 3 300 * E′L + 1 463 * E′M – 4 763 * E′S ) ÷ 4 096	(E-70)

NOTE 14 – In this case, for purposes of the ITP nomenclature used in Table E.5, E′Y, E′PB, and E′PR of Equations E-68, E-69, and E-70 may be referred to as I, T, and P, respectively.

# References

- SMPTE “Liaison statement from SMPTE to SC 29/WG 4 and 5 regarding Reserving VUI values to signal the SMPTE ST 2128 color space” m61406, Oct. 2022.

- ITU BT.2100-2 “Image parameter values for high dynamic range television for use in production and international programme exchange” Aug. 2018.

# Patent rights declaration(s)

Dolby may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).